3GPP TSG-RAN3 #121bis	R3-235266

9th October – 13th October 2023

Xiamen, China

Agenda Item:	14.2

Source:	Ericsson

Title:	(TP for LTM BL CR to TS 38.401) Solutions for LTM

Document for: Discussion, Decision

# 1	Introduction

In the previous meeting the issue of L1/L2 triggered mobility was discussed and some agreements were made. In this contribution we will further discuss the main components of the L1/L2 triggered mobility and put forward our proposals.

# 2	Discussion

In the previous meeting the following agreements were captured:

Turn the WA to agreement that a new Class-2 procedure is introduced to notify gNB-CU about the LTM cell switch.

The gNB-CU uses the UE Context Setup procedure to collect the TCI state configurations of candidate cells in LTM configuration phase for inter-DU LTM.

After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE.

The LTM Cell Change Notification procedure is reused to transfer the selected beam from the source gNB-DU to the target gNB-DU via gNB-CU.

RAN3 prioritizes specifying support for early TA acquisition “without RAR” and mark the method “with RAR” as open issue.

CU shall send the TA value to the source DU.

For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU.

Agree to add the RS configuration in UE Context Setup/Modification in stage2 and stage3.

The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations.

In inter-DU LTM, the gNB-CU provides the CFRA resource received from the candidate gNB-DU to the source gNB-DU via UE Context Modification procedure.

Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2.

WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU.

WA: to introduce (2)new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

How the source DU detect failure of TA info acquisition is up to implementation (eg., based on timer in source DU).

Source DU is in charge of checking the TA validity.

In case that L3 handover is triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority.

In case that LTM is triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority,

WA: In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority.

In the following we will look into the open points, analyse them and provide our views and proposals.

## 2.1	Execution of L1/L2 triggered mobility cell switch

In the previous meetings the issue of the execution of LTM cell switch was discussed repeatedly and it has been recognized that two main different approaches have come up as the key alternatives for the inter-DU case and further interaction between RAN2 and RAN3 took place about this issue. The understanding of both RAN2 and RAN3 is that both approaches are feasible. Approach 1 has been taken as baseline. Also an implementation is allowed to follow approach 2 according to RAN2’s reply LS.

- Approach 1 has been taken as baseline, and an implementation can be based on approach 2.

Based on that we propose to add a clarification in TS 38.401.

- Add a clarification in TS 38.401 that approach 2 can be realized in an implementation.

## 2.2	Selected beam transfer

Regarding the selected beam transfer the following FFS was captured in the Chair notes:

FFS for the detail of selected beam information.

For the part of CU to target DU is FFS of which procedure can be used.

FFS on the procedure from CU to target DU to inform the selected beam information and target cell ID.

Option 1: Two new class 2 procedures with different name.

Option 2: same new class 2 procedure

Option 3: class 1 procedure.

Down selection among those 3 options  at next meeting.

## To summarize the issue, RAN2 has among others stated the following in the incoming LS:

Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

Following that, in the previous RAN3 meeting the following agreement was reached

The LTM Cell Change Notification procedure is reused to transfer the selected beam from the source gNB-DU to the target gNB-DU via gNB-CU.

Regarding the signalling from gNB-CU to target gNB-DU, we believe that there is no essential difference, and the same procedure can be used. Based on that we propose to use the LTM Cell Change Notification procedure to transfer the selected beam from gNB-CU to target gNB-DU.

- We propose to use the LTM Cell Change Notification procedure to transfer the selected beam from gNB-CU to target gNB-DU.

## 2.3 	L1 measurement configuration and reporting

Regarding the issue of L1 measurement configuration and reporting the following FFS was captured in the Chair notes:

FFS on the details of Stage 2 and stage3.

FFS whether CU needs to request the DU to provide the RS configuration.

In the previous meeting the following agreements were reached:

For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU.

Agree to add the RS configuration in UE Context Setup/Modification in stage2 and stage3.

The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations.

Also in RAN2 the following agreements were reached:

- The RS configuration is provided to the UE per LTM candidate cell. - RAN2 assumes that Each candidate DU needs to know the RS configuration of each candidate DUs in order to provide the LTM candidate configuration. - RAN2 assumes that The CU transmits to each C-DU the RS configuration of S-DU (if this is an LTM candidate cell) and/or other C-DUs, to generate the corresponding L1 configuration for LTM. RAN2 assumes C-DU generates the RS configuration and send to the CU. The CU transmits to the Source DU the RS configuration per LTM candidate cell and the associated LTM candidate (when the CU receives LTM candidate configuration(s) from the C-DU). It is up to RAN3 whether the RS configuration is sent before (or at the same time of) the C-DU creates the LTM candidate configuration (and whether is semi-statis or UE

Based on the above agreements from RAN2 also it is evident that the best practice would be that CU requests the DU to provide the RS configuration and in that way the CU collects the RS configuration of the DU(s) and can then transmit to the candidate DU(s).

- We propose that the gNB-CU requests the gNB-DU to provide the RS configuration.

2.4 	TCI state configuration

In the previous meeting the following FFS was captured

FFS on the need of an explicit request from CU

Also the following agreements were made:

The gNB-CU uses the UE Context Setup procedure to collect the TCI state configurations of candidate cells in LTM configuration phase for inter-DU LTM.

After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE.

RAN1 has agreed that an indication of a beam or a TCI state (e.g. a TCI state index or some form of TCI identifier) will be included in the LTM cell switch command (i.e. MAC CE indicating the UE to perform LTM cell switch to an LTM candidate cell), with the intention that the UE identifies a TCI state of the LTM candidate cell during LTM cell switch. In addition, RAN1 has also agreed that the UE may receive the indication of a TCI state of an LTM candidate cell even before the LTM cell switch command is received, in a procedure which is sometimes called DL pre-sync with LTM candidate cell(s), since the TCI state ID provided even before the LTM cell switch command.

The CU may request explicitly a C-DU to provide the configuration associated to DL pre-sync either when it is requesting the C-DU to configure LTM candidate cell(s) in a UE CONTEXT SETUP REQUEST and/or afterwards, for an already configured LTM candidate, in a UE CONTEXT MODIFICATION REQUEST message.

The CU can receive the indication of the TCI of an LTM candidate cell in the UE Context Setup Response message and then can forward this indication to the serving DU via the UE Context Modification procedure. So legacy procedures can be used for early DL sync. Thus, we propose the following.

- The CU can request DL pre-sync configuration either in a UE CONTEXT SETUP REQUEST or a UE CONTEXT MODIFICATION REQUEST. In response, the CU receives the DL pre-sync configuration of in the UE Context Setup Response or the CONTEXT MODIFICATION RESPONSE message and then can forward this indication to the serving DU via the UE Context Modification procedure.

An example of a signaling flow between CU, S-DU and C-DU is provided below for the case in which the TCI state ID for an LTM candidate is included in a MAC CE received by the UE for pre-activating a TCI state before the reception of an LTM cell switch command.

Figure 6 DL pre-sync and provision of TCI state ID of a candidate before an LTM cell switch command

## 2.5	Reference configuration

The topic of reference configuration was discussed in the previous meeting and the following was captured in the Chair notes:

Prioritize the reference configuration on F1 at next meeting.

List all options and down selection

The options are listed below:

- A source gNB-DU may generate the reference configuration for initial LTM configuration.

- The gNB-DU takes the reference configuration into account when generating the lower layer RRC configurations.

- The gNB-CU may provide a reference configuration to the candidate gNB-DU in the UE Context Setup Request message to the gNB-DU.

- The candidate gNB-DU may generate the reference configuration for LTM configuration if not received from the gNB-CU.

- The candidate gNB-DU is not allowed to update the reference configuration if received from the gNB-CU.

The source gNB-DU can provide the current configuration to the target gNB-DU via gNB-CU and this can be used as reference configuration. Otherwise the gNB-CU nay have stored a reference configuration which can be then signalled to the candidate gNB-DU. Also if the gNB-CU does not have a reference configuration stored, it can request the candidate gNB-DU to generate a reference configuration which the gNB-CU then stores. The gNB-CU is in charge of asking the gNB-DU to generate a reference configuration so the candidate gNB-DU should not update the received reference configuration. Also even in the intra-DU case the gNB-CU may have a stored reference configuration that is different than the current reference configuration in use.

Based on the above we propose the following:

Proposal 5: A source gNB-DU can provide the current configuration to the gNB-CU and this can be used as reference configuration.

Proposal 6: The gNB-CU provides a reference configuration to the candidate gNB-DU in the UE Context Setup Request message or the UE Context Modification Request message to the gNB-DU.

Proposal 7: The candidate gNB-DU may generate the reference configuration for LTM configuration if requested to do so by the gNB-CU.

Proposal 8: The candidate gNB-DU is not allowed to update the reference configuration if received from the gNB-CU.

## 2.6	E1 aspects

Regarding E1 aspects the following agreements were made in the previous meeting:

For LTM without UP change case, propose to turn the following WF to an agreement:

In case of CP UP separation, once CUCP receives LTM cell switch signaling from (source)DU , CU CP initiates E1 bearer context modification to the CU UP including DL tunnel ID per DRB for target cell, for data transmission.

For LTM with UP change, once the CU-CP receives LTM cell switch signaling from (source) DU, the CU-CP initiates E1 bearer context modification to the target CU UP including DL tunnel ID per DRB for target cell for data transmission.

For LTM with UP change, the CU-CP initiates E1 bearer context modification to the source CU-UP for retrieving the latest PDCP status at the source CU-UP and exchanging the data forwarding information to target CU-UP.

For LTM with UP change after detecting the UE has accessed to the target cell, the CU-CP initializes Path switch procedure towards the core network.

The procedure used for inter-DU LTM with the change of gNB-CU-UP within a gNB is shown below.

Figure 8.2.1.z LTM with the change of gNB-CU-UP

0. The source gNB-DU forwards the Measurement Report to the gNB-CU-CP.

1. The gNB-CU-CP decides to initiate LTM configuration.

2. The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the target gNB-CU-UP.

3.	The target gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, DL TNL address information for NG-U, and the TNL address information for data forwarding to the target gNB-CU-UP.

4 - 5. F1 UE context setup procedure is performed to setup one or more bearers in the gNB-DU.

6 - 7.The gNB-CU-CP sends the RRC Reconfiguration message to the UE. Details are FFS.

8.  The UE sends the lower layer measurement result to the source gNB-DU, and the source gNB-DU decides to execute LTM to a candidate target cell.

9. WA: The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU-CP with the selected target cell ID.

10-11. The gNB-CU-CP performs the Bearer Context Modification procedure to retrieve the PDCP UL/DL status and to exchange the TNL address information for data forwarding for the bearers.

12-13.	The gNB-CU-CP performs the Bearer Context Modification procedure to send the DL TNL address information for F1-U and the PDCP UP/DL status to the target gNB-CU-UP.

14.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

15.  The target gNB-DU detects the UE in the target cell.

16. The target gNB-DU sends an ACCESS SUCCESS message to the gNB-CU-CP.

17 - 19. Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

20-21.	Bearer Context Release procedure may be performed to release the UE context in the source gNB-DU.

In principle, the call flow shown above can work and is not erroneous. Nevertheless, we note that the impact on CU-UP is substantial. Namely, the CU-UP will need to produce UL TNL per DRB for every configured candidate cell and signal them to CU-CP. For that reason, we investigated alternative ways that would lead to a minimization of the impact on CU-UP.

- The impact on CU-UP needs to be minimized.

In our quest to minimize the CU-UP impact we explored the following option.

In step 3 the CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing only one UL TNL address information for F1-U to all the candidate cells but only the target cell will be allowed to use it. In that case the BEARER CONTEXT SETUP REQUEST message indicates to CU-UP not to initiate sending downlink packets until the UE successfully accesses the target cell. Based on the above we make the following proposal.

- One option that can minimize the impact on CU-UP when performing LTM is that the CU-UP provides only one UL TNL address which will only be used by the target cell after successful execution of LTM cell switch.

## 2.7	TA management

Regarding TA management the following was agreed in the previous meeting:

RAN3 prioritizes specifying support for early TA acquisition “without RAR” and mark the method “with RAR” as open issue.

CU shall send the TA value to the source DU.

In inter-DU LTM, the gNB-CU provides the CFRA resource received from the candidate gNB-DU to the source gNB-DU via UE Context Modification procedure.

Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2.

WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU.

WA: to introduce (2)new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

How the source DU detect failure of TA info acquisition is up to implementation (eg., based on timer in source DU).

Source DU is in charge of checking the TA validity.

In the following we discuss the working assumptions and provide our views.

Indeed, we note that when the C-DU sends the TA values, there might be overlapping configurations, it can send the mapping between preamble and TA values to the CU and then to the S-DU, as agreed by RAN2:

From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

This could result in conflicts in RACH access, if UE served by different DUs use the same CFRA resources. To avoid such conflicts the best solution is to restrict the use of specific CFRA resources to UEs that are served by one DU only. In that way the conflicts are avoided. To be able to achieve this, there needs to be a way to differentiate between the different DUs. Specifically the CU would need to indicate in the request to the candidate DU for which source DU it requests CFRA resources. The simplest way would be to include in the request message the source DU ID. Based on the above we propose to turn into agreements the working assumptions about the CFRA resources.

Proposal 10: Turn into agreement the working assumption which states that the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2.

Proposal 11: Turn into agreement the working assumption which states that WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU.

To summarize the TA acquisition in order to move forward when the TA establishment is triggered the candidate DU receives the preamble, calculates the TA value and provides it to the CU which then forwards it to the source DU. To achieve this the best solution is the definition of two new message that would be non-UE associated and thus a better fit for the purpose of the procedure in question.

Below the signalling flow for inter-DU LTM configuration with TA establishment configuration and triggering is shown

Figure 4 signalling flow for inter-DU LTM configuration with TA establishment configuration and triggering.

As we can see for the steps 10 and 11, 2 new messages would be needed. In step 10 a non-UE associated message from candidate DU to CU and in step 11 a non-UE associated message from CU to source DU. But we believe that the same message can support both cases. Intrinsically the same information would be transferred, and we see no reason why to define two new procedures to do essentially the same thing. So based on the above we propose to turn into agreement the working assumption regarding the messages to signal the TA value but with the additional clarification that only one procedure needs to be introduced.

- We propose to agree to introduce 1 new class 2 non-UE associated F1 signaling to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

## 2.8	LTM and CHO coexistence

In the previous meetings we discussed the issues of coexistence of LTM and L3 HO. If we consider the case of LTM and CHO coexistence, then as in legacy procedure if the CHO procedure is successfully executed, then the LTM candidate cells may be released.

We propose to -as also done in legacy- add a note in TS 38.401

- Add a note in TS 38.401 to capture that in the case of successful CHO, the candidate LTM cells if any may be released.

## 2.9	F1AP restructure

In RAN2 the following agreement was taken:

RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch).

Also, in the endorsed running CR for TS 38.300 the early synchronization procedures are captured in a different section than the LTM procedure. Based on the above, we propose to reflect these agreements also in TS 38.473 and specify the early sync part in a different IE than the LTM part.

- RAN3 to agree the F1AP TP in [2].

Finally, the stage-2 signaling flows for LTM execution, Early UL sync, Early DL sync and CHO are provided in the Annex.

- RAN3 to agree the stage-2 signaling flows in the Annex.

# 3	Conclusion

In the previous sections we made the following observations:

- Approach 1 has been taken as baseline, and an implementation can be based on approach 2.

- The impact on CU-UP needs to be minimized.

In this paper we propose the followings.

- Add a clarification in TS 38.401 that approach 2 can be realized in an implementation.

- We propose to use the LTM Cell Change Notification procedure to transfer the selected beam from gNB-CU to target gNB-DU.

- We propose that the gNB-CU requests the gNB-DU to provide the RS configuration.

- The CU can request DL pre-sync configuration either in a UE CONTEXT SETUP REQUEST or a UE CONTEXT MODIFICATION REQUEST. In response, the CU receives the DL pre-sync configuration of in the UE Context Setup Response or the CONTEXT MODIFICATION RESPONSE message and then can forward this indication to the serving DU via the UE Context Modification procedure.

- A source gNB-DU can provide the current configuration to the gNB-CU and this can be used as reference configuration.

- The gNB-CU provides a reference configuration to the candidate gNB-DU in the UE Context Setup Request message or the UE Context Modification Request message to the gNB-DU.

- The candidate gNB-DU may generate the reference configuration for LTM configuration if requested to do so by the gNB-CU.

- The candidate gNB-DU is not allowed to update the reference configuration if received from the gNB-CU.

- One option that can minimize the impact on CU-UP when performing LTM is that the CU-UP provides only one UL TNL address which will only be used by the target cell after successful execution of LTM cell switch.

- Turn into agreement the working assumption which states that the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2.

- Turn into agreement the working assumption which states that WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU.

- We propose to agree to introduce 1 new class 2 non-UE associated F1 signaling to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

- Add a note in TS 38.401 to capture that in the case of successful CHO, the candidate LTM cells if any may be released.

- RAN3 to agree the F1AP TP in [2].

- RAN3 to agree the stage-2 signaling flows in the Annex.

# 4	References

RP-223520, Revised WID on Further NR mobility enhancements, MediaTek Inc., Apple

R3-235267, (TP for LTM BL CR to TS 38.473) F1AP impacts for LTM

# Annex A: TP to BL CR for TS 38.401 to support LTM

--------------------------------------------------------- the start of changes -------------------------------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

BH	Backhaul

CAG	Closed Access Group

CHO	Conditional Handover

CLI	Cross-Link Interference

CM	Connection Management

CMAS	Commercial Mobile Alert Service

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

EM	Element Manager

EN-DC	E-UTRA-NR Dual Connectivity

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

FTEID	Fully Qualified TEID

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

L2					Layer-2

LTM				L1/L2 Triggered Mobility

MBS	Multicast Broadcast Service

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MN	Master Node

MgNB	Master gNB

MRB	MBS Radio Bearer

MRDC	Multi-Radio Dual Connectivity

NAS	Non-Access Stratum

NID	Network identifier

NPN	Non-Public Network

NSA	Non Standalone

OAM	Operation, Administration and Maintenance

PNI-NPN	Public Network Integrated Non-Public Network

PTP	Point to Point

PTM	Point to Multipoint

PWS	Public Warning System

QoE	Quality of Experience

QoS	Quality of Service

RET	Remote Electrical Tilting

RIM	Remote Interference Management

RIM-RS	Remote Interference Management Reference Signal

RNL	Radio Network Layer

RRC	Radio Resource Control

SA	Standalone

SAP	Service Access Point

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SFN	System Frame Number

SgNB	Secondary gNB

SM	Session Management

SMF	Session Management Function

SN	Secondary Node

SNPN	Stand-alone Non-Public Network

SRAP	Sidelink Relay Adaptation Protocol

TA	Timing Advance

TCE	Trace Collection Entity

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TEID	Tunnel Endpoint Identifier

TMA	Tower Mounted Amplifier

TNL	Transport Network Layer

U2N	UE-to-Network

---------------------------------------------------------next change -------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.Y-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU LTM

1. The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2. The gNB-CU determines to configureLTM.

3. The gNB-CU sends a UE CONTEXT SETUP REQUEST message, for requesting an LTM configuration to the candidate gNB-DU, containing the cell ID of the cell being requested to be an LTM candidate cell. The gNB-CU indicates the source gNB-DU ID, and may request from the Candidate gNB-DU PRACH resources  for early TA acquisition (early UL sync) and/or TCI state configuration(s) for LTM candidate cell(s). To support sub-sequent LTM, the gNB-CU may also indicate CSI resource configuration(s) including potential LTM candidate cell(s) for that candidate gNB-DU, which may generate the CellGroupConfig for the LTM candidate cell taking that into account.

4. If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration of the accepted target candidate cell (CellGroupConfig to be applied in LTM execution), and, if accepted, the requested configuration for early UL sync and TCI states (e.g., TCI state configuration and RACH configuration) and the RS configuration (FFS) for the accepted target candidate cell, if requested. The candidate gNB-DU includes the CellGroupConfig for the LTM candidate cell taking that into account the CSI resource configuration, if provided for sub-sequent LTM.

5. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the RS configuration(s) of LTM candidate cell(s), the CSI resource configuration(s) based on the accepted LTM candidate cells, the TCI state configuration and the RACH configuration for early UL sync (Time Alignment establishment) of the accepted target candidate cell(s) in other gNB-DUs.

6. The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including an update of the UE’s current configuration including the LTM reporting configuration, based on the CSI resource configuration for the accepted LTM candidate cell(s). That may also include the RS configuration of the source cell (FFS), and prepared candidate cells (FFS).

7/8. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing  the cell ID(s), including a CSI resource configuration so the candidate gNB-DU(s) can update the lower layer configuration(s) for LTM candidate cells, for sub-sequent LTM, e.g. in case not all LTM candidate cells were accepted in step 3.

The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

9. The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10. The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11. The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12. The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13. Early Timing Advance (TA) acquisition (early UL synchronization) is performed, if configured, as specified in TS 38.300 [x].

14. The candidate gNB-DU sends a TA value and the associated CFRA resource information (FFS) to the source gNB-DU via the gNB-CU.

FFS on the message name.

15. The UE sends the lower layer measurement result to the source gNB-DU.

16. The source gNB-DU decides to execute LTM to an LTM candidate target cell.

FFS: Notifying the LTM triggering decision to the other nodes as well.

NOTE: The source gNB-DU may indicate LTM execution to the other nodes before sending LTM command to the UE by appropriate implementation.

17. The source gNB-DU sends LTM command to the UE, including the TA value and the identifier of the LTM candidate target cell.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

18. The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the selected beam information.

19. The gNB-CU sends the target cell ID and the selected beam information to the target gNB-DU.

FFS on the message to be used and details of the selected beam information.

20. FFS:  how the target gNB-DU detects the UE access.

21. The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

22. The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

23. The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

---------------------------------------------------------next change -------------------------------------------------------------

##### 8.2.1.Y-y	Early UL sync

Figure 8.2.1.Y-y: Early UL sync for inter-DU LTM

Early UL sync comprises the following steps:

- The UE sends a MeasurementReport message (L3 measurements) to the source gNB-DU containing measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 triggered mobility configuration and TA establishment

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cells and also request for TA establishment.

- If the candidate gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s) and the RACH configuration for TA establishment.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 triggered mobility configuration.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU sends PDCCH order to the UE for triggering TA establishment, indicating a RACH resources/ preamble from the RACH configuration for TA establishment.

- The UE sends to the candidate gNB-DU a RACH preamble according to RACH configuration for TA establishment as indicated in the PDCCH order from the S-DU.

- The candidate gNB-DU sends to the gNB-CU the TA value of the target candidate cell associated to the received RACH preamble. The reception in the source gNB-DU of the TA value from the gNB-CU indicates the successful reception of the RACH preamble in the C-DU, otherwise a RACH preamble re-transmission may be triggered by the S-DU.

FFS: introduce new message or reuse legacy message for the non-UE-associated signalling which includes the TA value.

- The gNB-CU sends to the source gNB-DU the TA value of the candidate target cell associated to the received RACH preamble.

FFS: introduce new message or reuse legacy message for the non-UE-associated signalling which includes the TA value.

- The UE sends the lower layer measurement result to the source gNB-DU.

- The source gNB-DU decides to execute L1/L2 triggered mobility to a candidate target cell.

- The source gNB-DU sends LTM command including the TA value of the target candidate cell to the UE.

- The UE transmits PUSCH/PUCCH to the candidate DU based on TA value of the target candidate cell.

---------------------------------------------------------next change -------------------------------------------------------------

##### 8.2.1.Y-z	Early DL sync

Figure 8.2.1.Y-z: Early DL sync for inter-DU LTM

DL sync comprises the following steps:

- The UE sends a MeasurementReport message (L3 measurements) to the source gNB-DU containing measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 triggered mobility configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cells and request the configuration associated to DL pre-sync.

- If the candidate gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s) and a TCI state configuration per accepted candidate target cell for DL pre-sync.

4a.	 The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU, including a TCI state configuration per accepted candidate target cell.

4b.	 Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the source gNB-DU sends a UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU with the updated version of the UE’s current CellGroupConfig including a TCI state configuration per accepted candidate target cell.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message including the updated version of the UE’s current CellGroupConfig including a TCI state configuration per accepted candidate target cell.

- The source gNB-DU forwards the received RRCReconfiguration message including the updated version of the UE’s current CellGroupConfig including a TCI state configuration per accepted candidate target cell to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU sends a UL RRC MESSAGE TRANSFER message to the gNB-CU, which includes the RRCReconfigurationComplete message.

- The UE sends the lower layer measurement result to the source gNB-DU.

- The source DU sends a TCI pre-activation command to the UE.

---------------------------------------------------------Next change -------------------------------------------------------------

#### 8.2.1.3	Inter-gNB-DU Conditional Handover or Conditional PSCell Change

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for conditional handover or conditional PSCell change. Figure 8.2.1.3-1 shows the inter-gNB-DU conditional mobility procedure for intra-NR.

Figure 8.2.1.3-1: Inter-gNB-DU Conditional Handover or Conditional PSCell Change for intra-NR

1-2.	The steps 1-2 are as defined in clause 8.2.1.1.

3.	The gNB-CU sends an UE CONTEXT SETUP REQUEST message to the candidate gNB-DU to create an UE context and setup one or more data bearers. The UE CONTEXT SETUP REQUEST message is sent for each candidate cell and includes a HandoverPreparationInformation (conditional handover) or a CG-ConfigInfo (conditional PSCell change).

4.	The candidate gNB-DU responds to the gNB-CU with an UE CONTEXT SETUP RESPONSE message including the target cell ID that was requested from the gNB-CU. The response message is sent for each requested candidate cell.

5.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes a generated RRCReconfiguration message.

6.	The step 6 is as defined in clause 8.2.1.1.

7-8.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message, for which the source gNB-DU forwards to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

9.	 An execution condition to trigger initiation of conditional handover or conditional PSCell change is fulfilled.

10. A Random Access procedure is performed at the candidate gNB-DU, which becomes the target gNB-DU if successful. The target gNB-DU sends a Downlink Data Delivery Status frame to inform the gNB-CU. The target gNB-DU also sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed.

11-12.	The steps 11-12 are as defined in steps 9-10 in clause 8.2.1.1.

13.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU and indicates to stop the data transmission for the UE. The source gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU.

NOTE:	The step 13 may happen before step 12, as soon as the gNB-CU knows which cell the UE has successfully accessed.

NOTE:	The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate target gNB-DUs, if any, to cancel conditional handover or conditional PSCell change for the UE.

NOTE:	The gNB-CU may initiate UE Context Release procedure toward the other candidate target gNB-DU(s), if any, to cancel LTM cell switch for the UE.

14.	The source gNB-DU responds to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

15 -16. 	The steps 15-16 are as defined in steps 11-12 in clause 8.2.1.1.

---------------------------------------------------------End of changes -------------------------------------------------------------