3GPP TSG-RAN WG3#121bis	R3-235289

Xiamen, China, 9th – 13th Oct 2023

Agenda Item: 		12.2.2.2

Source:		NEC

Title:	AI/ML Measured UE Trajectory information

Document for:		Discussion and decision

- Introduction

After RAN3#121 meeting, and the following agreements and open issues were captured:

12.2.2.2. ME and Xn procedures ….. RAN3#121: The measurement object to request the report of the UE Trajectory IE should be introduced in the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message. The AI/ML Measurement ID in HANDOVER REQUEST from source gNB is reused to trigger the target node feedback the UE trajectory information to the source gNB. AI/ML INFORMATION UPDATE from target node is used to convey the UE trajectory information. The Measured UE Trajectory reporting is one-time report. The conditions triggering the one-time reporting are at least: - Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE); - Condition B: UE leaves the target node; - Condition C: Expiration of the report time duration; - Condition D: The configured number of hand-overs within the same target node is reached. Report time duration and configured number of intra-NG-RAN node inter-cell handovers are included in DATA COLLECTION REQUEST message. The list of Cell IDs included in the Measured UE Trajectory is mandatory. Work on stage3 details

In this paper, we provide our views.

- Discussion

In last RAN3#121 meeting when the conditions to have the measured UE Trajectory reporting were agreed namely

- Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE);

- Condition B: UE leaves the target node;

- Condition C: Expiration of the report time duration;

- Condition D: The configured number of hand-overs within the same target node is reached.

there are still some more detail things need to have further discussed.

Whether an explicit or implicit indication on condition A and condition B or any other condition is signalled to the requesting node needs further discussion.

If the explicit indication is not indicated to the requesting node, which means it is up to the source NG-RAN node to guess, which is not good. Furthermore, since the UE Trajectory is inferred by the source NG-RAN node base on the result of learning/training, and the training may be based on some kind of learning, it would be beneficial from the source NG-RAN node perspective to know explicitly the condition that collected measured UE Trajectory.

Proposal 1: For the reporting of Measured UE Trajectory, explicit indication on conditions is signalled to the requesting node.

The list of Cell IDs included in the Measured UE Trajectory is mandatory. The UE Trajectory IE should contain the cell the UE visited, including the target cell, and the information related to the UE trajectory measured in the visited cell? Whether to reuse UE history information?

We think that it would be good to have specific IE for UE Trajectory that is for AI/ML, as other information specific for AI/ML may be included e.g. the condition that trigger the measured UE Trajectory reporting, which will be more future proof..

Proposal 2: Introduce new Measured UE Trajectory IE specific for AI/ML, this is more future proof.

Whether the predicted UE Trajectory report needs to be indicated in the Report Characteristics IE of DATA COLLECTION REQUEST message is FFS.

On the need of indicating of Measured UE Trajectory in the Report Characteristics IE of the DATA COLLECTION REQUEST message, is to have the AI/ML measurement ID that to identify the DATA COLLECTION UPDATE message that will be received later. However, since it has been more or less agreeable also to indicate the AI/ML Measurement ID IE in the HANDOVER REQUEST message, the need of such indication may be less i.e. no strong need for indicating of Measured UE Trajectory in the Report Characteristics IE.

Proposal 3: No strong need to indicate the Measured UE Trajectory in the Report Characteristics IE

- Proposals

In this paper we discussed the stage 3 detail for UE Trajectory information.

Proposal 1: For the reporting of Measured UE Trajectory, explicit indication on conditions is signalled to the requesting node.

Proposal 2: Introduce new Measured UE Trajectory IE specific for AI/ML, this is more future proof.

Proposal 3: No strong need to indicate the Measured UE Trajectory in the Report Characteristics IE

- Reference

[1] RP-231159_New WI Artificial Intelligence (AI) Machine Learning (ML) for NG-RAN

[2] TR 37.817 Study on enhancement for Data Collection for NR and EN-DC

[3] R3-234573 SoD of CB:# AIRAN3_ME

| R3-234785 | (BLCR to 38.300) Draft CR to 38.300 on AI/ML for NG-RAN | Rel-18 | 38.300 | NR_AIML_NGRAN-Core |
| --- | --- | --- | --- | --- |
| R3-233789 | CR to TS 38.401 for addition of AI/ML-RAN feature in the case of split architecture | Rel-18 | 38.401 | NR_AIML_NGRAN-Core |
| R3-234786 | (BLCR to 38.423) for AI/ML for NG-RAN | Rel-18 | 38.423 | NR_AIML_NGRAN-Core |