3GPP TSG-RAN WG3 Meeting #121bis                                                     	R3-235303

Xiamen, China, Oct 9th -13th 2023

Agenda item:	10.2.2

Source:	Qualcomm Incorporated

Title:	MRO enhancements for CPAC and fast MCG recovery

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss remaining issues on MRO for CPAC and fast MCG recovery based on agreements and open issues identified last meeting.

# Discussion

  - MRO for CPA/CPC

RAN3 agreed the following in previous meetings:

Include CPC candidate cell list and CPC execution condition(s) to the message from the MN to the source SN

The MN performs the initial analysis when SCGFailureInformation is received from the UE e.g., whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated.

Further, the following two options were captured as open issues in the SoD last meeting.

MRO for CPA or MN initiated CPC

Option 1:

- If the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell was selected by the (candidate) target SN via SCG Failure Information Report message.

  - The following information are included in the message from the MN to the (candidate) target SN:

    - Suitable PSCell ID

    - Indication whether a candidate PSCell is accepted by the (candidate) target

- Otherwise, the MN makes the optimization

Option 2:

- MN always send the SCG Failure Information Report message to the last serving SN

- The last serving SN feedback if the problem is not brought by itself

- the MN makes the optimization

MRO for SN initiated CPC

Option 1:

- The MN sends SCG failure information report message to source SN, and source SN performs root cause analysis

- if the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN,

  - the source SN sends message to the MN, then the MN sends message to the (candidate) target SN.

- Otherwise,

  - the source SN makes the optimization

- SCG FAILURE TRANSFER is used from the source SN to the MN

- SCG Failure Information Report message is used from the MN to the (candidate) target SN

Option 2: the same as the option 2 for MN initiated CPC.

Optimizing the candidate cells in case of CPA/CPC

We think the node which made the “mistake” (i.e., either the initiating node in proposing the list of candidate cells or the target SN in selecting the candidate cells among the proposed list) should be responsible for optimization and therefore the following are proposed:

Observation 1: The node which made the “mistake” (i.e., either the initiating node in proposing the list of candidate cells or the target SN in selecting the candidate cells among the proposed list) should be responsible for optimizing the candidate cells in case of CPA/CPC

Proposal 1: In case of CPA or MN initiated CPC, if the suitable PSCell is one of the candidate PSCells proposed by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, the (candidate) target SN is responsible for optimization. Otherwise, the MN is responsible for optimization.

Proposal 2: In case of SN initiated CPC, if the suitable PSCell is one of the candidate PSCells proposed by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN, the (candidate) target SN is responsible for optimization. Otherwise, the source SN is responsible for optimization.

Suppose it was the (candidate) target SN that made the mistake in selecting the wrong list of candidate cells among the proposed list. While performing optimization, the (candidate) target SN might have forgotten the list of cells which it accepted among the proposed list. We therefore propose the following:

Proposal 3: MN should also provide an indication whether the candidate PSCell was accepted by the target SN while providing the list of candidate PSCells from MN to each (candidate) target SN.

Whether to support pre-Rel-17 UEs for MRO of CPAC

In Rel-17, a two-step procedure was introduced to support pre-Rel-17 UEs (first step to check with last serving SN whether there were intra-SN PSCell changes without MN involvement) and if the last serving SN responds via SCG FAILURE TRANSFER that the problem is not brought by itself, MN sends the SCG FAILURE INFORMATION REPORT to the source SN.

Observation 2: In Rel-17, a two-step procedure was introduced in case of MRO for PSCell change failures to support pre-Rel-17 UEs (which don’t have enhanced SCGFailureInformation)

- MN first checks with last serving SN whether there were intra-SN PSCell change failures via SCG FAILURE INFORMATION REPORT

- If the last serving SN responds via SCG FAILURE TRANSFER that the problem is not brought by itself (i.e., there are no intra-SN PSCell change failures), MN sends the SCG FAILURE INFORMATION REPORT to the source SN

Similar to Rel-17, we think MN should be able to identify if there was an intra-SN PSCell change failure before a CPC execution (irrespective of MN/SN initiated CPC) to correctly identify the “problem causing node”.

Proposal 4: MN should be able to identify if there was an intra-SN PSCell change failure before a CPC execution, even for pre-Rel-17 UEs.

Proposal 5: Reuse Rel-17 procedures for MRO for CPC, i.e.,

- MN always sends the SCG FAILURE INFORMATION REPORT message to the last serving SN.

- The last serving SN may feedback that the problem is not brought by itself via SCG FAILURE TRANSFER

- If SCG FAILURE TRANSER is received, MN sends the SCG FAILURE INFORMATION REPORT to the source SN or (candidate) target SNs

In case of CPA, there can be no intra-SN PSCell change before the CPA execution as there is no serving SN.

Observation 3: In case of CPA, there can be no intra-SN PSCell change before the CPA execution as there is no serving SN.

Proposal 6: In case of CPA, either the MN performs the optimization or MN sends the SCG FAILURE INFORMATION REPORT to (candidate) target SNs i.e., there is no need to use SCG FAILURE TRANSFER to check if there were intra-SN PSCell changes before the CPA

  - MRO for Fast MCG Recovery

There is one open issue (which RAN2 is also discussing) on whether to also add the time between MCG failure and SCG failure in RLF Report

In our view, the “time between MCG failure and SCG failure” might not be very useful in our opinion as RLF and SCG failure are independent events.

gNB can’t optimize much knowing this exact timer value and this just provides a random statistical knowledge. We therefore think there is no need to include “time between MCG failure and SCG failure” in RLF Report

Observation 4: RLF and SCG failure are independent events. There is no benefit in knowing the time between these independent events as no guaranteed optimization can be done with such information.

Proposal 7: There is no need to include “time between MCG failure and SCG failure” in RLF Report

# Conclusion

Optimizing the candidate cells in case of CPA/CPC

Observation 1: The node which made the “mistake” (i.e., either the initiating node in proposing the list of candidate cells or the target SN in selecting the candidate cells among the proposed list) should be responsible for optimizing the candidate cells in case of CPA/CPC

Proposal 1: In case of CPA or MN initiated CPC, if the suitable PSCell is one of the candidate PSCells proposed by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, the (candidate) target SN is responsible for optimization. Otherwise, the MN is responsible for optimization.

Proposal 2: In case of SN initiated CPC, if the suitable PSCell is one of the candidate PSCells proposed by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN, the (candidate) target SN is responsible for optimization. Otherwise, the source SN is responsible for optimization.

Proposal 3: MN should also provide an indication whether the candidate PSCell was accepted by the target SN while providing the list of candidate PSCells from MN to each (candidate) target SN.

Whether to support pre-Rel-17 UEs for MRO of CPAC

Observation 2: In Rel-17, a two-step procedure was introduced in case of MRO for PSCell change failures to support pre-Rel-17 UEs (which don’t have enhanced SCGFailureInformation)

- MN first checks with last serving SN whether there were intra-SN PSCell change failures via SCG FAILURE INFORMATION REPORT

- If the last serving SN responds via SCG FAILURE TRANSFER that the problem is not brought by itself (i.e., there are no intra-SN PSCell change failures), MN sends the SCG FAILURE INFORMATION REPORT to the source SN

Proposal 4: MN should be able to identify if there was an intra-SN PSCell change failure before a CPC execution, even for pre-Rel-17 UEs.

Proposal 5: Reuse Rel-17 procedures for MRO for CPC, i.e.,

- MN always sends the SCG FAILURE INFORMATION REPORT message to the last serving SN.

- The last serving SN may feedback that the problem is not brought by itself via SCG FAILURE TRANSFER

- If SCG FAILURE TRANSER is received, MN sends the SCG FAILURE INFORMATION REPORT to the source SN or (candidate) target SNs

Observation 3: In case of CPA, there can be no intra-SN PSCell change before the CPA execution as there is no serving SN.

Proposal 6: In case of CPA, either the MN performs the optimization or MN sends the SCG FAILURE INFORMATION REPORT to (candidate) target SNs i.e., there is no need to use SCG FAILURE TRANSFER to check if there were intra-SN PSCell changes before the CPA.

MRO for Fast MCG Recovery

Observation 4: RLF and SCG failure are independent events. There is no benefit in knowing the time between these independent events as no guaranteed optimization can be done with such information.

Proposal 7: There is no need to include “time between MCG failure and SCG failure” in RLF Report

# 4. References