3GPP TSG-RAN WG3 #121bis								R3-235318

Xiamen, China, 9th – 13th Oct 2023

Agenda Item:	31.2

Source:	China Telecom, Ericsson, ZTE

Title: 	Support of less than 5MHz Bandwidth for FR1 over Xn and F1

Document for:	Discussion

Introduction

The WID on NR support for dedicated spectrum less than 5MHz for FR1 was agreed in Rel-18 [1]. According to the status report of this WI submitted in RAN#101 meeting [2], the completion level of core part is 95% and RAN1 work was closed. Currently, RAN3 is not involved in this WID. Per latest RAN4 specifications, the minimum bandwidth of channels is 5MHz. One of objectives in this WID is to introduce a new 3MHz Channel Bandwidth. From the perspective of RAN3, the channel bandwidth information or cell bandwidth need to be exchanged over Xn and F1 interface. Therefore, there is need to discuss the impact of 3MHz Channel Bandwidth on Xn and F1 interface.

Discussion

Conclusions in RAN4

The CR on BS RF requirements was agreed in RAN4#108 meeting [3], where the agreements on BS RF requirements for NR <5MHz are specified.

### 5.3.2	Transmission bandwidth configuration

The transmission bandwidth configuration NRB for each BS channel bandwidth and subcarrier spacing is specified in table 5.3.2.-1 for FR1 and table 5.3.2-2 for FR2.

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Where 3MHz Channel Bandwidth and 15 PRB Transmission Bandwidth were introduced in TS38.104.

RAN3 impact

- F1 interface

Since the concept of Channel Bandwidth in F1 interface is only applied in NR-U, there is no need to update the definition of Channel Bandwidth IE. The NRB IE in UL/DL Transmission Bandwidth indicates the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 38.104 [17]).  Therefore, it is proposed to introduce a new codepoint, i.e.,15PRB, to the existing Transmission Bandwidth IE.

Proposal 1: To introduce a new codepoint, i.e.,15PRB, to the existing Transmission Bandwidth IE.

- Xn interface

Similar to F1 interface, the NRB IE in NR Transmission Bandwidth indicates the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 38.104 [17]).  Therefore, it is proposed to introduce a new codepoint, i.e.,15PRB, to the existing NR Transmission Bandwidth IE.

Proposal 2: To introduce a new codepoint, i.e.,15PRB, to the existing NR Transmission Bandwidth IE.

The corresponding CRs on XnAP and F1AP are provided in [4] and [5].

Proposal 3: to agree XnAP CR and F1AP CR.

Since the current WID does not involve RAN3, we need to provide a LS to RAN1/2/4 to inform the RAN3 endorsed CRs. The draft LS to RAN1/RAN2/RAN4 are provided in [6].

Proposal 4: to agree the draft LS to RAN1/RAN2/RAN4.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: To introduce a new codepoint, i.e.,15PRB, to the existing Transmission Bandwidth IE.

Proposal 2: To introduce a new codepoint, i.e.,15PRB, to the existing NR Transmission Bandwidth IE.

Proposal 3: to agree XnAP CR and F1AP CR.

Reference

- RP-231713, Revised WID: NR support for dedicated spectrum less than 5MHz forFR1, Nokia, Nokia Shanghai Bell

- RP-231712	Status report for WI NR support for dedicated spectrum less than 5MHz for FR1; rapporteur: Nokia

- R4-2311670, “Big CR to TS 38.104 on introduction of 3 MHz channel bandwidth”, Nokia, Nokia Shanghai Bell.

- R3-235319, CR to TS38.423 on introduction of 3 MHz channel bandwidth, Ericsson, China Telecom, ZTE

- R3-235328, CR to TS38.473 on introduction of 3 MHz channel bandwidth, ZTE, China Telecom, Ericsson

- R3-235329, draft LS on introduction of 3MHz Channel Bandwidth over Xn and F1 interface, China Telecom