3GPP TSG-RAN WG3 Meeting #121bis	R3-235320

Xiamen, China, 9th– 13th October 2023

Agenda Item:	8.3

Source:	ZTE

Title:	Discussion on pre-configured measurement gap for RedCap UE

Document for:	Discussions

1	Introduction

In last meeting, RAN2 has replied the LS on applicability of pre-configured measurement gaps for RedCap UE in [1].

In this contribution, we provide our understanding on this issue and give more clarification on the SI delivery issue for RedCap UE.

- Discussion

- 2.1        Pre-configured measurement gap for RedCap UE

In the reply LS from RAN2 on the pre-configured measurement gap for RedCap UE, RAN2 has confirmed that only one pre-configured measurement gap is allowed per gap type in Rel-17, and the details are given as below.

RAN2 discussed following RAN3 question: Based on “gapToAddModList IE” definition in MeasGapConfig, RAN3 would like to ask RAN2 whether it is possible that this IE can be used to configure different Pre-MGs for different BWPs, since the RedCap UE will require a MG whenever it performs a measurement outside its active BWP. RAN2 Answer: RAN2 would like to inform RAN3 that due to RAN4 restrictions, only one pre-configured measurement gap is allowed per gap type (i.e, UE type or per FR type), by using “gapToAddModList IE” in MeasGapConfig. RAN2 would like to inform RAN3 that pre-configured MGs are applicable to both RedCap and non-RedCap UEs.

Considering the solution to active BWP swithching via DCI for RedCap UEs in [2], based on the RAN2’s reply, the Preconfigured measurement Gap Request List in the CU to DU RRC Information should be removed since it cannot be supported over Uu in Rel-17.

In addition, it has been agreed in RAN3#117bis that the gNB-DU shall decide the gap type [3]. Since the gNB-DU knows the UE capability, and it also knows the BWP configuration as well as the physical layer configuration, the gNB-DU doesn’t need to expose the BWP configurations to the gNB-CU. Therefore, the Configured BWP List from the gNB-DU to the gNB-CU may be not needed.

However, one potential enhancement is that the gNB-CU provides the candidate measured frequencies per servingCellMO to the gNB-DU, then the gNB-DU is able to generate suitable gap patterns. While, since there are still some unclear understanding, the whole F1 signalling to support the active BWP switching via DCI should be further discussed.

Proposal 1: For measurement gap configuration for RedCap UE, the gNB-DU doesn’t need to expose the BWP configuratins to the gNB-CU.

- 2.2        SI delivery for RedCap UE during handover

When a non-RedCap UE accesses a first active DL BWP in the target cell upon handover, the first active BWP is either the initial BWP or the BWP associated with CD-SSB. Therefore, there is no problem for it to receive broadcast information in the target cell.

- In Rel-17, the NCD-SSB feature is introduced for RedCap UEs. A RedCap UE may not receive broadcast information in target cell upon handover because the first active DL BWP associated with an NCD-SSB is not configured with CSS (common search space). In this case, if needed, SIBs information can be provided via dedicated signaling,e.g,HO command to the UE, In this way, the UE can receive system information from the target cell in time during the handover.

- However, for a CU/DU split gNB, the CU is not aware of whether the UE can receive broadcast information in the target cell during handover. That is, the CU does not know the UE's first active DL BWP and the NCD-SSB/CSS information associated with this BWP. Then, there is no way for the gNB to decide whether there is a need to send SIBs to UE via HO command.

- To support NG-RAN node determining the delivery of system information via HO command, it is proposed that CU can initiate a request to ask DU whether the UE can receive broadcast information in the first active BWP of the target cell during handover.

As mentioned in the previous section, the mechanism of pre-configured measurement gap configuration for RedCap UE is still FFS, an individual correction is needed to solve this SI delivery issue in Rel-17.

Proposal 2: To solve the SI delivery issue for RedCap UE during handover in the CU-DU split architecture, it is proposed to introduce the indication on whether the UE can receive system information from broadcast in the first active BWP of the target cell.

Conclusion

Proposal 1: For measurement gap configuration for RedCap UE, the gNB-DU doesn’t need to expose the BWP configuratins to the gNB-CU.

Proposal 2: To solve the SI delivery issue for RedCap UE during handover in the CU-DU split architecture, it is proposed to introduce the indication on whether the UE can receive system information from broadcast in the first active BWP of the target cell.

The corresponding F1AP CR on the SI delivery to RedCap UE during handover is given in [4].

References

- R3-234560, Reply LS on applicability of pre-configured measurement gap for RedCap UE, RAN2

- R3-233058, Support of preconfigured Measurement GAPs for RedCap UE, Ericsson, Qualcomm Incorporated, CATT

- RAN3#117bis Chairman Notes

- R3-235321, Correction on SI delivery to RedCap UE during handover, ZTE