3GPP TSG-RAN WG3 Meeting #121bis	R3-235333

Xiamen, China, October 9 – 13, 2023

Agenda item:	14.4

Source:	Samsung

Title:	Discussion on selective activation of the cell groups

Document for:	Discussion & Decision

Introduction

In the last meeting, some agreements were achieved. However there are still some issues that need further discussion and our considerations are given in this contribution.

How to transfer the data forwarding addresses to the selected SN to start early data forwarding?

How to support reference configuration?

How to support execution conditions?

New indicators in S-NODE MODIFICATION REQUEST message, UE CONTEXT SETUP/MODIFICAION REQUEST message and BEARER CONTEXT SETUP REQUEST message?

Discussion

How to transfer the data forwarding addresses to the selected SN to start early data forwarding

According to the discussion in the last meeting, there are three alternatives to transfer the data forwarding addresses to the selected SN to start early data forwarding when it needs:

-Alt 1: single enhanced Xn-U Address Indication procedure which carries the data forwarding addresses of all other candidate SNs

-Alt 2: multiple Xn-U Address Indication procedures, and each procedure carries data forwarding addresses of one candidate SN

-Alt 3: single SN RRC Reconfiguration Complete procedure which carries the data forwarding addresses of all other candidate SNs

In the procedure of CPC in Rel-17, when the MN receives the RRCReconfigurationComplete message from UE, it will transfer the SN Reconfiguration Complete message to the target SN. Therefore, it seems straightforward to carry the data forwarding addresses of the other candidate SNs in the SN Reconfiguration Complete message for the target SN to start early data forwarding.

Observation 1: It seems straightforward to carry the data forwarding addresses of the other candidate SNs in the SN Reconfiguration Complete message for the target SN to start early data forwarding.

Proposal 1: Enhancement on the SN Reconfiguration Complete message is needed to provide the data forwarding addresses of the other candidate SNs to the selected SN in S-CPAC.

How to support execution conditions?

There might be two options to support the execution condition configuration for Subsequent CPAC.

Option1: The other candidate PSCells and corresponding execution conditions are sent from the candidate target SNs to the MN via SN ADDITION REQUEST ACK message.

Option2: To support candidate SN generating execution condition for Subsequent CPAC, RAN3 supports two rounds of configurations among network nodes before sending the Subsequent CPAC configuration to UE, 1. Candidate SCG configuration 2. Execution condition configuration.

From our side, option2 seems reasonable. For the execution condition for the S-CPAC can be generated by the candidate SN when it gets the other candidate PSCells from the MN. But the MN could only obtain the candidate PSCells finally decided by the candidate SNs after the SN addition request procedures. Therefore, two rounds of configurations among network nodes are needed, the first one for the candidate SCG configuration and the second one for execution condition configuration.

Proposal 2: Two rounds of configurations among network nodes are needed to support the execution condition configuration for Subsequent CPAC.

New indicators

Issue1: a new indicator over F1AP for selective activation.

According to the agreement above, RAN3 agreed to enhance XnAP and F1AP to support NR Selective Activation, and some agreements on XnAP signalling enhancement were achieved. Therefore, the F1AP signalling enhancement is also needed as that of XnAP interface. A new indicator should be added in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST for S-CPAC to distinguish from the legacy CHO/CPAC preparation. With the indicator, gNB-DU could be notified that the request is for S-CPAC, and the resource allocated should be reserved for a period of time, so it could decide whether to accept the request.

Proposal 3:   Introduce a new indicator in UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message to indicate that the request is for Subsequent CPAC.

Issue2:  a new indicator over E1AP for SCG selective activation.

CHO Initiation IE is contained in the BEARER CONTEXT SETUP REQUEST message to indicate that the request is for conditional handover or conditional PSCell change or conditional PSCell addition. To distinguish from the legacy conditional PSCell change or conditional PSCell addition, a new indicator should be introduced in BEARER CONTEXT SETUP REQUEST message to indicate that the request is for S-CPAC. With the indicator, gNB-CU-UP could be notified that the request is for S-CPAC, and the resource allocated should be reserved for a period of time, so it could determine whether to accept the request.

Proposal 4: Introduce a new indicator in BEARER CONTEXT SETUP REQUEST message to indicate that the request is for Subsequent CPAC.

Conclusion

Based on the discussion, we have the following proposals.

Observation 1: It seems straightforward to carry the data forwarding addresses of the other candidate SNs in the SN Reconfiguration Complete message for the target SN to start early data forwarding.

Proposal 1: Enhancement on the SN Reconfiguration Complete message is needed to provide the data forwarding addresses of the other candidate SNs to the selected SN in S-CPAC.

Proposal 2: Two rounds of configurations among network nodes are needed to support the execution condition configuration for Subsequent CPAC.

Proposal 3:   Introduce a new indicator in UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message to indicate that the request is for Subsequent CPAC.

Proposal 4: Introduce a new indicator in BEARER CONTEXT SETUP REQUEST message to indicate that the request is for Subsequent CPAC.

References