3GPP TSG-RAN WG3 Meeting #121bis	R3-235340

Xiamen, China, October 9 – 13, 2023

Agenda Item:	10.2.5

Source:	Huawei

Title:	(TP for SON BL CR for TS 38.300): SON for NR-U

Document for:	Other

1	Introduction

In the last RAN3 #121 meeting, the following agreements and open issues have been achieved in SON for NR-U:

Introduce an optional load metric on Radio Resource Status per NR-U Channel in XnAP RESOURCE STATUS UPDATE message and in F1AP RESOURCE STATUS UPDATE message. There is no need to transfer the UL EDT in F1AP RESOURCE STATUS UPDATE message. Continue the discussion on RLF MRO report optimization: - actual UL EDT - actual minimum UL EDT - lowest detected power For HO execution, FFS whether the source node can deduce from UE reports – excluding UL LBT failures information - whether the target node suffered from DL LBT issues during the HO execution.

In this paper, we provide our view on the RAN3 related issue and TPs.

2	Discussion

2.1 DL LBT issues during the HO execution

For HO execution, FFS whether the source node can deduce from UE reports – excluding UL LBT failures information - whether the target node suffered from DL LBT issues during the HO execution.

The FFS above is how to handle the HO execution failure with DL LBT issues. Before discussing the specific solution, we should clarify the specific scenario for DL LBT failure of target node during the HO execution. In the legacy handover procedure, the target gNB prepares the handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB, which includes a transparent container to be sent to the UE as an RRC message to perform the handover. Then, the source gNB triggers the Uu handover by sending an RRCReconfiguration message to the UE, containing the information required to access the target cell, which includes RACH resources, system information of the target cell, etc. Finally, the UE synchronises to the target cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to target gNB.

At the same time, during the above entire HO execution, the target gNB may suffer from DL LBT issues when it attempts to send SSB in NR-U channels.

However, actually, the SSB sent by the target gNB is cell-level, and is not associated with any individual UE who is handovering in. In our opinion, the DL LBT failure of the target gNB due to SSB transmission failure in NR-U channels does not only affect the HO execution of the UE but also all the UEs served by the target cell.

And it should further study and clarify the occruance possibility of HO failure or RLF due to SSB transmission failure. Considering the complexity of the network based solution and the limited time of the Rel-18 WI, we propose not to consider this issue in Rel-18.

Therefore, we propose:

The SSB transmission failure issue due to DL LBT failure during handover execution in NR-U channels is not pursued in Rel-18.

2.3 stage 2 TP

In the end, the MLB for NR-U is almost completed in RAN3, it is proposed to proceed the following stage 2 TP to TS 38.300:

#### 15.5.1.2	Load reporting for intra-RAT and intra-system inter-RAT load balancing

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces.

The following load related information should be supported which consists of:

-	Radio resource usage (per-cell and per SSB area PRB usage: DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, DL/UL total PRB usage, and DL/UL scheduling PDCCH CCE usage; PRB usage for slice(s): DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, and DL/UL Total PRB allocation);

-	TNL capacity indicator (UL/DL TNL offered capacity and available capacity);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-	Capacity value (per cell, per SSB area and per slice: UL/DL available capacity);

-	HW capacity indicator (offered throughput and available throughput over E1, percentage utilisation over F1);

-	RRC connections (number of RRC connections, and available RRC Connection Capacity);

-	Number of active UEs;

-	NR-U channel load (channel occupancy time percentage DL/UL, energy detection threshold DL/UL, per cell radio resource usage).

To agree on the stage 2 TP to the BLCR of TS 38.300.

3	Conclusion

In this contribution, we have discussed the remaining issuss about SON for NR-U and propose the following.

- The SSB transmission failure issue due to DL LBT failure during handover execution in NR-U channels is not pursued in Rel-18.

To agree on the stage 2 TP to the BLCR of TS 38.300.

The stage 2 TP is provided in the annex.

Annex1	TP for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 15.5.1.2	Load reporting for intra-RAT and intra-system inter-RAT load balancing

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces.

The following load related information should be supported which consists of:

-	Radio resource usage (per-cell and per SSB area PRB usage: DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, DL/UL total PRB usage, and DL/UL scheduling PDCCH CCE usage; PRB usage for slice(s): DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, and DL/UL Total PRB allocation);

-	TNL capacity indicator (UL/DL TNL offered capacity and available capacity);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-	Capacity value (per cell, per SSB area and per slice: UL/DL available capacity);

-	HW capacity indicator (offered throughput and available throughput over E1, percentage utilisation over F1);

-	RRC connections (number of RRC connections, and available RRC Connection Capacity);

-	Number of active UEs;

-	NR-U channel load (channel occupancy time percentage DL/UL, energy detection threshold DL/UL, per cell radio resource usage).

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>