3GPP TSG-RAN WG2 #121bis	R3-235350

9th – 13th Oct 2023 Xiamen, China

Agenda item:		14.4

Source:				CATT

Title:	Discussion on subsequent CPAC

Document for:		Discussion and Decision

1		Introduction

In RAN3 #121 meeting, the below agreements and open issues are captured in [1] as below.

RAN3 follows RAN2 and uses the same name “subsequent CPAC” or “S-CPAC” for the selective activation.

Configuration of Subsequent CPAC

The information of other candidate PSCells in other candidate target SNs should be provided in the SN Addition Request message/SN Modification Request message from the MN to the candidate target SNs, if RAN2 agrees.

After CPC execution from the source SN to other SN, the MN should inform the source SN of the CPC execution and start late data forwarding.

- Case 1: if the source SN is configured as a candidate SN for subsequent CPAC, the source SN needs to be informed.

FFS on which procedure to inform.

- Case 2: if the source SN is not configured as a candidate SN for subsequent CPAC, the source SN is released.

The SN reconfiguration complete message is sent to the selected SN after CPC execution.

How to transfer the data forwarding addresses to the selected SN to start early data forwarding?

How to support reference configuration?

How to support execution conditions?

New indicators in S-NODE MODIFICATION REQUEST message, UE CONTEXT SETUP/MODIFICAION REQUEST message and BEARER CONTEXT SETUP REQUEST message?

And in RAN2#123 meeting, the below agreements for selective SCG are captured.

For subsequent CPAC it is useful to support use of A3 A5

A3 A5 is supported with SN-initiated subsequent CPAC

Proposal 1: For MN-initiated subsequent CPAC,  MN initially triggers the candidate cell preparation of subsequent CPAC procedure, i.e. MN triggers the procedure as defined in Section 10.5.2 and Section 10.2.2 of TS 37.340 in the endorsed running CR.

Proposal 2: For SN-initiated inter-SN subsequent CPAC, SN initially triggers the candidate cell preparation of subsequent CPAC procedure, i.e. source SN triggers the procedure as defined in Section 10.5.2 of TS 37.340 in the endorsed running CR.

Proposal 3 (option2): For MN-initiated subsequent CPAC, the execution condition configuration is provided as following:

- MN generates the execution conditions (A4 event) for initial CPAC execution, and the measID refers to the measurement configuration associated with MCG;

- candidate SN generates the execution conditions (A3/A5 event)  for subsequent CPC execution, and the measID refers to the measurement configuration associated with SCG.

UE autonomously releases the subsequent CPAC configurations in the following cases: upon RRC re-establishment and RRC release (to RRC_IDLE and/or RRC_INACTIVE)

No need for an optimized single-indication-release of CPAC configuration. Can rely on explicit release for other cases.

Will support the SA3 solution, i.e. update of Sk-counter at inter-SN-mobility, based on pre-configured multiple Sk-counter. UE need to know when Sk counter need to change.

Detailed solution discussed in long Post-meeting email discussion

This contribution will continue discussing the selective activation for cell group based on above agreements and open issue list.

2		Discussion

How to support reference configuration?

RAN2 agreed: The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

According the agreements, the reference configuration should be sent from the node which initially generates it to all candidates. Whatever which node generates it, the reference configuration should be transfer from MN to candidate SNs via S-NODE Addition Request message when the SN added. In R17 and before version, the CG-ConfigInfo IE carried in M-NG-RAN node to S-NG-RAN node Container IE is used to transfer the RAN2 defined CG information from MN to SN. So the reference configuration also can be transferred in this IE from MN to SN.

Proposal 1: Transferring reference configuration from MN to SN in the container in which including RAN2 defined CG-ConfigInfo

The SN will be confirmed whether the reference configuration is used for the SCG delta configuration. The indication can be within the CG-CandidateList IE in the S-NG-RAN node to M-NG-RAN node Container IE in S-NODE Addition Request Acknowledge message.

Proposal 2: The indication for denoting whether the reference configuration is used can be within the CG-CandidateList IE in the S-NG-RAN node to M-NG-RAN node Container IE.

How to support execution conditions?

RAN2 agreed:

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

For MN-initiated subsequent CPAC, the execution condition configuration is provided as following: MN generates the execution conditions (A4 event) for initial CPAC execution, and the measID refers to the measurement configuration associated with MCG; candidate SN generates the execution conditions (A3/A5 event)  for subsequent CPC execution, and the measID refers to the measurement configuration associated with SCG.

According these agreements, all candidate SN nodes generated the execution conditions for all subsequent CPC execution. RAN2 is discussing the information transferred among these nodes. They may include this information in the CG-ConfigInfo and CG-CandidateList . It should not impact RAN3 spec. But we need wait for RAN2 progress

Proposal 3: candidate SN generates execution conditions for subsequent CPC should not impact RAN3 spec.

How to transfer the data forwarding addresses to the selected SN to start early data forwarding?

The selected SN should be informed the data forwarding address of other configured and kept candidate SNs after one candidate executed to support the early data forwarding. So then the new SN can perform the early data forwarding to those candidate SNs. We agreed: Reuse the Xn-U Address Indication message and the Early Status Transfer message to support early data forwarding for subsequent CPAC. The data forwarding address of the candidate may be sent to each other at the CPAC configuration phase for early data forwarding, or for late data forwarding after one candidate executed.

Proposal 4: The data forwarding address of the candidates is sent to each other via Xn-U Address Indication message at the CPAC configuration phase or after one candidate executed.

The candidate SN CU-CP should inform the CU-UP via E1 about the data forwarding information when the data forwarding address is changed.

Proposal 5: The Bearer Context Modification (gNB-CU-CP initiated) procedure can be used for the updating of data forwarding address on E1

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: Transferring reference configuration from MN to SN in the container in which including RAN2 defined CG-ConfigInfo

Proposal 2: The indication for denoting whether the reference configuration is used can be within the CG-CandidateList IE in the S-NG-RAN node to M-NG-RAN node Container IE.

Proposal 3: candidate SN generates execution conditions for subsequent CPC should not impact RAN3 spec.

Proposal 4: The data forwarding address of the candidates is sent to each other via Xn-U Address Indication message at the CPAC configuration phase or after one candidate executed.

Proposal 5: The Bearer Context Modification (gNB-CU-CP initiated) procedure can be used for the updating of data forwarding address on E1

4		Reference

- RAN3_121_agenda_20230825_EOM