3GPP TSG-RAN WG3 Meeting #121-bis	R3-235359

Xiamen, CN, 09–13 Oct, 2023

Agenda Item:	12.2.2.2

Source:	CMCC

Title:	Discussion on Measured UE Trajectory Collection

Document for:	Discussion and Decision

# 1 Introduction

In the last RAN3 meeting, the details of Predicted UE Trajectory and the procedure for measured UE trajectory collection were discussed, the following agreements were achieved:

The measurement object to request the report of the UE Trajectory IE should be introduced in the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message.

The AI/ML Measurement ID in HANDOVER REQUEST from source gNB is reused to trigger the target node feedback the UE trajectory information to the source gNB.

AI/ML INFORMATION UPDATE from target node is used to convey the UE trajectory information.

The Measured UE Trajectory reporting is one-time report.

The conditions triggering the one-time reporting are at least:

- Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE);

- Condition B: UE leaves the target node;

- Condition C: Expiration of the report time duration;

- Condition D: The configured number of hand-overs within the same target node is reached.

Report time duration and configured number of intra-NG-RAN node inter-cell handovers are included in DATA COLLECTION REQUEST message.

The list of Cell IDs included in the Measured UE Trajectory is mandatory.

And the following issues were captured for predicted UE Trajectory report and measured UE trajectory collection and need to be continued in next meeting:

Whether an explicit or implicit indication on condition A and condition B or any other condition is signalled to the requesting node needs further discussion.

The presence of time of stay in the Measured UE Trajectory is FFS.

Whether to re-use the UE History Information IE or introduce a new IE or re-use the IE defined for predicted UE Trajectory is FFS.

Whether the predicted UE Trajectory report needs to be indicated in the Report Characteristics IE of DATA COLLECTION REQUEST message is FFS.

In this contribution, we would further discuss on the issues of Predicted UE Trajectory and the measured UE trajectory collection and provide some proposals.

# 2 Discussion

2.1 Predicted UE Trajectory

RAN3#120 has agreed that the UE trajectory prediction is only limited to the next one hop target NGRAN node in Rle-18 as below:

In Rel18, the UE trajectory prediction is only limited to the next one hop target NGRAN node, and the source NGRAN node can collect measured UE trajectory (in the form of UE History Information) related to one or multiple cells within the single next hop target NGRAN node after handing over the concerned UE(s).

If there are more than one next one hop target NGRAN node for a UE in the predicted UE trajectory (e.g. with the different probability) , how to provide the information in the predicted UE Trajectory need to be considered. We think there are two potential solutions as below:

- Opt1. Add the probability information into the predicted UE Trajectory, each of the possible next one hop target NGRAN node is indicated together with a probability.

- Opt2. The predicted UE Trajectory is limited to one next one hop target NGRAN node which has the maximum probability.

Compared with Opt1, Opt2 has no extra requirements for AI/ML model and also can make the signaling for predicted UE Trajectory more simpler.

Proposal 1: The Predicted UE Trajectory is limited to one single target NG-RAN node which has the maximum probability.

There is an issue captured for predicted UE Trajectory report in the last RAN3 meeting as shown below:

Whether the predicted UE Trajectory report needs to be indicated in the Report Characteristics IE of DATA COLLECTION REQUEST message is FFS.

In our understanding, the Report Characteristics IE in DATA COLLECTION REQUEST message is used as the indication of the measurement object requested by source gNB for the report of the Measured UE Trajectory or the other UE performance information. And the AI/ML Measurement ID in HANDOVER REQUEST is used to trigger the Measured UE trajectory information or UE performance feedback. If the Report Characteristics IE is reused to trigger the predicted UE Trajectory report from the source gNB, it will lead to confusion and complexity for DATA COLLECTION REQUEST message.

Proposal 2: The Report Characteristics IE of DATA COLLECTION REQUEST message can not be used as the indication of the predicted UE Trajectory report to trigger the predicted UE Trajectory from the source gNB.

From our point of view, the existing procedure is enough for source node to sent the predicted UE Trajectory to target node. HANDOVER REQUEST message is used to convey the predicted UE Trajectory which is as the reference for the resource reservation in target node and there is no need to introduce an additional indication to trigger the predicted UE Trajectory report.

Proposal 3: There is no need to introduce an additional indication to trigger the predicted UE Trajectory report.

2.2 Measured UE Trajectory Collection

The conditions triggering the one-time reporting are at least:

- Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE);

- Condition B: UE leaves the target node;

- Condition C: Expiration of the report time duration;

- Condition D: The configured number of hand-overs within the same target node is reached.

Report time duration and configured number of intra-NG-RAN node inter-cell handovers are included in DATA COLLECTION REQUEST message.

In light of the above agreement achieved in RAN3#121, we can see some common understanding on the conditions triggering the measured UE trajectory reporting as below:

- Once one of the conditions is satisfied, the target node should sent the measured UE trajectory report to the source node. Especially after Condition A or B is satisfied, the target node can not measure the visited cells and time of stay for the UE any more. The target node doesn’t need to keep the measured UE trajectory report for some time and decide another exact time to sent the measured UE trajectory report to the source node. It will make the target node implementation simple. Therefore, we can see that Condition A~D are mandatory for the target node.

- For Condition C, the start of report time duration is the time instant the UE handed over into the target node and the end of report time duration corresponds to the configuration of time duration.

- Based on the above analysis, we think the following clarifications should be considered:

- Proposal 4: Condition A~D triggering the measured UE trajectory reporting are mandatory for the target node.

Proposal 5: When one of the conditions is satisfied, the target node should sent the measured UE trajectory report to the source node at once.

Proposal 6: For Condition C, the start of report time duration is the time instant of successful Handover Execution and the end of report time duration corresponds to the configuration of time duration.

- According to the agreement achieved in RAN3#121 as below, a new IE is introduced in the Data Collection Request for indicating the Reporting Duration for the UE Performance Feedback.

A new IE is introduced in the Data Collection Request, indicating the Reporting Duration for the UE Performance Feedback, starting at successful Handover Execution.

- Considering that the Reporting Duration for the UE Performance Feedback and for the Measured UE Trajectory may be different based on the different usage requirements for these two type of feedback information, it is proposed to introduced a dedicated IE for Report time duration of the Measured UE Trajectory in DATA COLLECTION REQUEST message. It will make the configuration for the UE Performance Feedback and the UE Performance Feedback more flexible and clearer.

- Proposal 7: To introduce a dedicated IE for Report time duration of the Measured UE Trajectory in DATA COLLECTION REQUEST message.

Whether an explicit or implicit indication on condition A and condition B or any other condition is signalled to the requesting node needs further discussion.

We think it is helpful for AI/ML model inference of UE Trajectory Prediction to know which condition is satisfied and trigger the measured UE trajectory report. For example, Cell 1 and Cell 2 within the target node are included in the cell list in the predicted UE Trajectory, but only Cell 1 is included in the measured UE trajectory since the UE handed over from Cell 1 to another cell which belongs to a 3rd node, that means Condition B is satisfied. The source node need the cause value to identify the actual UE Trajectory is different from the predicted UE Trajectory. So it is proposed to introduce an additional IE into measured UE trajectory information of DATA COLLECTION UPDATE message for the indication of the satisfied condition (cause value) triggering the measured UE trajectory reporting. Considering at least Condition A, B and C can be reused to trigger the reporting of UE performance feedback, the indication of the satisfied condition (cause value) is applicable to both the measured UE trajectory reporting and UE performance feedback.

Proposal 8: To introduce an additional IE into measured UE trajectory information of DATA COLLECTION UPDATE message for the indication of the satisfied condition (cause value) triggering the measured UE trajectory reporting and UE performance feedback reporting.

The list of Cell IDs included in the Measured UE Trajectory is mandatory.

The presence of time of stay in the Measured UE Trajectory is FFS.

Whether to re-use the UE History Information IE or introduce a new IE or re-use the IE defined for predicted UE Trajectory is FFS.

Regarding to the content of the Measured UE Trajectory, we think time of stay is mandatory. Different from the predicted UE Trajectory, it is not capability constraint for the target node to measure the time of stay for RRC_CONNECTED UEs in the coverage of the target NG-RAN node just like the same way for UE History Information. Therefore, the predicted UE Trajectory IE can not be reused for the Measured UE Trajectory and a new IE is needed.

Proposal 9: The time of stay in the Measured UE Trajectory is mandatory.

Proposal 10: The Measured UE Trajectory has the same structure as UE History Information IE and a new IE is introduced in DATA COLLECTION UPDATE message.

# 3 Conclusion

In this paper, we discuss on the details of Predicted UE Trajectory and the measured UE trajectory, and we have the following proposals:

For the Predicted UE Trajectory:

Proposal 1: The Predicted UE Trajectory is limited to one single target NG-RAN node which has the maximum probability.

Proposal 2: The Report Characteristics IE of DATA COLLECTION REQUEST message can not be used as the indication of the predicted UE Trajectory report to trigger the predicted UE Trajectory from the source gNB.

Proposal 3: There is no need to introduce an additional indication to trigger the predicted UE Trajectory report.

For the Measured UE Trajectory:

- Proposal 4: Condition A~D triggering the measured UE trajectory reporting are mandatory for the target node.

Proposal 5: When one of the conditions is satisfied, the target node should sent the measured UE trajectory report to the source node at once.

Proposal 6: For Condition C, the start of report time duration is the time instant of successful Handover Execution and the end of report time duration corresponds to the configuration of time duration.

- Proposal 7: To introduce a dedicated IE for Report time duration of the Measured UE Trajectory in DATA COLLECTION REQUEST message.

Proposal 8: To introduce an additional IE into measured UE trajectory information of DATA COLLECTION UPDATE message for the indication of the satisfied condition (cause value) triggering the measured UE trajectory reporting and UE performance feedback reporting.

Proposal 9: The time of stay in the Measured UE Trajectory is mandatory.

Proposal 10: The Measured UE Trajectory has the same structure as UE History Information IE and a new IE is introduced in DATA COLLECTION UPDATE message.

# Reference

- RAN3_121_agenda_20230825_EOM1