3GPP TSG-RAN WG3 Meeting #121-bis	R3-235360

Xiamen, CN, 09–13 Oct, 2023

Agenda Item:	12.2.2.2

Source:	CMCC

Title:	(TP for TS38.423) Measured UE Trajectory Collection

Document for:	Discussion and Approval

# 1 Introduction

This paper provides text proposal to request the measured UE trajectory collection from source node as discussed in paper R3-235359.

# 2 Text Proposal based on TS38.423 BLCR

***********************************   Start of 1st Change**********************************

8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

-	FFS

If the Registration Request IE is set to “start” in the DATA COLLECTION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all of the requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable to provide some but not all of the requested information, it shall initiate the AI/ML related information reporting for the admitted requested information and include the Node Measurement Initiation Result IE or the Per Cell Measurement Initiation Result IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the information requested applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the Measured UE Trajectory Reporting Configuration IE (FFS on the name)  in the DATA COLLECTION REQUEST message is present, it indicates the conditions triggering the reporting of measured UE trajectory, that is to address the scenario in which the report time duration expired or the configured number of hand-overs within the same target node reached. The NG-RAN node2 shall report only once.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the DATA COLLECTION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements or prediction on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, “Predicted Radio Resource Status” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”. FFS on the details of Predicted Radio Resource Status IE.

-	the Predicted Number of Active UEs IE, if the second bit, “Predicted Number of Active UEs” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”;

-	the Predicted RRC Connections IE, if the third bit, “Predicted RRC Connections” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Average UE Throughput DL IE, if the fourth bit, “Average UE Throughput DL” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Average UE Throughput UL IE, if the fifth bit, “Average UE Throughput UL” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Average Packet Delay IE, if the sixth bit, “Average Packet Delay” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Average Packet Loss IE, if the seventh bit, “Average Packet Loss” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Energy Cost IE, if the eighth bit, “Energy Cost” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Measured UE Trajectory IE, if the ninth bit, “Measured UE Trajectory” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

***********************************   End of 1st Change**********************************

***********************************   Start of 2nd Change**********************************

9.1.3.CC	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (0..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. For periodic reporting, it indicates the points in time, measured from the reception of the DATA COLLECTION REQUEST message and shifted by each reporting period, for which predictions are provided. | YES | ignore |
| Measured UE Trajectory Reporting Configuration (FFS on the name) |  |  | 9.2.3.KK |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

***********************************   End of 2nd Change**********************************

***********************************   Start of 3rd Change**********************************

9.1.3.FF	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item(FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| UE Associated Info Result |  | 0..1 |  |  | YES | FFS |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >> UE Assistant Identifier (FFS on the name) | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1 | – |  |
| >>UE Performance | O |  | 9.2.3.Y |  | – |  |
| >>UE trajectory | O |  | 9.2.3.N1 |  | – |  |
| >>Cause Value | O |  | BITSTRING (SIZE(2)) | 2 bits indicate the Condition which trigger the Measured UE Trajectory Reporting “00” = Switch of UE state “01” = UE leaves the target node “10” = Expiration of the report time duration “11” = The configured number of hand-overs within the same target node is reached FFS on semantics text |  |  |
| Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. Energy Consumption is measured on a linear scale. (FFS) | YES | FFS |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

***********************************   End of 3rd Change**********************************

***********************************   Start of 4th Change**********************************

    - 9.2.3.KK	Measured UE Trajectory Reporting Configuration

The Measured UE Trajectory Reporting Configuration IE contains Report time duration and configured number of intra-NG-RAN node inter-cell handovers for triggering the Measured UE Trajectory Reporting.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Reporting Time Duration | O |  | ENUMERATED (1s, 2s, 5s, 10s, 20s, 60s, ...) FFS on value range | It indicates the time point in seconds, end of which Measured UE Trajectory should be reported. FFS on semantics text |
| Number of intra-NG-RAN node inter-cell handovers | O |  | INTEGER (1..FFS) | It indicates the Maximum Number of UE hand-overs within the same target node, which is reached Measured UE Trajectory should be reported. FFS on semantics text |

    - 9.2.3.N1	Measured UE Trajectory

The Measured UE Trajectory IE contains the list of cells within the target NG-RAN node that the UE has visited after being handed over and the reporting cause value.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Trajectory Cell |  | 1..<maxnoofCellsTrajectory> |  | List of cells where the UE visited in RRC connected state, in chronological order. |
| >Measured UE Trajectory Cell Information | M |  | 9.2.3.N2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsTrajectory | Maximum number of cells that can be reported for UE trajectory. Value is FFS. |

9.2.3.N2	Measured UE Trajectory Cell Information

The Measured UE Trajectory Information contains the cell ID of the NG-RAN cell for measured UE trajectory collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Measured UE Trajectory Cell Information |  |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 |  |
| >>Time UE Stays in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. |

***********************************   End of the Change**********************************