3GPP TSG-RAN WG3 Meeting #121-bis	R3-235362

Xiamen, CN, 09–13 Oct, 2023

Agenda Item:	16.3

Source:	CMCC

Title:	Remaining issues on service continuity

Document for:	Discussion

Introduction

Service continuity had been discussed in last RAN3 meeting. Solution D5 has been concluded as the solution for downlink lossless Handover for SL relay. As captured in current BLCR, one note was added into the service continuity procedure. For the target gNB, it is still FFS. In this contribution, we discuss the behavior of target gNB.

NOTE: In order to support the DL lossless handover for the L2 U2N Remote UE, the source gNB may not discard the DL data even though the delivery of the data have been acknowledged by the L2 U2N Relay UE based on the gNB implementation. Then, the source gNB forwards the buffered DL data to the target gNB during the data forwarding procedure.

Discussion

In legacy gNB behavior, the source gNB may inform the target eNB about the next DL PDCP SN to allocate to a packet which does not have a PDCP sequence number yet. After the data forwarding, the target gNB re-transmits and prioritizes all downlink PDCP SDUs forwarded by the source gNB, with the exception of PDCP SDUs of which the reception was acknowledged through PDCP SN based reporting by the UE.

For solution-D5, source gNB may forwarding all the buffered data at PDCP transmitting entity to target gNB during the data forwarding procedure. From the target gNB’s perspective, it can skip the acknowledged PDCP SDUs to avoid the redundant packets; for the lost data indicated in the PDCP status report, target gNB can do nothing. So, no specific impact is introduced for target gNB to support lossless delivery.

Proposal 1: No specific impact is introduced for target gNB to support lossless delivery.

Conclusions

According the above discussion we have following proposals:

Proposal 1: No specific impact is introduced for target gNB to support lossless delivery.

References

[1] R2-2304305, Summary of candidate solutions for lossless delivery, NEC