3GPP TSG-RAN WG3 Meeting #121-bis	R3-235375

Xiamen, China, 9 – 13 October 2023

Agenda Item:	14.2

Source:	ZTE

Title:	(TP for LTM BL CR to TS 38.401) Discussion on L1L2 triggered mobility

Document for:	Discussions & Approval

- Introduction

We made the following progress at the RAN3#121 meeting.

Turn the WA to agreement that a new Class-2 procedure is introduced to notify gNB-CU about the LTM cell switch. The gNB-CU uses the UE Context Setup procedure to collect the TCI state configurations of candidate cells in LTM configuration phase for inter-DU LTM. After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE. The LTM Cell Change Notification procedure is reused to transfer the selected beam from the source gNB-DU to the target gNB-DU via gNB-CU. RAN3 prioritizes specifying support for early TA acquisition “without RAR” and mark the method “with RAR” as open issue. CU shall send the TA value to the source DU. For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU. Agree to add the RS configuration in UE Context Setup/Modification in stage2 and stage3. The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations. In inter-DU LTM, the gNB-CU provides the CFRA resource received from the candidate gNB-DU to the source gNB-DU via UE Context Modification procedure. Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2. WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU. WA: to introduce (2)new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU. How the source DU detect failure of TA info acquisition is up to implementation (eg., based on timer in source DU). Source DU is in charge of checking the TA validity. In case that L3 handover is triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority. In case that LTM is triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority, WA: In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority.

This paper further discusses how to support the LTM feature.

# 2	Discussion

2.1	RS configuration

At the last meeting, it is FFS whether to use UE associated signalling (UE Context Setup/Modification) or non-UE associated signalling (F1 SETUP) to collect RS configuration.

In R18, only SSB based L1 measurement is used which is cell specific, therefore non-UE associated signalling can be used to collect RS configuration. However, in future releases, CSI-RS based L1 measurement may be introduced which is UE specific. In such case, non-UE associated signalling can not be used to collect CSI-RS configuration. Therefore, using UE associated signalling to collect RS configuration is more future proof.

Observation 1: In R18, only SSB based L1 measurement is used which is cell specific, therefore non-UE associated signalling can be used to collect RS configuration.

Observation 2: If CSI-RS based L1 measurement is introduced in the future, non-UE associated signalling can not be used to collect CSI-RS configuration.

Proposal 1: The CU uses UE associated signalling to collect RS configuration from the candidate DU.

It is also FFS whether the CU needs to request the DU to provide the RS configuration. Considering that the RS configuration is a mandatory configuration to generate the corresponding L1 measurement reporting configuration for LTM, the DU should transfer the RS configuration proactively.

Observation 3: The RS configuration is a mandatory configuration to generate the corresponding L1 measurement reporting configuration for LTM.

Proposal 2:The DU should transfer the RS configuration to the CU proactively.

2.2	TCI state configuration

According to the following RAN1 agreement, the TCI state index is always provided in the cell switch command. Thus, the CU does not need to request the DU to provide the TCI state configuration, which indicates that the DU should transfer the TCI state configuration proactively.

Agreement Confirm the following working assumption achieved in RAN-112bis-e. Agreement On top the confirmed working assumption, on the presence of beam indication within cell switch command, at least for scenario 2 following is supported: - A field to indicate 1 joint or 1 pair of UL and DL unified TCI State index for the target cell field is always present in the cell switch command. - FFS UE behaviour for the beam indication field for the RACH-based handover scenario after cell switch command

Observation 4: The TCI state index is always provided in the cell switch command.

Proposal 3: The DU should transfer the TCI state configuration to the CU proactively.

2.3	Selected beam transfer

It is FFS on the procedure from CU to target DU to inform the selected beam information and target cell ID.

Option 1: new class 2 procedure with different name.

Option 2: same class 2 procedure

Option 3: class 1.

From our point of view, we prefer to use option 2 since we have already introduced the LTM Cell Change Notification procedure and it seems not necessary to introduce a new class 2 message.

Proposal 4: The CU uses the LTM Cell Change Notification procedure to inform the target DU of the selected beam information and target cell ID.

2.4	TA acquisition

At the last meeting, the following WA is achieved.

WA: to introduce (2) new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

Since the CFRA resources are shared among UEs in a candidate DU, thus the candidate DU can not differentiate which UE transmits the preamble. Thus, the candidate DU also needs to provide UE identification information to help the source DU identify the corresponding UE. The following UE identification information is listed for FFS.

- TA timer associated with the TA value,

- Detected RAP ID,

- Serving gNB-DU ID,

- Candidate cell ID.

- RO information

- SSB information

- Carrier information

Therefore, we think non-UE associated F1 signaling is more suitable to deliver the TA information and UE identification information from the candidate DU to the source DU via the CU.

Proposal 5: Introduce new class 2 non-UE associated F1 signaling to deliver the TA information and UE identification information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

As for the UE identification, we believe that RAN3 can only decide whether the serving gNB-DU ID is needed. The discussion of the rest UE identification information needs to wait for RAN2 progress.

Proposal 6: The serving gNB-DU ID needs to be transferred together with the TA information.

Proposal 7: RAN3 waits for RAN2 progress before discussing whether other UE identification information is needed to be transferred together with the TA information.

It is FFS whether to study the fallback case. From our point of view, this is not needed since if the TA info acquisition procedure is failed, the source DU implementation can decide whether to trigger early TA acquisition procedure again.

Proposal 8: If the TA info acquisition procedure is failed, the source DU implementation can decide whether to trigger early TA acquisition procedure again.

Another left issue is whether the source DU indicates the valid TA values to target DU after LTM for subsequent LTM. If the source DU sends the valid TA values to the target DU, the target DU may need to check TA validity due to long time duration. If the target DU wants to do RACHless LTM, it can trigger the early TA acquisition procedure to get the TA values.

Observation 5: If the source DU sends the valid TA values to the target DU, the target DU may need to check TA validity due to long time duration.

Observation 6: If the target DU wants to do RACHless LTM, it can trigger the early TA acquisition procedure to get the TA values.

Proposal 9: The source DU does not indicate the valid TA values to target DU after LTM for subsequent LTM.

# 3	Conclusion

Based on above analysis, we provide the following observations and proposals.

Observation 1: In R18, only SSB based L1 measurement is used which is cell specific, therefore non-UE associated signalling can be used to collect RS configuration.

Observation 2: If CSI-RS based L1 measurement is introduced in the future, non-UE associated signalling can not be used to collect CSI-RS configuration.

- Proposal 1: The CU uses UE associated signalling to collect RS configuration from the candidate DU.

Observation 3: The RS configuration is a mandatory configuration to generate the corresponding L1 measurement reporting configuration for LTM.

- Proposal 2:The DU should transfer the RS configuration to the CU proactively.

Observation 4: The TCI state index is always provided in the cell switch command.

- Proposal 3: The DU should transfer the TCI state configuration to the CU proactively.

- Proposal 4: The CU uses the LTM Cell Change Notification procedure to inform the target DU of the selected beam information and target cell ID.

- Proposal 5: Introduce new class 2 non-UE associated F1 signaling to deliver the TA information and UE identification information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

Proposal 6: The serving gNB-DU ID needs to be transferred together with the TA information.

- Proposal 7: RAN3 waits for RAN2 progress before discussing whether other UE identification information is needed to be transferred together with the TA information.

- Proposal 8: If the TA info acquisition procedure is failed, the source DU implementation can decide whether to trigger early TA acquisition procedure again.

Observation 5: If the source DU sends the valid TA values to the target DU, the target DU may need to check TA validity due to long time duration.

Observation 6: If the target DU wants to do RACHless LTM, it can trigger the early TA acquisition procedure to get the TA values.

- Proposal 9: The source DU does not indicate the valid TA values to target DU after LTM for subsequent LTM.

4	Text Proposal for BL CR TS 38.401

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 8.2.1.X	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU LTM

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate LTM configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing one target candidate cell ID. The gNB-CU requests PRACH resources from the gNB-DU.

- If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations  (e.g., TCI state configuration and RACH configuration) and the RS configuration for the accepted target candidate cell.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

FFS: whether it is UL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION RESPONSE message.

- Early synchronization is performed as specified in TS 38.300 [x].

- The UE sends the lower layer measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

- The gNB-DU sends the LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the selected beam (FFS).

- FFS: How the target gNB-DU detects the UE access.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

- The gNB-CU may send the UE CONTEXT MODIFICATION message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

---------------------------------------------------------Next change -------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.Y-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU LTM

1. The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2. The gNB-CU determines to initiate LTM configuration.

3. The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing one target candidate cell ID. The gNB-CU indicates the source gNB-DU ID, and requests PRACH resources from the Candidate gNB-DU.

4. If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration (e.g., TCI state configuration and RACH configuration) and the RS configuration for the accepted target candidate cell.

5. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the collected RS configuration, TCI state configuration and RACH configuration for the accepted target candidate cell(s) in other gNB-DUs.

6. The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which may include the RS configuration of the source cell, prepared candidate cells and the generated CSI resource configuration (FFS).

7. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing  the cell ID(s) of the prepared candidate cell(s) and associated RS configuration for each candidate cell in other candidate gNB-DU(s).

Note: The candidate cell may be the same cell as source cell.

8. The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated CSI resource configuration (FFS).

9. The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10. The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11. The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12. The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13. Early synchronization is performed as specified in TS 38.300 [x].

14. The candidate gNB-DU sends the TA and the associated CFRA resource information (FFS) to the source gNB-DU via the gNB-CU.

FFS on the message name.

15. The UE sends the lower layer measurement result to the source gNB-DU.

16. The source gNB-DU decides to execute LTM to a candidate target cell.

17. The source gNB-DU sends LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

18. The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the selected beam information.

19. The gNB-CU sends the LTM CELL CHANGE NOTIFICATION message to the target gNB-DU including the target cell ID and the selected beam information.

FFS on details of the selected beam information.

20. FFS:  how the target gNB-DU detects the UE access.

21. The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

22. The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

23. The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>