3GPP TSG-RAN WG3 Meeting #121bis	R3-235382

# Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | Draft | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of PDU session split |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, CMCC, China Telecom, China Unicom, Nokia, Nokia Shanghai Bell, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-09-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During PDU session split, if the MN decides to setup a new tunnel at the SN side upon receiving a request for PDU session split from 5GC, this may take a long time since Uu measurement triggering and reporting, SN addition and RRC reconfiguration may need to be performed. After the AMF sends out the PDU session Resource Setup Request or PDU session Resource Modify Request, it expects to receive the corresponding response in a given time, otherwise it may consider the procedure is not performed successfully. The waiting time for the AMF to receive a response message for establishing a split PDU session requested by SMF is the same as the waiting time without a request from SMF to establish split PDU session because the AMF is agnostic to the request from SMF. The waiting time in the AMF may then be too short when MN needs to trigger Uu measurement triggering and reporting. The behaviour of MN is undetermined in this scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add notes to indicate that if the MN determines that it cannot take a decision to split the PDU session during the PDU session resource setup/modify procedure time (e.g. due to need of measurement triggering and reporting) it shall execute the PDU session resource setup/modify procedure without considering the split PDU session request from SMF and may trigger the RAN initiated QoS flow offloading procedure after the completion of the PDU session resource setup/modify procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GC may not receive the response message form the MN in time and may consider the procedure is not successfully performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.14.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 10.14.3	PDU Session Split at UPF (RAN initiated QoS flows offloading from MN to SN)

When some QoS flows are offloaded from the MN to the SN, the MN may decide to split the PDU session served by the MN into more than one NG-U tunnels. The MN sends the SN Addition/Modification Request message including UPF UL TEID address used at the MN. Later on, if the MN receives a new UL TEID in the PDU Session Resource Modify Confirm message, the MN may provide the new UL TEID to the SN.

NOTE:	If the MN determines that it cannot take a decision to split the PDU session during the PDU session resource setup or modify procedure time (e.g. due to need of measurement triggering and reporting), it executes the PDU session resource setup or modify procedure without considering the split PDU session request from SMF and may trigger the RAN initiated QoS flow offloading procedure after the completion of the PDU session resource setup or modify procedure.

Figure 10.14.3-1: PDU Session Split at UPF during RAN initiated PDU session resource modify (QoS flows offloading from MN to SN)

1-2.	If the MN decides to split a PDU session, it uses the SN Addition procedure or the MN-initiated SN Modification procedure, including current UPF UL NG-U tunnel used at the MN. If in-order delivery is required for some QoS flows, an UL forwarding tunnel may be setup for the PDU session at this stage.

NOTE 1:	In case the MN offloads some QoS flows to the SN within a PDU session already split between the MN and the SN, the MN initiated SN Modification procedure is used.

3-6d.	If in-order delivery is required, the SN buffers the first packets received from the UE for a certain QoS flow until it receives an GTP-U end marker packet over the UL forwarding tunnel indicating that the MN has delivered all UL packets from the source side to UPF for that QoS flow. Then the SN starts delivering UL packets to UPF for that QoS flow using the UPF UL TEID address used at the MN received at step 1.

7-8.	The MN uses the PDU Session Resource Modify Indication message to inform 5GC that the PDU session is split into two tunnels and indicate which QoS flows are associated with which DL tunnel. The 5GC triggers the sending of DL End Marker packets without QFI tag at step 7a and confirms with the PDU Session Resource Modify Confirm message and allocates corresponding uplink tunnels.

After receiving the End Marker packet(s) from UPF at step 7a, the MN determines that the End Marker packets only work on the offloaded QoS flows, and may stop delivering and discard DL packets of the offloaded QoS flows, and the MN shall continue transmiting DL packets for the not offloaded QoS flows, if any.

7a./7b.	After receiving the DL end marker from 5GC at step 7a, the MN may generate at step 7b DL End Marker packets without QFI tag towards the SN.

9-10.	If the MN receives a new UL TEID in the PDU Session Resource Modify Confirm message for itself, the MN will use it to deliver UL packets to UPF. If the MN receives a new UL TEID for the SN, then the MN-initiated SN Modification procedure (i.e., step 9 and step 10) is used to provide the new UL TEID to the SN and then the SN switches to use the new UL TEID to deliver UL packets.