3GPP TSG-RAN WG3 Meeting #121-bis            				    R3-235393

Xiamen, China, Oct 9th – 13th, 2023

Agenda Item:		16.3

Source:			China Telecom

Title:		Discussion on service continuity leftover issues

Document for:	 	Discussion, Decision

- Introduction

Last RAN3 meeting companies discussed the signaling details of indirect path switching procedures. The relevant agreements are listed below [1].

RAN3 agrees on proactive Data forwarding from source gNB to target gNB.

FFS on whether to capture the behavior of the target gNB for Solution-D5 into the inter-gNB i2x path switch procedures.

FFS on E1 impact to support D5.

In this contribution, we will further discuss left open issues.

- Discussion

Last meeting RAN3 had discussed the solutions for downlink lossless data delivery for inter-gNB path switch cases and adopted D5. this solution allow the source gNB to forward all the buffered DL data to the target gNB without receiving the request from target gNB. One of the remaining open issues is how the source gNB ensure that buffered data is not removed when the data forwarding starts. Contribution [2] points out that according to the current specification DDDS cannot reflect the transmission status of DL data on the PC5 link. As the gNB-CU-UP does not know when the i2x path switch procedure is initiated, it has to always buffer a large amount of unnecessary DL data, even though the successful delivery is indicated by DDDS.

Observation 1: To support lossless data transmission during remote UE path switching, the CU-UP should not remove the buffered DL data even though the successful delivery is indicated by DDDS, which may result in it having to always buffer a large amount of unnecessary DL data.

The following mechanism can be used to make CU-UP knows when the i2x path switching is initiated:

1.	UE associated signaling, i.e. CU-CP triggers CU-UP to buffer DL data via explicit signaling.

2.	Based on gNB implementation.

For the first method, the information related to i2x path switching with the remote UE is signaled beforehand from the CP to the UP, for example via the Bearer Context Modification procedure (step 7 in following Figure). Then, the UP will not discard the buffered DL data even though the successful delivery is indicated by DDDS. This will have an impact on E1AP.

Figure 8.9.4-1: Inter-gNB handover involving gNB-CU-UP change (TS38.401)

For the second method, we can maintain a consistent configuration across CP and Ups by implementation, avoiding the need for any signaling to indicate additional buffering of DL data over E1. Then, the source UP forwards the buffered DL data to the target UP during the data forwarding procedure.

In order to support the DL lossless handover for the L2 Remote UE, we propose to further discuss the above aspects. Due to the additional specification impact of signaling based solutions, it is preferred to adopt the gNB based implementation method.

Proposal 1: During i2x path switching, the CU-UP should not discard the buffered DL data, it can be achieved based on CU-CP and CU-UP implementation.

- Conclusions

This contribution briefly analysed the support DL lossless handover of L2 Remote UE. Based on the above discussion, we propose the following:

Observation 1: To support lossless data transmission during remote UE path switching, the CU-UP should not remove the buffered DL data even though the successful delivery is indicated by DDDS, which may result in it having to always buffer a large amount of unnecessary DL data.

Proposal 1: During i2x path switching, the CU-UP should not discard the buffered DL data, it can be achieved based on CU-CP and CU-UP implementation.

- References

[1]	RAN3#121 Meeting report.

[2]	R3-234026, Another issue on DL lossless delivery, Samsung.