3GPP TSG-RAN WG3 Meeting #121bis	R3-235418

Xiamen, China, 09 – 13 October, 2023	revision of R3-234143

Agenda Item:	13.5

Source:	Huawei, Ericsson, Xiaomi

Title:	(TP for NR_mobile_IAB BL CR for TS 38.455): Positioning enhancement on uplink E-CID measurement for mobile TRP

Document for:	Discussion

1	Introduction

In last RAN3 #120 meeting, two BL CRs (R3-233518 for TS38.455, R3-233519 for TS38.473) are endorsed to reflect the RAN3 progress on how to align the UE positioning solution in case the mobile TRP is involved. The TRP Information Exchange procedure, and the Measurement procedure are enhanced to allow the mobile TRP location information to be included.

In this paper, we discuss the further enhancement needed for the positioning based on SA2’s solution.

2	Discussion

2.1 Enhancement for E-CID measurement

RAN3 discussed the change to the NRPPa and F1AP procedure, according to the TS23.273, “If Network Assisted procedure is used, as in 6.11.2, the NG-RAN may provide the MBSR updated location and velocity information and the time obtained them to the LMF as part of the NRPPa procedure for the UE.” Considering the network assisted procedure can be the Measurement procedure, RAN3 enhanced the measurement results to include the mobile TRP location information. Besides the Measurement procedure, the uplink E-CID measurement also can be used as the “network assisted procedure” for UE positioning. The below Uplink E-CID Positioning Procedure is copied from the section 8.3.4 of TS 38.305.

Figure 8.3.4.3.1-1: LMF-initiated Position Measurement Procedure

So, for the NRPPa procedure, we see it is also necessary to include the mobile TRP location information in the E-CID Measurement Result IE, if the uplink E-CID positioning procedure is adopted for the UE positioning.

Since TRP is out of E-CID function, we propose to include a Mobile Access Point Location Information IE refering to the moving gNB node location that is associated to the  mobile-IAB-MT (UE) location.

Proposal 1:	Include the Mobile Access Point Location Information IE in the E-CID Measurement Result IE.

Accordingly, the TP for TS38.455 is provided in the appendix, and the TP for TS 38.473 is provided in R3-235419.

3	Conclusion

In this paper, we analyse the enhancement for the E-CID measurement procedure, and propose:

Proposal 1:	Include the Mobile Access Point Location Information IE in the E-CID Measurement Result IE.

4	Reference

- R3-233518, Support for mobile TRP Location Information

- R3-233519, Support of mobile TRP location information.

Appendix: TP for BL CR of TS38.455

-------------------------------------------Start of changes-------------------------------------------

### 8.2.3	E-CID Measurement Report

#### 8.2.3.1	General

The purpose of E-CID Measurement Report procedure is for the NG-RAN node to provide the E-CID measurements for the UE to the LMF.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: E-CID Measurement Report procedure, successful operation

The NG-RAN node initiates the procedure by sending an E-CID MEASUREMENT REPORT message. The E-CID MEASUREMENT REPORT message contains the E-CID measurement results according to the measurement configuration in the respective E-CID MEASUREMENT INITIATION REQUEST message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT REPORT message when measurement results other than the "Cell-ID" have been requested.

If available, the NG-RAN node shall include the NG-RAN Access Point Position IE or the Geographical Coordinates IE which is the configured estimated serving antenna position in the E-CID Measurement Result IE within the E-CID MEASUREMENT REPORT message. Upon reception of this NG-RAN Access Point Position IE, the LMF may use the value as the geographical position of the NG-RAN access point.

If available, the NG-RAN node shall include the Cell Portion ID IE in the E-CID MEASUREMENT REPORT message. Upon reception of the Cell Portion ID IE, the LMF may use the value as the cell portion for the measurement.

If available, the NG-RAN node shall include the Mobile Access Point Location Information IE in the E-CID MEASUREMENT REPORT message. Upon reception of the Mobile Access Point Location Information IE, the LMF may use the value as the location information of the mobile NG-RAN access point that is associated to the mobile TRP.

-------------------------------------------Next change-------------------------------------------

9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | NG-RAN CGI 9.2.6 | NG-RAN Cell Identifier of the serving cell | - |  |
| Serving Cell TAC | M |  | TAC 9.2.11 | Tracking Area Code of the serving cell | - |  |
| NG-RAN Access Point Position | O |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell. If the Geographical Coordinates IE is used, the NG-RAN Access Point Position IE shall be ignored. | - |  |
| Measured Results |  | 0..1 |  | Measurement results of the serving RAT. | - |  |
| >CHOICE Measured Results Value |  | 1 ..<maxnoMeas> |  |  | - |  |
| >>Value Angle of Arrival EUTRA | M |  | INTEGER (0..719) | According to mapping in TS 36.133 [9] | - |  |
| >>Value Timing Advance Type 1 EUTRA | M |  | INTEGER (0..7690) | As defined in TS 36.214 [17] | - |  |
| >>Value Timing Advance Type 2 EUTRA | M |  | INTEGER (0..7690) | As defined in TS 36.214 [17] | - |  |
| >>Result RSRP EUTRA |  | 1 |  |  | - |  |
| >>>Result RSRP EUTRA Item |  | 1 .. <maxCellReport> |  |  | - |  |
| >>>>PCI EUTRA | M |  | INTEGER (0..503) | Physical Cell Identifier of the reported E-UTRA cell | - |  |
| >>>>EARFCN | M |  | INTEGER (0.. 262143, …) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] | - |  |
| >>>>CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell | - |  |
| >>>>Value RSRP EUTRA | M |  | INTEGER (0..97, …) |  | - |  |
| >>Result RSRQ EUTRA |  | 1 |  |  | - |  |
| >>>Result RSRQ EUTRA Item |  | 1 . <maxCellReport> |  |  | - |  |
| >>>>PCI EUTRA | M |  | INTEGER (0..503) | Physical Cell Identifier of the reported E-UTRA cell | - |  |
| >>>>EARFCN | M |  | INTEGER (0..262143, …) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] | - |  |
| >>>>CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell | - |  |
| >>>>Value RSRQ EUTRA | M |  | INTEGER (0..34, …) |  | - |  |
| >>Result SS-RSRP |  | 1 |  |  | YES | ignore |
| >>>Result SS-RSRP Item |  | 1 .. <maxCellReportNR> |  |  | - |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>>Value SS-RSRP Cell | O |  | INTEGER (0..127) | SS-RSRP measurement aggregated at cell level | - |  |
| >>>>SS-RSRP per SSB Resource |  | 0..1 |  |  | - |  |
| >>>>>SS-RSRP per SSB Resource Item |  | 1 .. <maxIndexesReport> |  |  | - |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>>Value SS-RSRP | M |  | INTEGER (0..127) | SS-RSRP measurement per SSB resource | - |  |
| >>Result SS-RSRQ |  | 1 |  |  | YES | ignore |
| >>>ResultSS-RSRQ-Item |  | 1 .. <maxCellReportNR> |  |  | - |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>>Value SS-RSRQ Cell | O |  | INTEGER (0..127) | SS-RSRQ measurement aggregated at cell level | - |  |
| >>>>SS-RSRQ per SSB Resource |  | 0..1 |  |  | - |  |
| >>>>>SS-RSRQ PerSSB Resource Item |  | 1 .. <maxIndexesReport> |  |  | - |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>>Value SS-RSRQ | M |  | INTEGER (0..127) | SS-RSRQ measurement per SSB resource | - |  |
| >>Result CSI-RSRP |  | 1 |  |  | YES | ignore |
| >>>Result CSI-RSRP Item |  | 1 .. <maxCellReportNR> |  |  | - |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>>Value CSI-RSRP Cell | O |  | INTEGER (0..127) | CSI-RSRP measurement aggregated at cell level | - |  |
| >>>>CSI-RSRP per CSI-RS Resource |  | 0..1 |  |  | - |  |
| >>>>>CSI-RSRP per CSI-RS Resource Item |  | 1.. <maxIndexesReport> |  |  | - |  |
| >>>>>>CSI-RS Index | M |  | INTEGER (0..95) |  | - |  |
| >>>>>>Value CSI-RSRP | M |  | INTEGER (0..127) | CSI-RSRP measurement per CSI-RS resource | - |  |
| >>Result CSI-RSRQ |  | 1 |  |  | YES | ignore |
| >>>Result CSI-RSRQ Item |  | 1 .. <maxCellReportNR> |  |  | - |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>>Value CSI-RSRQ Cell | O |  | INTEGER (0..127) | CSI-RSRQ measurement aggregated at cell level | - |  |
| >>>>CSI-RSRQ per CSI-RS Resource |  | 0..1 |  |  | - |  |
| >>>>>CSI-RSRQ per CSI-RS Resource Item |  | 1 .. <maxIndexesReport> |  |  | - |  |
| >>>>>>CSI-RS Index | M |  | INTEGER (0..95) |  | - |  |
| >>>>>>Value CSI-RSRQ | M |  | INTEGER (0..127) | CSI-RSRQ measurement per CSI-RS resource | - |  |
| >>Angle of Arrival NR | M |  | UL Angle of Arrival 9.2.38 |  | YES | ignore |
| >>Value Timing Advance NR | M |  | INTEGER (0.. 7690) | As defined in TS 38.215 [19] | YES | ignore |
| Geographical Coordinates | O |  | 9.2.46 |  | YES | ignore |
| Mobile Access Point Location Information | O |  | Mobile TRP Location Information 9.2.X | The location information of the mobile access point of the cell that is associated to the mobile TRP. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |
| maxCellReportNR | Maximum no. of NR cells that can be reported with one message. Value is 9. |
| maxIndexesReport | Maximum no. of beam level measurement results that can be reported with one message. Value is 64. |

-------------------------------------------Next change-------------------------------------------

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-CGI-NR,

id-SFNInitialisationTime-NR,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-ResultNR,

id-ResultEUTRA,

maxCellinRANnode,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-OtherRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxWLANchannels,

maxnoFreqHoppingBandsMinusOne,

id-TDD-Config-EUTRA-Item,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs,

maxnoPosMeas,

maxnoTRPs,

maxnoTRPInfoTypes,

maxNoOfMeasTRPs,

maxNoPath,

maxnoofAngleInfo,

maxnolcs-gcs-translation,

maxnoBcastCell,

maxnoSRSTriggerStates,

maxnoSpatialRelations,

maxNRMeas,

maxEUTRAMeas,

maxIndexesReport,

maxCellReportNR,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxPRS-ResourceSets,

maxPRS-ResourcesPerSet,

maxNoSSBs,

maxnoofPRSresourceSet,

maxnoofPRSresource,

maxnoofULAoAs,

maxNoPathExtended,

maxnoARPs,

maxnoTRPTEGs,

maxnoUETEGs,

maxFreqLayers,

maxnoPRSTRPs,

maxNumResourcesPerAngle,

maxnoAzimuthAngles,

maxnoElevationAngles,

id-Cell-ID,

id-TRPInformationTypeItem,

id-SrsFrequency,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-PRS-Resource-ID,

id-OnDemandPRS,

id-AoA-SearchWindow,

id-ZoA,

id-MultipleULAoA,

id-UL-SRS-RSRPP,

id-SRSResourcetype,

id-ExtendedAdditionalPathList,

id-ARPLocationInfo,

id-ARP-ID,

id-LoS-NLoSInformation,

id-NumberOfTRPRxTEG,

id-NumberOfTRPRxTxTEG,

id-TRPTxTEGAssociation,

id-TRPTEGInformation,

id-TRP-Rx-TEGInformation,

id-TRPBeamAntennaInformation,

id-NR-TADV,

id-pathPower,

id-SRSPortIndex,

id-UETxTimingErrorMargin,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-StartRBHopping,

id-StartRBIndex,

id-transmissionCombn8

id-Mobile-TRP-LocationInformation,

id-Mobile-IAB-MT-UE-ID,

id-MobileAccessPointLocation

-------------------------------------------Next change-------------------------------------------

>>>>>>>>>>>>unchanged parts are skipped<<<<<<<<<<<<

#### -- E

E-CID-MeasurementResult ::= SEQUENCE {

servingCell-ID					NG-RAN-CGI,

servingCellTAC					TAC,

nG-RANAccessPointPosition		NG-RANAccessPointPosition	OPTIONAL,

measuredResults					MeasuredResults				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL,

...

}

E-CID-MeasurementResult-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-GeographicalCoordinates	CRITICALITY ignore	EXTENSION GeographicalCoordinates PRESENCE optional}|

{ ID id-MobileAccessPointLocation	CRITICALITY ignore EXTENSION Mobile-TRP-LocationInformation	PRESENCE optional },

...

}

EUTRACellIdentifier ::= BIT STRING (SIZE (28))

-------------------------------------------Next change-------------------------------------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

>>>>>>>>>>>>unchanged parts are skipped<<<<<<<<<<<<

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-LMF-UE-Measurement-ID										ProtocolIE-ID ::= 2

id-ReportCharacteristics										ProtocolIE-ID ::= 3

id-MeasurementPeriodicity										ProtocolIE-ID ::= 4

id-MeasurementQuantities										ProtocolIE-ID ::= 5

id-RAN-UE-Measurement-ID										ProtocolIE-ID ::= 6

id-E-CID-MeasurementResult										ProtocolIE-ID ::= 7

id-OTDOACells													ProtocolIE-ID ::= 8

id-OTDOA-Information-Type-Group									ProtocolIE-ID ::= 9

id-OTDOA-Information-Type-Item									ProtocolIE-ID ::= 10

id-MeasurementQuantities-Item									ProtocolIE-ID ::= 11

id-RequestedSRSTransmissionCharacteristics						ProtocolIE-ID ::= 12

id-Cell-Portion-ID												ProtocolIE-ID ::= 14

id-OtherRATMeasurementQuantities								ProtocolIE-ID ::= 15

id-OtherRATMeasurementQuantities-Item							ProtocolIE-ID ::= 16

id-OtherRATMeasurementResult									ProtocolIE-ID ::= 17

id-WLANMeasurementQuantities									ProtocolIE-ID ::= 19

id-WLANMeasurementQuantities-Item								ProtocolIE-ID ::= 20

id-WLANMeasurementResult										ProtocolIE-ID ::= 21

id-TDD-Config-EUTRA-Item										ProtocolIE-ID ::= 22

id-Assistance-Information										ProtocolIE-ID ::= 23

id-Broadcast													ProtocolIE-ID ::= 24

id-AssistanceInformationFailureList								ProtocolIE-ID ::= 25

id-SRSConfiguration												ProtocolIE-ID ::= 26

id-MeasurementResult											ProtocolIE-ID ::= 27

id-TRP-ID														ProtocolIE-ID ::= 28

id-TRPInformationTypeListTRPReq									ProtocolIE-ID ::= 29

id-TRPInformationListTRPResp									ProtocolIE-ID ::= 30

id-MeasurementBeamInfoRequest									ProtocolIE-ID ::= 31

id-ResultSS-RSRP												ProtocolIE-ID ::= 32

id-ResultSS-RSRQ												ProtocolIE-ID ::= 33

id-ResultCSI-RSRP												ProtocolIE-ID ::= 34

id-ResultCSI-RSRQ												ProtocolIE-ID ::= 35

id-AngleOfArrivalNR												ProtocolIE-ID ::= 36

id-GeographicalCoordinates										ProtocolIE-ID ::= 37

id-PositioningBroadcastCells									ProtocolIE-ID ::= 38

id-LMF-Measurement-ID											ProtocolIE-ID ::= 39

id-RAN-Measurement-ID											ProtocolIE-ID ::= 40

id-TRP-MeasurementRequestList									ProtocolIE-ID ::= 41

id-TRP-MeasurementResponseList									ProtocolIE-ID ::= 42

id-TRP-MeasurementReportList									ProtocolIE-ID ::= 43

id-SRSType														ProtocolIE-ID ::= 44

id-ActivationTime												ProtocolIE-ID ::= 45

id-SRSResourceSetID												ProtocolIE-ID ::= 46

id-TRPList														ProtocolIE-ID ::= 47

id-SRSSpatialRelation											ProtocolIE-ID ::= 48

id-SystemFrameNumber											ProtocolIE-ID ::= 49

id-SlotNumber													ProtocolIE-ID ::= 50

id-SRSResourceTrigger											ProtocolIE-ID ::= 51

id-TRPMeasurementQuantities										ProtocolIE-ID ::= 52

id-AbortTransmission											ProtocolIE-ID ::= 53

id-SFNInitialisationTime										ProtocolIE-ID ::= 54

id-ResultNR														ProtocolIE-ID ::= 55

id-ResultEUTRA													ProtocolIE-ID ::= 56

id-TRPInformationTypeItem										ProtocolIE-ID ::= 57

id-CGI-NR														ProtocolIE-ID ::= 58

id-SFNInitialisationTime-NR										ProtocolIE-ID ::= 59

id-Cell-ID														ProtocolIE-ID ::= 60

id-SrsFrequency													ProtocolIE-ID ::= 61

id-TRPType														ProtocolIE-ID ::= 62

id-SRSSpatialRelationPerSRSResource								ProtocolIE-ID ::= 63

id-MeasurementPeriodicityExtended								ProtocolIE-ID ::= 64

id-PRS-Resource-ID												ProtocolIE-ID ::= 65

id-PRSTRPList													ProtocolIE-ID ::= 66

id-PRSTransmissionTRPList										ProtocolIE-ID ::= 67

id-OnDemandPRS												ProtocolIE-ID ::= 68

id-AoA-SearchWindow												ProtocolIE-ID ::= 69

id-TRP-MeasurementUpdateList									ProtocolIE-ID ::= 70

id-ZoA															ProtocolIE-ID ::= 71

id-ResponseTime													ProtocolIE-ID ::= 72

id-UEReportingInformation										ProtocolIE-ID ::= 73

id-MultipleULAoA												ProtocolIE-ID ::= 74

id-UL-SRS-RSRPP													ProtocolIE-ID ::= 75

id-SRSResourcetype												ProtocolIE-ID ::= 76

id-ExtendedAdditionalPathList									ProtocolIE-ID ::= 77

id-ARPLocationInfo												ProtocolIE-ID ::= 78

id-ARP-ID														ProtocolIE-ID ::= 79

id-LoS-NLoSInformation											ProtocolIE-ID ::= 80

id-UETxTEGAssociationList										ProtocolIE-ID ::= 81

id-NumberOfTRPRxTEG												ProtocolIE-ID ::= 82

id-NumberOfTRPRxTxTEG											ProtocolIE-ID ::= 83

id-TRPTxTEGAssociation											ProtocolIE-ID ::= 84

id-TRPTEGInformation											ProtocolIE-ID ::= 85

id-TRP-Rx-TEGInformation													ProtocolIE-ID ::= 86

id-TRP-PRS-Information-List										ProtocolIE-ID ::= 87

id-PRS-Measurements-Info-List									ProtocolIE-ID ::= 88

id-PRSConfigRequestType											ProtocolIE-ID ::= 89

id-UE-TEG-Info-Request											ProtocolIE-ID ::= 90

id-MeasurementTimeOccasion										ProtocolIE-ID ::= 91

id-MeasurementCharacteristicsRequestIndicator					ProtocolIE-ID ::= 92

id-TRPBeamAntennaInformation									ProtocolIE-ID ::= 93

id-NR-TADV														ProtocolIE-ID ::= 94

id-MeasurementAmount											ProtocolIE-ID ::= 95

id-pathPower													ProtocolIE-ID ::= 96

id-PreconfigurationResult										ProtocolIE-ID ::= 97

id-RequestType													ProtocolIE-ID ::= 98

id-UE-TEG-ReportingPeriodicity									ProtocolIE-ID ::= 99

id-SRSPortIndex													ProtocolIE-ID ::= 100

id-procedure-code-101-not-to-be-used							ProtocolIE-ID ::= 101

id-procedure-code-102-not-to-be-used							ProtocolIE-ID ::= 102

id-procedure-code-103-not-to-be-used							ProtocolIE-ID ::= 103

id-UETxTimingErrorMargin										ProtocolIE-ID ::= 104

id-MeasurementPeriodicityNR-AoA									ProtocolIE-ID ::= 105

id-SRSTransmissionStatus										ProtocolIE-ID ::= 106

id-repetitionFactorExtended										ProtocolIE-ID ::= 108

id-StartRBHopping												ProtocolIE-ID ::= 109

id-StartRBIndex													ProtocolIE-ID ::= 110

id-transmissionCombn8											ProtocolIE-ID ::= 111

id-Mobile-TRP-LocationInformation								ProtocolIE-ID ::= xx1

id-Mobile-IAB-MT-UE-ID											ProtocolIE-ID ::= xx2

id-MobileAccessPointLocation								ProtocolIE-ID ::= xx3

END

-- ASN1STOP

-------------------------------------------End of changes-------------------------------------------