# 3GPP TSG-RAN WG3 Meeting #121bis	R3-235428

# Xiamen, China, Oct.9th - Oct.13th, 2023

# Agenda Item:	12.2.2.3

# Source:	ZTE

# Title:	(Draft LS out) Further discussion on AI/ML based Energy Saving

# Document for:	Discussion

1	Introduction

Regarding the AI/ML based Energy Saving use cases, after heated discussion, RAN3 agree that the definition and signaling over RAN interfaces of the Additional Load as well as Inferred EC are pursed in Rel-18.

Following are the agreements in the last meeting:

Define the Energy Cost IE as an INTEGER (0..10000,…), it can be revisited based on reply from SA5. The definition and signaling over RAN interfaces of the Additional Load as well as Inferred EC are not pursued in Rel-18.

Besides, RAN3 received the reply LS [1] from SA5 about NG-RAN Energy Saving Energy Cost index, and SA5 put forward four questions to RAN3 for clarification. In this contribution, we provide our views on the LS and provide the corresponding reply LS.

2	Discussion

Following is the content of the reply LS from SA5:

1. Overall Description: 3GPP SA5 thanks RAN3 for your LS on AI/ML for NG-RAN Energy Saving Energy Cost index. Here below, 3GPP SA5 provides its understanding of the problem to be solved, preliminary thoughts about potential solutions and questions for clarification. SA5 understanding of the problem is: - NG-RAN nodes (i.e. gNBs and ng-eNBs) should exchange information about their respective energy consumption. However, instead of exchanging their actual energy consumption (e.g. in Kw/h), RAN3 expects the actual energy consumption be converted into an ‘Energy Cost’, i.e. an index which reflects the energy consumption of the NG-RAN Nodes. - The rule to convert the actual energy consumption of the NG-RAN nodes into the Energy Cost is provisioned via OAM into the NG-RAN nodes and is unified across the Operator’s network. 2. Questions to RAN3 for clarification: - Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other? - Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)? - Should the Energy Cost be calculated in near-real time or non-real time (see above)? - What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

During the normative work for AI/ML based energy saving, RAN3 agreed to introduce a new metric named Energy Cost. The Energy Cost provides a representation of the energy consumption (Kw/H) at an NG-RAN node, in order to avoid transferring the actual energy consumption as a kind of sensitive information over RAN interface, and avoid security threat. The mapping rules shall be the same at least for all neighbouring NG-RAN nodes within the area where a request on EC reporting is triggered by a NG-RAN node.

Observation 1: From RAN3 perspective, the Energy Cost, encoded as an index, is introduced to avoid the exposure of sensitive information and security threat.

Question 1: Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other?

Considering the intention behind introducing energy cost metrics and the aim of AI/ML based energy-saving methods to optimize the energy consumption within specific areas, we consider that the mapping rules should be defined by the operator based on its own requirements, to ensure mapping rules used for the nodes within the area where the request on EC reporting is the same.

Proposal 1: Answer the Q1 that the mapping rule is defined by operator based on its own requirement, to ensure that the nodes within the area where triggers energy cost reporting leverage the same mapping rule.

Question 2: Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)?

From our perspective, it is hard to collect the instantaneous power consumption, so the calculation of the energy cost is based on energy consumption measurements (Kw/H) over a past period. Moreover, energy cost represent of a linear scale of energy consumption defined by SA5 below. The measurement mechanism should remain consistent.

6.7.3.4.2 gNB EC a) ECgNB b) This KPI describes the Energy Consumption (EC) of the gNB. It is obtained by summing up the Energy Consumption of all the Network Functions (NF) that constitute the gNB. For the Energy Consumption of Network Functions (ECNF), see clause 6.7.3.1. The unit of this KPI is J.

Proposal 2: Answer the Q2 that the calculation of the Energy Cost should be based on the energy consumption over a past period, and the measurement mechanism should remain consistent with the energy consumption.

Question 3: Should the Energy Cost be calculated in near-real time or non-real time (see above)?

Q3 has relationship with the Q2. In light of energy cost is calculated by energy consumption (KwH), instead of the instantaneous power consumption, it needs to be calculated over a period of time, which means they are non-real time measurements.

Proposal 3:Answer the Q3 that the Energy Cost should be calculated in non-real time.

Question 4: What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

This question is related to the energy cost in the case of split CU/DU architecture. In our view, since the NG-RAN nodes components can be geographically or distributed or virtualized (e.g., gNB-CU-CP, gNB-CU-UP), it is proposed to use the same Energy Cost concept for each logical entity to have a clean solution for transferring energy consumption in RAN side to avoid the misinterpretation of the value.

Because it was agreed that energy cost over Xn interface is a node-level metric, CU needs to collect the energy consumption of gNB-DU, gNB-CU-UP and gNB-CU-CP, and then calculate the total energy consumption of the NG-RAN node. Energy cost may also needs to be transferred in the case of split architecture.The discussion on how to apply it to split architectures is still on-going in RAN3.

Proposal 4:Answer the Q4 that the same Energy Cost could be also applied for split architecture.

3	Conclusion

[1] R3-235031, Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost Index, SA5

4	Reference

Observation 1: From RAN3 perspective, the Energy Cost, encoded as an index, is introduced to avoid the exposure of sensitive information and security threat.

Proposal 1: Answer the Q1 that the mapping rule is defined by Operator by network implementation, to ensure that the nodes within the area where triggers energy cost reporting leverage the same mapping rule.

Proposal 2: Answer the Q2 that the calculation of the Energy Cost should be based on the energy consumption over a past period, and the measurement mechanism should remain consistent with the energy consumption.

Proposal 3:Answer the Q3 that the Energy Cost should be calculated in non-real time.

Proposal 4:Answer the Q4 that the same Energy Cost should be also applied for split architecture.

5	[Draft] Reply LS to SA5

# 3GPP TSG-RAN WG3 Meeting #121bis	R3-23xxxx

# Xiamen, China, Oct.9th - Oct.13th, 2023

Title:	[DRAFT] Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index

Response to:	S5-235777

Release:	Rel-18

Work Item:	NR_AIML_NGRAN-Core

Source:	RAN3

To:	SA5

Cc:

Contact person:	Jiajun Chen

Chen.jiajun1(at)zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1. Overall Description:

3GPP RAN3 thanks SA5 for your Reply LS on AI/ML for NG-RAN Energy Saving Energy Cost index, which requests RAN3 for clarification.

Question 1: Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other?

RAN3’s answer: The mapping rule is defined by operator based on its own requirement, to ensure that the nodes within the area where triggers energy cost reporting leverage the same mapping rule.

Question 2: Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)?

RAN3’s answer: The calculation of the Energy Cost should be based on the energy consumption over a past period, and the measurement mechanism should remain consistent with the energy consumption.

Question 3: Should the Energy Cost be calculated in near-real time or non-real time (see above)?

RAN3’s answer: In light of the calculation of the Energy Cost is based on the energy consumption over a past period, the Energy Cost is calculated in non-real time.

Question 4: What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

RAN3’s answer: The same Energy Cost could be also applied for split architecture. The same Energy Cost concept for each logical entity to have a clean solution for transferring energy consumption in RAN side to avoid the misinterpretation of the value. The discussion on how to apply it to split architectures is still on-going in RAN3.

# 2. Actions:

To SA5

ACTION: 	RAN3 kindly ask SA5 to take the above into account and to provide feedback to RAN3.

# 3. Date of Next TSG-RAN3 Meetings:

TSG RAN3 Meeting #122		13 - 17 Nov 2023			Chicago

TBD