3GPP TSG-RAN WG3 Meeting #121-bis	R3-235432

Xiamen, China, Oct. 9th – Oct. 13th, 2023

Agenda Item:	13.2

Source: 	Canon Research Centre France

Title:	Discussion on IAB-node mobility

Document for:	Discussion

# Introduction

In this paper, we discuss various aspects related to the mobility of an IAB-node based on the following agreements and identified issues from the previous RAN3 meeting reports [1], [2], [3], [4], and [5]:

At RAN3#118 [1]: - The mIAB-MT’s source donor CU can send the info on the mIAB-MT’s target donor CU to the mIAB-DU’s donor CU after the completion of IAB-MT HO. - The mIAB-MT ID sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU is the XnAP UE ID. FFS which donor generates this ID. - The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the: • gNB ID of the target donor CU for the mIAB-MT HO. • ID(s) of the mIAB-MT. At RAN3#119 [2]: - In case the target logical mIAB-DU’s CU is different from the mIAB-MT’s CU, the target logical mIAB-DU’s CU needs to be informed about the mIAB-MT’s CU ID and the mIAB-MT ID so that it can initiate the Xn TMM procedures towards mIAB-MT’s CU. At RAN3#119bis [3]: - Turn into an agreement the WA stating that: The mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs. - Down select between the following two options for providing the gNB-ID of the mIAB-MT’s CU and the XnAP UE ID of the mIAB-MT at this CU to mIAB-DU’s target CU: - Option A: XnAP signalling from the mIAB-DU’s source CU. - Option B: F1AP signalling from the target logical mIAB-DU. For Option B, discuss whether and how the mIAB-DU can obtain the gNB-ID of the mIAB-MT’s CU and the XnAP UE ID of the mIAB-MT at this CU. At RAN3#120 [4]: - During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU. - It is FFS whether the XnAP ID can be transparently passed from the mIAB-MT’s CU to the mIAB-MT via RRC, then from the co-located mIAB-DU to mIAB-DU’s CU via F1AP. At RAN3#121 [5]: - For the case of DU migration, F1AP signalling from the target logical mIAB-DU is selected for providing the gNB-ID of the mIAB-MT’s CU to the target mIAB-DU’s CU. - Option B will be considered as baseline for transferring of remaining information i.e. the ID of the IAB MT. - In the next meeting, RAN3 to decide which ID assigned by the MT CU for the IAB MT should be used for the TMM procedure and how this ID can be mapped to a corresponding IAB node.

# Discussion

## Acceptance/Rejection of DU migration

When the DU of a mobile IAB-node is migrated from a source F1 donor-CU toward a target F1 donor-CU, the UEs served by the mobile IAB-node shall also be handed over toward this target F1 donor-CU. Upon reception of a handover request for a UE served by the mobile IAB-node, the target F1 donor-CU may reject the handover. If this happens, the cause of handover preparation failure should not be related to the radio network layer as the traffic associated to the UE is handled by the RRC terminating donor-CU serving the co-located mobile IAB-MT. However, the cause may be a control processing overload as a CU can handle a limited number of connected UEs. In case only a few donor-CUs support mobile IAB, those donor-CUs would handle many mobile IAB-nodes and thus many UEs.

A situation where most handovers of UEs served by a mobile IAB-node are rejected by the target F1 donor-CU should be avoided, as these UEs, and especially the onboard UEs, will have to be handed over to another cell and they will lose the benefit of the mobile cell. Thus, it may be useful to get the status at the target F1 donor-CU before initiating the first UE handover.

For this purpose, the F1 setup request in the scope of a DU migration may include an information related to the number of UEs served by the mobile IAB-node. This information would help the target donor-CU for the decision to accept or to reject the F1 setup request (due to control processing overload). In case of rejection, another target donor-CU may be selected, or a new attempt may be performed later.

Proposal 1: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU may include the number of connected UEs served by the mobile IAB-node.

Besides, the target F1 donor-CU may be allowed to partially accept the DU migration. For instance, the target F1 donor-CU may partially accept the DU migration in the F1 setup response by indicating the number of UEs it can accommodate.

Proposal 2: The target F1 donor-CU may accept the DU migration of a mobile IAB-node with some conditions, e.g. with a maximum number of UEs that can be served.

## Providing identification information to a F1 donor-CU

At DU migration of a mobile IAB-node

In the scope of DU migration of a mobile IAB-node, we have the following agreement from [3]:

Down select between the following two options for providing the gNB-ID of the mIAB-MT’s CU and the XnAP UE ID of the mIAB-MT at this CU to mIAB-DU’s target CU:

- Option A: XnAP signalling from the mIAB-DU’s source CU.

- Option B: F1AP signalling from the target logical mIAB-DU.

Also, from [5] we have the following agreements and note:

For the case of DU migration, F1AP signalling from the target logical mIAB-DU is selected for providing the gNB-ID of the mIAB-MT’s CU to the target mIAB-DU’s CU.

Option B will be considered as baseline for transferring of remaining information i.e. the ID of the IAB MT.

RAN3 to decide which ID assigned by the MT CU for the IAB MT should be used for the TMM procedure and how this ID can be mapped to a corresponding IAB node.

Actually, this ID assigned by the RRC terminating donor-CU (i.e. the mIAB-MT’s CU) may be a BAP address or a UE XnAP ID.

The BAP address selected by the RRC terminating donor-CU can be provided by the target logical mobile IAB-DU to the target F1 terminating donor-CU (i.e. the mIAB-DU’s CU) in the legacy Rel-16/17 F1 setup request message. In case only the BAP address is provided (and not the UE XnAP ID allocated at the RRC terminating donor-CU), the TMM request message shall be amended to allow the use of the BAP address IE instead of the non-F1 terminating IAB-donor UE XnAP ID IE. Moreover, the behavior of the RRC terminating donor-CU receiving this TMM request message shall be adapted accordingly. Indeed, according to the Release 17 specifications (TS 38.423), the presence of UE XnAP IDs allocated by the F1 terminating donor-CU and by the RRC terminating donor-CU is mandatory for the TMM procedure.

To avoid the modifications of the TMM procedure and of the IAB-donor-CUs’ behavior, the UE XnAP ID allocated at the RRC terminating donor-CU should be provided to the target F1 terminating donor-CU. For this purpose, there are two options:

- F1AP signalling from the target logical mobile IAB-DU, e.g. with the F1 setup request message or the gNB-DU Configuration Update message. This option involves that the UE XnAP ID is provided to the mobile IAB-node beforehand, e.g. through RRC by the RRC terminating donor-CU. With this option, it is not needed to provide a BAP address in the F1 setup request message.

- XnAP signalling from the source F1 terminating donor-CU. This option may involve the introduction of a new XnAP procedure. Besides, to avoid any confusion in case of DU migration of several mobile IAB-nodes at the same time, an additional piece of information known by the target F1 terminating donor-CU shall be provided along with the UE XnAP ID. This additional information may be the BAP address selected by the RRC terminating donor-CU for the mobile IAB-node, assuming this BAP address was present in the F1 setup request message from the target logical mobile IAB-DU to the target F1 terminating donor-CU, and assuming this BAP address is also available at the source F1 terminating donor-CU.

Based on the above considerations, we can conclude that there is less specifications impact when F1AP signalling from the target mobile IAB-DU is selected for transferring an ID of the mobile IAB MT to the target F1 terminating donor-CU, and that it is preferable to use the UE XnAP ID for the identification of the mobile IAB-MT.

Proposal 3: For the case of DU migration of a mobile IAB-node, F1AP signalling from the target mobile IAB-DU is selected for transferring the UE XnAP ID of the mobile IAB MT (at the RRC terminating donor-CU) to the target F1 terminating donor-CU.

At network integration of a mobile IAB-node

From [4], we have the following agreement:

During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU.

Considering the network integration case, there are two options to pass the identification information to the F1 terminating donor-CU (i.e. the mIAB-DU’s CU):

- Option 1: through F1AP signalling, the mobile IAB-node provides the identification information based on information received from the RRC terminating donor-CU (i.e. the mIAB-MT’s CU),

- Option 2: through XnAP signalling, the RRC terminating donor-CU provides the identification information once the RRC terminating donor-CU knows the identifier of the F1 terminating donor-CU from the mobile IAB-node.

With the option 2, the RRC terminating donor-CU shall provide an identifier of the mobile IAB-node known by the F1 terminating donor-CU. This identifier may be the BAP address of the mobile IAB-MT selected by the RRC terminating donor-CU, assuming this BAP address was present in the F1 setup request message from the mobile IAB-DU to the F1 terminating donor-CU.

With the option 1, the F1AP signalling can be the same as for the case of DU migration. Moreover, it seems there is less specifications impact with option 1 than with option 2.

Proposal 4: For the case of network integration of a mobile IAB-node, F1AP signalling from the mobile IAB-DU is selected for transferring the gNB-ID of the RRC terminating donor-CU and the UE XnAP ID of the mobile IAB MT (at this RRC terminating donor-CU) to the F1 terminating donor-CU.

At MT migration of a mobile IAB-node

Assuming the same F1AP signalling is adopted to pass identification information at DU migration and at network integration of a mobile IAB-node, then, for the sake of consistency, the mobile IAB-node should also provide identification information related to the target RRC terminating donor-CU at MT migration.

Proposal 5: For the case of MT integration of a mobile IAB-node, F1AP signalling from the mobile IAB-DU is selected for transferring the gNB-ID of the target RRC terminating donor-CU and the UE XnAP ID of the mobile IAB MT (at this RRC terminating donor-CU) to the F1 terminating donor-CU.

The F1AP procedure gNB-DU Configuration Update is appropriate to inform the F1 terminating donor-CU in the case of MT migration.

Proposal 6: The gNB-DU Configuration Update procedure can be used by a mobile IAB-node to inform the F1 terminating donor-CU about the gNB-ID of the RRC terminating donor-CU and the UE XnAP ID of the mobile IAB-MT at this CU.

From [1], there is an agreement stating that:

The mIAB-MT’s source donor CU can send the info on the mIAB-MT’s target donor CU to the mIAB-DU’s donor CU after the completion of IAB-MT HO

Actually, the proposal is compatible with this agreement as the source RRC terminating donor-CU (i.e. the mIAB-MT’s source donor CU) would pass the information to the F1 terminating donor-CU (i.e. the mIAB-DU’s donor CU), not directly, but through the mobile IAB-node.

# Conclusion

In this contribution, we have discussed the possible rejection of DU migration for a mobile IAB-node, and the procedures at network integration, MT migration and DU migration, to inform a F1 terminating donor-CU about the identifier of the RRC terminating donor-CU and the ID of the mobile IAB-MT at this CU. Then, we have made the following proposals:

Proposal 1: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU may include the number of connected UEs served by the mobile IAB-node.

Proposal 2: The target F1 donor-CU may accept the DU migration of a mobile IAB-node with some conditions, e.g. with a maximum number of UEs that can be served.

Proposal 3: For the case of DU migration of a mobile IAB-node, F1AP signalling from the target mobile IAB-DU is selected for transferring the UE XnAP ID of the mobile IAB MT (at the RRC terminating donor-CU) to the target F1 terminating donor-CU.

Proposal 4: For the case of network integration of a mobile IAB-node, F1AP signalling from the mobile IAB-DU is selected for transferring the gNB-ID of the RRC terminating donor-CU and the UE XnAP ID of the mobile IAB MT (at this RRC terminating donor-CU) to the F1 terminating donor-CU.

Proposal 5: For the case of MT integration of a mobile IAB-node, F1AP signalling from the mobile IAB-DU is selected for transferring the gNB-ID of the target RRC terminating donor-CU and the UE XnAP ID of the mobile IAB MT (at this RRC terminating donor-CU) to the F1 terminating donor-CU.

Proposal 6: The gNB-DU Configuration Update procedure can be used by a mobile IAB-node to inform the F1 terminating donor-CU about the gNB-ID of the RRC terminating donor-CU and the UE XnAP ID of the mobile IAB-MT at this CU.

# References

[1] Chairman notes of RAN3#118 meeting

[2] Chairman notes of RAN3#119 meeting

[3] Chairman notes of RAN3#119-bis-e meeting

[4] Chairman notes of RAN3#120 meeting

[5] Chairman notes of RAN3#121 meeting