3GPP TSG-RAN WG3 #121bis                                                R3-235449

Xiamen, China, 9th – 13th Oct, 2023

Agenda Item:	10.2.2

Source:	CATT

Title:	(TP for SON 37.340 and 38.423) Discussion on MRO enhancement for CPAC

Document for:	other

Introduction

In last RAN3 meeting, we made general discuss and achieved some agreements. In the document, we provide some analysis on the below topic of MRO enhancements:

MRO for CPC and CPA based on the R17 NR-DC MRO solution

Discussion

2.1 stage impact

In last RAN3 meeting, we agreed stage2 description as below:

10.18.X Conditional PSCell addition or change failure One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows: - Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE. - Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE. - CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases: - if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA to a (candidate) target SN, but not one of the candidate PSCells selected by the (candidate) target SN, it is wrong target PSCell selection at the (candidate) target SN; - else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA.

There is a figure below to illustrate the relationship between every kind of candidate cell list.

The yellow part indicates the candidate cell list which is allocated by initiating node.

The green part within the yellow part indicates the candidate cell list which is selected by target SN.

There are handover source PSCell and target PSCell in the figure.

Except for above cells, the blue part indicates the cells other than source, target and candidate cells.

In CPC/CPA Execution to wrong PSCell failure type in stage 2 description, the branch “if ” indicates the yellow part(not including green part), i.e. the suitable PSCell is in the list of candidate PSCell provided by the initiating node, but not selected by target SN which is illustrated in CASE 1 in the figure.

Except for CASE 1, there are two parts: blue part and green part which equal to the branch “else”.

The blue part (CASE 3) means the suitable PSCell is not in the candidate PSCell list at all. This case is also CPC/CPA Execution to wrong PSCell failure type, and if the suitable PSCell is within blue part, it is wrong candidate PSCell list selection at the initiating node should be optimized.

The green part (CASE 2) means the suitable PSCell is definitely selected by target SN but not selected by UE at last. This case is also CPC/CPA Execution to wrong PSCell failure type, but it is wrong candidate execution condition needs optimization.

So, for the branch “else”, not only candidate PSCell list selection (CASE 3) but also candidate PSCell execution condition may be optimized (CASE 2).

The stage2 description may be updated as below:

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA to a (candidate) target SN, but not one of the candidate PSCells selected by the (candidate) target SN, it is wrong target PSCell selection at the (candidate) target SN;

-	else, it is wrong candidate PSCell list selection and/or wrong candidate PSCell execution conditions at the node initiating the CPC or at the MN initiating the CPA.

Proposal 1: It is proposed for RAN3 to agree the above stage2 description.

2.2 The content of SCGFailureInformation message

In previous RAN3 meeting, we have agreed the principle on which information shall be included in the SCGFailureInformation as below:

Information available in the network nodes should not be included in the SCGFailureInformation.

We think time related information is known by network and the following information is not needed:

- time elapsed since the last CPC configuration until SCG failure

- time elapsed since CPAC execution until SCG failure

- time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell

In R17 MRO for CHO, we discussed how to define and record time related information in RLF Report which mainly includes the following time point:

a). the time when UE receives RRCReconfiguration message which includes CHO configuration.

b). the time when CHO executes.

c). the time when failure occurs.

As for MRO for CPA/CPC, the corresponding time is as below:

a). the time when UE receives RRCReconfiguration message which includes CPA/CPC configuration.

b). the time when CPA/CPC executes.

c). the time when SCG failure occurs.

For a), it is network to send RRCReconfiguration message, so, network knows the time point.

For b), when CPA/CPC executes, RRCReconfigurationComplete message which includes selectedCondRRCReconfig field is initiated from UE to network to indicate the selected PSCell. So, network is aware of the execution time point and the target cell towards which the CPAC was executed. Also, for (7), failedPSCellId field has been included in SCG failure information message which indicates the physical cell id and carrier frequency of the cell in which SCG failure is detected or the target PSCell of the failed PSCell change or failed PSCell addition. It can been reused in CPAC case.

RRCReconfigurationComplete message

-- ASN1START

-- TAG-RRCRECONFIGURATIONCOMPLETE-START

RRCReconfigurationComplete ::=              SEQUENCE {

rrc-TransactionIdentifier                   RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

rrcReconfigurationComplete                  RRCReconfigurationComplete-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

……

RRCReconfigurationComplete-v1700-IEs ::=    SEQUENCE {

needForGapNCSG-InfoNR-r17                   NeedForGapNCSG-InfoNR-r17                                               OPTIONAL,

needForGapNCSG-InfoEUTRA-r17                NeedForGapNCSG-InfoEUTRA-r17                                            OPTIONAL,

selectedCondRRCReconfig-r17                 CondReconfigId-r16                                                      OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                                                             OPTIONAL

}

-- TAG-RRCRECONFIGURATIONCOMPLETE-STOP

-- ASN1STOP

selectedCondRRCReconfig This field indicates the ID of the selected conditional reconfiguration the UE applied upon the execution of CPA or inter-SN CPC.

For c), SCGFailureInformation message is initiated from UE to network when SCG failure occurs. So, network is aware of the SCG failure time point.

In a word, since network is aware of the every time point during CPA/CPC procedures, it is not needed for UE to keep and report them.

Proposal 2: Since network is aware of the every time point during CPA/CPC procedures, it is not needed for UE to keep and report time related information.

2.3 XN interface message

Here we continue discussing the XN interface impace,according to the agreement as below:

Reusing R17 signalling mechanism to report CPA/CPC failure/ related information over Xn from MN to source SN or last serving SN

In R17, SCG Failure Information Report and SCG Failure Transfer message have been introduced to perform NR-DC MRO analysis. MRO for CPAC can reuse this R17 procedure.

In last RAN3 meeting, the following agreement has been captured.

Include CPC candidate cell list and CPC execution condition(s) to the message from the MN to the source SN.

SCG FAILURE INFORMATION REPORT message is used to report a PSCell change failure event from MN to SN, so, we propose to include candidate PSCell list and execution conditions in SCG FAILURE INFORMATION REPORT message.

Proposal 3: It is proposed to include candidate PSCell list and execution conditions in SCG FAILURE INFORMATION REPORT message.

2.4 intra-SN issues

In R17 MRO for PSCell change, MN may first use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN.

In last RAN3 meeting, intra-SN issue was also discussed in case of CPAC and the open issue is as below:

- For CPA or MN initiated CPC,  there are two options:

Option 1:

  - if the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell was selected by the (candidate) target SN via SCG Failure Information Report message.

  - The following information are included in the message from the MN to the (candidate) target SN:

    - Suitable PSCell ID

    - FFS: Indication whether a candidate PSCell is accepted by the (candidate) target (The candidate PSCell list recommended by the initiating node is already agreed).

  - Otherwise, the MN makes the optimization

Option 2:

  - MN always send the SCG Failure Information Report message to the last serving SN.

  - The last serving SN feedback if the problem is not brought by itself.

  - the MN makes the optimization

- For SN initiated CPC,  there are two options:

Option 1:

  - The MN sends SCG failure information report message to source SN, and source SN performs root cause analysis

  - if the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN, the source SN sends message to the MN, then the MN sends message to the (candidate) target SN.

  - Otherwise, the source SN makes the optimization

    - SCG FAILURE TRANSFER is used from the source SN to the MN

    - SCG Failure Information Report message is used from the MN to the (candidate) target SN

Option 2: the same as the option 2 for MN initiated CPC

We think MN cannot be aware of the occurrence of intra-SN CPC. There are main two cases for intra-SN CPC as below:

Case 1: SCG failure occurs shortly after a successful intra-SN CPC.

Case 2: intra-SN CPC failure.

For case 1, SN may inform MN about each PSCell change if MN sets PSCell Change History IE to "reporting full history" included in the S-NODE ADDITION REQUEST message. But for case 2, MN is not informed because intra-SN CPC is not finished successfully. So, MN may be not aware of the occurrence of intra-SN CPC and should check it with SN.

Proposal 4: It is proposed for MN to first check with SN about the occurrence of intra-SN CPC because MN may be not aware of the case 1 and case 2 mentioned above.

If intra-SN CPC occurs, it is definitely the last serving SN to select the suitable PSCell and then make optimization. While in other cases, MN shall select the next suitable PSCell. If the suitable PSCell is one of the candidate PSCells provided by the MN/source SN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell was selected by the (candidate) target SN via SCG Failure Information Report message.

It seems a little complicated as the figure below:

After informed about no intra-SN CPC occurs in step 2, MN may have to send SCG Failure Information Report message again to last serving SN to provide the suitable PSCell. We think it is a little complicated and it is proposed to include the suitable PSCell in step 1, i.e. during checking intra-SN issue.

Proposal 5: It is proposed to include the suitable PSCell in the first SCG Failure Information Report message after receiving SCG failure information from UE.

Therefore, the finally procedures may be:

- After receiving SCG Failure Information from UE, MN first select the suitable PSCell and then send SCG Failure Information Report message including the suitable PSCell selected by MN to last serving SN to check intra-SN CPC issue.

- Last serving SN makes optimization based on the suitable PSCell selected by SN if intra-SN CPC occurs. Otherwise, if the suitable PSCell provided by the MN is one of the candidate PSCells, but not one of the candidate PSCells selected by the last serving SN, last serving SN may optimize candidate PSCell selection. Also, last serving SN may send SCG Failure Transfer message to MN if there is other node needs optimization.

- MN may make optimization based on suitable PSCell selected by MN when receiving SCG Failure Transfer message and then send SCG Failure Information Report message including the suitable PSCell to source SN for SN initiated CPC.

- Source SN make optimization based on suitable PSCell selected by MN when receiving SCG Failure Information Report message from MN.

Proposal 6: It is proposed for RAN3 to discuss the above procedures.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Proposal 1: It is proposed for RAN3 to agree the above stage2 description.

Proposal 2: Since network is aware of the every time point during CPA/CPC procedures, it is not needed for UE to keep and report time related information.

Proposal 3: It is proposed to include candidate PSCell list and execution conditions in SCG FAILURE INFORMATION REPORT message.

Proposal 4: It is proposed for MN to first check with SN about the occurrence of intra-SN CPC because MN may be not aware of the case 1 and case 2 mentioned above.

Proposal 5: It is proposed to include the suitable PSCell in the first SCG Failure Information Report message after receiving SCG failure information from UE.

Proposal 6: It is proposed for RAN3 to discuss the above procedures.

# 4. Reference

[1] Chair notes, RAN3#121

# 5. TP for 37.340

10.18.1	General

For analysis of PSCell change failure, the UE makes the SCG Failure Information available to the MN.

MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

For conditional PSCell addition or change failure case, after MN performs initial analysis to identify the node that caused the failure, the MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN which may also include the suitable PSCell selected by MN to inform the last serving SN to make optimization on candidate PSCell list selection and/or wrong candidate PSCell execution conditions.

10.18.A	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA to a (candidate) target SN, but not one of the candidate PSCells selected by the (candidate) target SN, it is wrong target PSCell selection at the (candidate) target SN;

-	else, it is wrong candidate PSCell list selection and/or wrong candidate PSCell execution conditions at the node initiating the CPC or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

# 6. TP for 38.423

9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node  S-NG-RAN node .

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | The SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| CPAC Configuration | O |  | 9.2.2.xy |  | YES | ignore |
| Suitable PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell for PSCell change and addition procedure | YES | ignore |

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-HashedUEIdentityIndexValue,

id-RaReportIndicationList,

id-SuccessfulPSCellChangeReportInformation,

id-CPACConfiguration,

id-SuitablePSCellCgi,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells

FROM XnAP-Constants;

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

-- SCG FAILURE INFORMATION REPORT

--

-- **************************************************************

ScgFailureInformationReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureInformationReport-IEs}},

...

}

ScgFailureInformationReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-SourcePSCellCGI								CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-FailedPSCellCGI								CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-SCGFailureReportContainer					CRITICALITY ignore		TYPE SCGFailureReportContainer					PRESENCE mandatory }|

{ ID id-SNMobilityInformation				CRITICALITY ignore		TYPE SNMobilityInformation						PRESENCE optional }|

{ ID id-CPACConfiguration					CRITICALITY ignore		TYPE CPACConfiguration				         PRESENCE optional}|

{ ID id-SuitablePSCellCgi					CRITICALITY ignore		TYPE GlobalNG-RANCell-ID					         PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-HashedUEIdentityIndexValue																		ProtocolIE-ID ::= 372

id-ChannelOccupancyTimePercentageUL																	ProtocolIE-ID ::= xx1

id-EnergyDetectionThresholdUL																		ProtocolIE-ID ::= xx2

id-SuccessfulPSCellChangeReportInformation															ProtocolIE-ID ::= xx3

id-PSCellListContainer																				ProtocolIE-ID ::= xx4

id-RadioResourceStatusNR-U																			ProtocolIE-ID ::= xx5

id-CPACConfiguration																				ProtocolIE-ID ::= xxx

id-SuitablePSCellCgi																				ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>