3GPP TSG-RAN WG3 Meeting #121bis	R3-235487

Xiamen, China, October 9 – 13, 2023

Title: 	(TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Load Balancing use case

Source: 	Huawei

Agenda item:	12.2.2.1

Document Type:	Discussion and decision

# 1. Introduction

In last RAN3#121 meeting, the following agreements and left issues were captured concerning the support of the inference-based Load Balancing (LB) use case over the Xn interface [1][2]:

Do not support the finer granularity of UE performance feedback in R18.

The addition of a Partial Reporting Indication in the Data Collection Request message is not pursued in Rel18

Agree to introduce cause value(s) indicating failures due to timing issues. Further discussions are needed on which timing issues to address.

Introduction of cause values indicating failures due to combination of requested information is not pursued in Rel18.

A new IE is introduced in the Data Collection Request, indicating the Reporting Duration for the UE Performance Feedback, starting at successful Handover Execution. Any UE Performance Feedback reporting should occur no later than the expiration of the Reporting Duration.

Periodic and one time UE Performance Feedback Reporting are supported.

It needs to be further discussed whether to reuse Reporting Periodicity or to introduce a new dedicated Reporting Period for periodic reporting of the UE Performance Feedback

If the UE moves to Idle/Inactive or it hands over to another cell during the Reporting Duration, collection of UE Performance Feedback is terminated. Whether an explicit or implicit indication on the latter events or any other events is signalled to the requesting node needs further discussions.

In this paper, we discuss the above listed left issues for enabling the LB use case by means of AI/ML.

# 2. Discussion

## 2.1 Partial reporting

The support of partial reporting in the agreed new class 1/2 procedures helps the requesting node to obtain more data for AI/ML purposes, even in the case where only a subset of the requested measurements can be performed by the requested node. One of the left issues discussed in RAN3#121 meeting was related to enhancements for this mechanism, with the following options to select among [1]:

Partial Response:

- 	Opt1: Add a flag in the request message

- 	Opt2: Introduce new cause values in the failure measurement objectives in the response message, e.g., “partial 					measurement not available”

- 	Op3: Add priority indication for one or more requested measurement objectives in the request message

Eventually RAN3 agreed that

The addition of a Partial Reporting Indication in the Data Collection Request message is not pursued in Rel18

Still concerning the partial reporting mechanism, from the RAN3#119bis-e meeting there was another open issue about the need to introduce in the DATA COLLECTION RESPONSE message (commonly known as AI/ML INFORMATION RESPONSE at that time) the list of measurements that will be successfully performed by the requested node – see highlighted text in yellow reported below [3]:

Introduce the failed measurement in the response message to indicate partial reporting result. The successful measurement list and failure cause need to be further discussed.

We do not see the need for such information to be provided to the requesting node: since we already agreed that the requested node will provide the set of failed measurements, then the requesting node can deduce the measurements that will be performed by the requested node – and reported in the DATA COLLECTION UPDATE message – by simply comparing the set of measurements initially requested in the DATA COLLECTION REQUEST message with the set of failed measurements reported in the DATA COLLECTION RESPONSE message. Therefore, based on the above, we propose the following:

Proposal 1: The list of measurements that will be successfully performed by the requested node is not needed in the DATA COLLECTION RESPONSE message.

In RAN3#119bis-e RAN3 agreed to indicate in the DATA COLLECTION RESPONSE message the set of measurements that failed to be performed by the requested node, so as to notify the requesting node of the partial reporting results. In addition to this, RAN3 also discussed about the possible failure causes that could help the requesting node to understand why these measurements failed to be performed, but no consensus was reached. The issue was re-discussed in the past two RAN3 meetings, and in the last RAN3#121 it was agreed that only cause values for failed measurements due to timing issues are considered in Rel-18, hence ruling out cause values due to combination of requested information [1]:

Agree to introduce cause value(s) indicating failures due to timing issues. Further discussions are needed on which timing issues to address.

Introduction of cause values indicating failures due to combination of requested information is not pursued in Rel18.

There is however an open issue on which actual timing issues to address and to encode as corresponding cause values – see highlighted text in cyan above.

In [4] it was proposed to introduce the following failure causes in the AI/ML INFORMATION RESPONSE (FFS on the name) message – now renamed in DATA COLLECTION RESPONSE message – on top of the legacy ones “measurement not supported” and “measurement temporarily not available”:

- measurement not supported with requested reporting periodicity

- measurement not available with requested reporting periodicity

- measurement not supported with current combination of requested information

- measurement not available with current combination of requested information

Among the above reported failure causes, we think that the ones that could at least be introduced are the “not available” ones. The other ones, i.e. the “not supported” type of failure causes, provide limited information to the requesting node to counter-react, other than exposing the AI/ML model’s capabilities. But since there is a legacy cause which is a “not supported” one, we think that also this type of failure cause could be worth to be introduced. In light of the agreement that

Introduction of cause values indicating failures due to combination of requested information is not pursued in Rel18.

and also considering that some of the proposed failure causes above concern only a subset of the AI/ML prediction timing information (i.e., the “reporting periodicity”) we think it could be worth to introduce also the failure causes related to the not support of and not availability of measurements configured with other AI/ML prediction timing information, at least the specific “requested prediction time” as indicated in the DATA COLLECTION REQUEST message.

Proposal 2: Introduce the failure cause per measurement to be indicated to the requesting node in the DATA COLLECTION RESPONSE message along with the set of failed measurements. Possible failure causes are:

- measurement not available with requested reporting periodicity

- measurement not available with the specific requested prediction time

- measurement not supported with requested reporting periodicity

- measurement not supported with the specific requested prediction time

Moreover, if a certain measurement that is needed by the requesting node is “not available”, then the requesting node may try to ask for such measurement again once it will be possible to comply with the requested node’s current configuration (e.g. for the case of “measurement not available with requested reporting periodicity”) or current operating conditions (e.g. for the case of “measurement temporarily not available”). To this end, we think it could be good for the requesting node to have an indication on when it will be possible to ask for those measurements that were not available before, in order to avoid a «trial-and-error» recursive process and hence reduce signalling exchange over RAN interfaces.

- Proposal 3: RAN3 to discuss and agree on the introduction of timing information per failed measurement whose failure cause is a “not available” type of failure in the DATA COLLECTION RESPONSE message in order for the requesting node to determine when to request again the previously failed measurement.

## 2.2 UE performance usage

From the RAN3#119bis-e meeting [3] there is the open issue reported below:

Discuss whether UE performance can be used as both input or feedback first.

The above FFS comes from the discussion on the need to have an indication in the agreed new class 1 procedure to inform the requested node about whether the needed measurements need to be reported immediately or after a specific AI/ML action (e.g., handover event). To be specific, such indication should prevent the requested node from sending measurements immediately but only after UEs have been handed over to the requested node.

We think that if the Report Characteristics IE in the DATA COLLECTION REQUEST message has the “Average UE Throughput DL” – “Average UE Throughput UL” – “Average Packet Delay” – “Average Packet Loss” bits set to 1, then it is obvious that such UE performance will be collected for those UEs that were actually handed over, and hence after the HO event only. In addition, introducing such kind of indication is contradicting with the agreement we had on the fact that procedures used for AI/ML purposes have to be data type agnostic. Moreover, we acknowledge that UE performance can be used as both input or feedback as per TR 37.817 (refer to §5.2.2.4), however, among the possible input data to the AI/ML model, the TR explicitly mentions “UE performance measurement at traffic offloaded neighbouring cell”, meaning that such UE performance needs to be collected after a traffic offload (i.e., a handover) happened. Therefore, we propose the following:

Proposal 4: RAN3 to agree that no explicit indication is needed in the DATA COLLECTION REQUEST message to indicate that UE performance needs to be reported after an inference-based action (e.g. handover event).

In the last RAN3 meeting the timing configuration to be provided to the requested node for the UE performance feedback measurement reporting was discussed and the following was agreed [1]:

A new IE is introduced in the Data Collection Request, indicating the Reporting Duration for the UE Performance Feedback, starting at successful Handover Execution. Any UE Performance Feedback reporting should occur no later than the expiration of the Reporting Duration.

Periodic and one time UE Performance Feedback Reporting are supported.

Referring to Figure 1, in order for Node 2 (requested node) to provide Node 1 (requesting node) with the UE performance feedback associated to those UEs that have actually been handed over from Node 1 to Node 2 as a consequence of an inference-based LB action taken at Node 1 level, Node 2 needs to be properly “instructed” not only by listing the UE performance feedback metrics that Node 1 intends to monitor, but also indicating the timing configuration that Node 2 needs to apply for collecting and then reporting such UE performance feedback metrics.

Figure 1 – Call flow for the inference-based LB action.

Based on the agreements from the last RAN3 meeting, the collection of UE performance feedback measurements is done within a measurement collection time window indicated as Report Duration (refer to Figure 2) which starts at successful handover execution (indicated as t1 for the sake of the graphical representation).

Figure 2 – UE performance feedback measurement collection and reporting.

The Report Duration is indicated by means of a new IE in the DATA COLLECTION REQUEST message, during which periodic reporting of UE performance feedback is possible, but also one-time reporting is allowed to happen at the end of the Report Duration, i.e., at the end of the measurement collection time window (at time t2 as indicated in Figure 2).

Still concerning the UE performance feedback reporting timing configuration, there are some open issues from the last meeting as reported below [1]:

It needs to be further discussed whether to reuse Reporting Periodicity or to introduce a new dedicated Reporting Period for periodic reporting of the UE Performance Feedback

If the UE moves to Idle/Inactive or it hands over to another cell during the Reporting Duration, collection of UE Performance Feedback is terminated. Whether an explicit or implicit indication on the latter events or any other events is signalled to the requesting node needs further discussions.

The fist issue to be dealt with concerns details of the periodic reporting of UE performance feedback, where the UE performance feedback could be sent multiple times within the Report Duration, each reporting being performed with a certain periodicity. An issue to solve is related to whether the existing Reporting Periodicity IE within the DATA COLLECTION REQUEST message can be reused or a new IE needs to be introduced to determine the time spacing between two adjacent reporting occasions, i.e., between two DATA COLLECTION UPDATE messages received one after the other, each message containing a sample of the performance metric(s) of the concerned UE.

We think that the periodicity for UE performance feedback reporting should be set by means of a new IE in the DATA COLLECTION REQUEST message – indicated as Tfeed in Figure 2 – which is different from the currently specified Reporting periodicity IE in the DATA COLLECTION REQUEST message: this because the Reporting periodicity IE should apply in principle only to the reporting of measurement objects for which a periodic prediction is requested, which might be needed to be reported more frequently than UE performance feedback. Therefore, we propose the following:

Proposal 5: RAN3 to agree that, for the case of periodic reporting, a new IE is introduced in the DATA COLLECTION REQUEST message to indicate the periodicity of UE performance feedback reporting occasions, which is different from the currently specified Reporting periodicity IE.

Another open issue to solve is related to the effects on the UE performance feedback collection and reporting due to those UE events that could occur during the Report Duration, i.e., during the UE performance feedback measurement collection time window, such as subsequent UE handover to another cell/NG-RAN node or UE’RRC state change (from connected to idle/inactive). RAN3 agreed that when these events happen the collection of UE performance feedback is terminated, however it needs to be discussed whether an explicit or implicit indication on the events (or any other events) is signalled to the requesting node. In our view, it should be possible for the requested node to still provide to the requesting node the UE performance feedback collected up to the time instant at which a particular event (RRC state change, subsequent UE handover, …) occurred, since the collected UE performance feedback could still be useful for e.g., AI/ML model monitoring and retraining.

Based on that, we think that the introduction of an explicit indication to let the requesting node know that a particular UE event happened during the Report Duration is not needed: since the requesting node knows the Report Duration, the fact that it receives the UE performance feedback from the requested node before the expiration of such measurement collection time window is per se an implicit indication that a certain event occurred. Moreover, in case an explicit indication is introduced, this indication should be characterized further in order to highlight which particular UE event took place, which over-complexifies the signalling while providing negligible benefit. Therefore, we have the following proposals:

Proposal 6: RAN3 to agree that the requested node provides to the requesting node the UE performance feedback collected up to the time instant at which a particular UE event (RRC state change, subsequent UE handover, …) occurred during the Report Duration time window.

Proposal 7: RAN3 to agree that no explicit indication is needed to be introduced in order for the requesting node to understand that a specific UE event occurred during the Report Duration time window.

## 2.3 Remaining issues

- With the collected resource status information from the requested node itself and neighbour nodes, including the inference output for predicted resource occupation, the requesting node can use the trained AI/ML model to further inference the LB strategies. For example, the requesting node has foreseen that its own resource occupation will be heavy within a period of time. Then, for offloading purposes, the requesting node will choose a number of UEs and handover them to a neighbour node with predicted light load.

- However, since LB strategies are based on predictions which, by definition, are not absolutely accurate, the neighbour node should be able to deny the incoming handover aimed for offloading purposes. Thus, we think that the requesting node should, when using the handover procedure, indicate that this incoming handover is for inference-based LB purpose.

- Proposal 8: RAN3 to discuss and agree that an incoming handover for the purpose of inference-based LB should be identifiable by the requested (target) node.

- Once the requested node is selected, the requesting node should inform the requested node of the time of handover before it starts. In our view, the requested node should be aware of the timestamp of the inference-based handover, in order to prepare for the handover resource timely.

- Proposal 9: Requesting (source) node to inform the requested (target) node of the handover timestamp related to the inference-based handover.

In the following agreement that RAN3 reached in RAN3#119bis-e meeting there is an open point, see text highlighted in grey [3]:

Introduce the pair Measurement ID (e.g., NG-RAN node1 Measurement ID and NG-RAN node2 Measurement ID) in the HO request message, to establish relationship with the AI/ML INFORMATION REQUEST message. Any additional information to be added can be further discussed.

The above agreement is related to the details of the trigger indication in the HANDOVER REQUEST message to indicate that UE performance feedback is requested after HO completion. Even though we agreed that the pair of Measurement IDs in the HANDOVER REQUEST message is enough to relate the HANDOVER REQUEST message itself to the DATA COLLECTION REQUEST message (previously known as AI/ML INFORMATION REQUEST message), we think that both the handover timestamp and inference-based LB handover cause indication in the HANDOVER REQUEST message could be jointly used as an additional implicit trigger indication in the HANDOVER REQUEST message that UE performance feedback is requested after HO completion. Therefore, we propose:

- Proposal 10: The handover timestamp and inference-based LB handover cause indication in the HANDOVER REQUEST message can be used jointly as additional implicit trigger indication that UE performance feedback is requested after HO completion.

The corresponding TP to TS 38.423 reflecting the above proposals is provided in Annex-1.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

Proposal 1: The list of measurements that will be successfully performed by the requested node is not needed in the DATA COLLECTION RESPONSE message.

Proposal 2: Introduce the failure cause per measurement to be indicated to the requesting node in the DATA COLLECTION RESPONSE message along with the set of failed measurements. Possible failure causes are:

- measurement not available with requested reporting periodicity

- measurement not available with the specific requested prediction time

- measurement not supported with requested reporting periodicity

- measurement not supported with the specific requested prediction time

- Proposal 3: RAN3 to discuss and agree on the introduction of timing information per failed measurement whose failure cause is a “not available” type of failure in the DATA COLLECTION RESPONSE message in order for the requesting node to determine when to request again the previously failed measurement.

Proposal 4: RAN3 to agree that no explicit indication is needed in the DATA COLLECTION REQUEST message to indicate that UE performance needs to be reported after an inference-based action (e.g. handover event).

Proposal 5: RAN3 to agree that, for the case of periodic reporting, a new IE is introduced in the DATA COLLECTION REQUEST message to indicate the periodicity of UE performance feedback reporting occasions, which is different from the currently specified Reporting periodicity IE.

Proposal 6: RAN3 to agree that the requested node provides to the requesting node the UE performance feedback collected up to the time instant at which a particular UE event (RRC state change, subsequent UE handover, …) occurred during the Report Duration time window.

Proposal 7: RAN3 to agree that no explicit indication is needed to be introduced in order for the requesting node to understand that a specific UE event occurred during the Report Duration time window.

- Proposal 8: RAN3 to discuss and agree that an incoming handover for the purpose of inference-based LB should be identifiable by the requested (target) node.

- Proposal 9: Requesting (source) node to inform the requested (target) node of the handover timestamp related to the inference-based handover.

- Proposal 10: The handover timestamp and inference-based LB handover cause indication in the HANDOVER REQUEST message can be used jointly as additional implicit trigger indication that UE performance feedback is requested after HO completion.

# 4. Reference

- 3GPP RAN3#120 Chairman’s notes (RAN3_121_agenda_20230825_EOM1.doc)

- R3-234548, Summary of Offline Discussion: CB: # AIRAN2_LB, Ericsson

- 3GPP RAN3#119bis-e Chairman’s notes (RAN3_119bis-e_agenda_202304026_EOM1.doc)

- R3-231616, (TP for AI/ML BLCR to TS38.423) Partial success for the AI-ML Assistance Data Reporting procedure, Ericsson, InterDigital

# 5. Annex - 1: TP to 38.423 (based on BL CR in R3-234786)

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

****** skip unchanged parts ******

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the Cell Based UE Trajectory Prediction IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider the content of this list as the predicted cells for UE trajectory by the source NG-RAN node, and may use it for e.g. mobility decisions.

If the Intended Timestamp for Handover Execution IE is contained in the HANDOVER REQUEST message, the target NG-RAN node considers the time as the predicted time the UE accesses the target cell, and may use it for e.g. access parameter optimization.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

Interaction with Data Collection Reporting procedure:

If the Data Collection Measurement ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after the completion of handover procedure, via the Data Collection Reporting procedure, the requested AI/ML information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement and the NG-RAN node2 Measurement ID IEs.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.AA	Data Collection Reporting Initiation

#### 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

-	FFS

If the Registration Request IE is set to “start” in the DATA COLLECTION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all of the requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable to provide some but not all of the requested information, it shall initiate the AI/ML related information reporting for the admitted requested information and include the Node Measurement Initiation Result IE or the Per Cell Measurement Initiation Result IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the information requested applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the Report Duration IE in the DATA COLLECTION REQUEST indicating the time window applicable to the collection of the requested objects in the Report Characteristics IE is included, the NG-RAN node2 shall, if supported, provide the objects up to the Report Duration IE.

If the Measurement Collection Reporting Periodicity IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related measurement information. The NG-RAN node2 shall report only once, unless otherwise requested within the Measurement Collection Reporting Periodicity IE.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.x |  | YES | ignore |
| Data Collection Measurement ID | O |  | 9.2.3.M |  | YES | Ignore |
| Intended Timestamp for Handover Execution | O |  | INTEGER (0..1000000, …) | Expressed in units of 1 ms. | – | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.CC	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Energy Cost | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested prediction objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (0..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. For periodic reporting, it indicates the points in time, measured from the reception of the DATA COLLECTION REQUEST message and shifted by each reporting period, for which predictions are provided. | YES | ignore |
| Report Duration | O |  | FFS | Time window for UE feedback measurement collection | YES | ignore |
| Measurement Collection Reporting Periodicity | O |  | FFS | Periodicity that can be used for reporting of requested measurement objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.DD	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information, for all or part of the measurement objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | ignore |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | EACH | ignore |
| >>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost. Other bits shall be ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| >>New time configuration (FFS on the name) | O |  | FFS | Time configuration indication on when it will be possible to ask for those measurements that were not available to be provided in the current request. |  |  |
| Per Cell Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | ignore |
| >Per Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | EACH | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  | – |  |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss. Other bits shall be ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| >>>>New time configuration (FFS on the name) | O |  | FFS | Time configuration indication on when it will be possible to ask for those measurements that were not available to be provided in the current request. |  |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

#### 9.1.3.EE	DATA COLLECTION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of  the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the XnAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( Cell not Available, Handover Desirable for Radio Reasons, Handover Target not Allowed, Invalid AMF Set ID, No Radio Resources Available in Target Cell, Partial Handover, Reduce Load in Serving Cell, Resource Optimisation Handover, Time Critical Handover, TXnRELOCoverall Expiry, TXnRELOCprep Expiry, Unknown GUAMI ID, Unknown Local NG-RAN node UE XnAP ID, Inconsistent Remote NG-RAN node UE XnAP ID, Encryption And/Or Integrity Protection Algorithms Not Supported, Multiple PDU Session ID Instances, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple QoS Flow ID Instances, Switch Off Ongoing, Not supported 5QI value, TXnDCoverall Expiry, TXnDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve User Bit Rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, UP integrity protection not possible, UP confidentiality protection not possible, Resources not available for the slice(s), UE Maximum integrity protected data rate reason, CP Integrity Protection Failure, UP Integrity Protection Failure, Slice(s) not supported by NG-RAN, MN Mobility, SN Mobility, Count reaches max value, Unknown Old NG-RAN node UE XnAP ID, PDCP Overload, DRB ID not available, Unspecified, …, UE Context ID not known, Non-relocation of context, CHO-CPC resources to be changed, RSN not available for the UP, NPN access denied, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available, Measurement not Supported For The Object, UE Power Saving, Not existing NG-RAN node2 Measurement ID, Insufficient UE Capabilities, Normal Release, Value out of allowed range, SCG activation deactivation failure, SCG deactivation failure due to data transmission, Handover Desirable for inference-based load balancing, Measurement not available with requested reporting periodicity, Measurement not available with the specific requested prediction time, Measurement not supported with requested reporting periodicity, Measurement not supported with the specific requested prediction time.) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure, O&M Intervention, Not enough User Plane Processing Resources, Unspecified, …) |  |

The meaning of the different cause values is specified in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Cell not Available | The concerned cell is not available. |
| Handover Desirable for Radio Reasons | The reason for requesting handover is radio related. |
| Handover Target not Allowed | Handover to the indicated target cell is not allowed for the UE in question. |
| Invalid AMF Set ID | The target NG-RAN node doesn’t belong to the same AMF Set of the source NG-RAN node, i.e. NG handovers should be attempted instead. |
| No Radio Resources Available in Target Cell | The target cell doesn’t have sufficient radio resources available. |
| Partial Handover | Provides a reason for the handover cancellation. The target NG-RAN node did not admit all PDU Sessions included in the HANDOVER REQUEST and the source NG-RAN node estimated service continuity for the UE would be better by not proceeding with handover towards this particular target NG-RAN node. |
| Reduce Load in Serving Cell | Load in serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| Resource Optimisation Handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Value out of allowed range | The action failed because the proposed Handover Trigger parameter change in the NG-RAN node2 Proposed Mobility Parameters IE is too low or too high. |
| Time Critical Handover | Handover is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| TXnRELOCoverall Expiry | The reason for the action is expiry of timer TXnRELOCoverall. |
| TXnRELOCprep Expiry | Handover Preparation procedure is cancelled when timer TXnRELOCprep expires. |
| Unknown GUAMI ID | The target NG-RAN node belongs to the same AMF Set of the source NG-RAN node and recognizes the AMF Set ID. However, the GUAMI value is unknown to the target NG-RAN node. |
| Unknown Local NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node does not recognise the local NG-RAN node UE XnAP ID. |
| Inconsistent Remote NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node considers that the received remote NG-RAN node UE XnAP ID is inconsistent.. |
| Encryption And/Or Integrity Protection Algorithms Not Supported | The target NG-RAN node is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
| Multiple PDU Session ID Instances | The action failed because multiple instances of the same PDU Session had been provided to the NG-RAN node. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown in the NG-RAN node. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknown in the NG-RAN node. |
| Multiple QoS Flow ID Instances | The action failed because multiple instances of the same QoS flow had been provided to the NG-RAN node. |
| Switch Off Ongoing | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving NG-RAN node in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
| Not supported 5QI value | The action failed because the requested 5QI is not supported. |
| TXnDCoverall Expiry | The reason for the action is expiry of timer TXnDCoverall. |
| TXnDCprep Expiry | The reason for the action is expiry of timer TXnDCprep |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. In the current version of this specification applicable for Dual Connectivity only. |
| Reduce Load | Load in the cell(group) served by the requesting node needs to be reduced. In the current version of this specification applicable for Dual Connectivity only. |
| Resource Optimisation | The reason for requesting this action is to improve the load distribution with the neighbour cells. In the current version of this specification applicable for Dual Connectivity only. |
| Time Critical action | The action is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where radio resources are likely to be dropped if the requested action is not performed. In the current version of this specification applicable for Dual Connectivity only. |
| Target not Allowed | Requested action towards the indicated target cell is not allowed for the UE in question. In the current version of this specification applicable for Dual Connectivity only. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. In the current version of this specification applicable for Dual Connectivity only. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. In the current version of this specification applicable for Dual Connectivity only. |
| Encryption Algorithms Not Supported | The requested NG-RAN node is unable to support any of the encryption algorithms supported by the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. In the current version of this specification applicable for Dual Connectivity only. |
| RRM purpose | The procedure is initiated due to node internal RRM purposes. In the current version of this specification applicable for Dual Connectivity only. |
| Improve User Bit Rate | The reason for requesting this action is to improve the user bit rate. In the current version of this specification applicable for Dual Connectivity only. |
| User Inactivity | The action is requested due to user inactivity on all PDU Sessions. The action may be performed on several levels: - on UE Context level, if NG is requested to be released in order to optimise the radio resources; or S-NG-RAN node didn’t see activity on the PDU session recently. - on PDU Session Resource or DRB or QoS flow level, e.g. if Activity Notification indicate lack of activity In the current version of this specification applicable for Dual Connectivity only. |
| Radio Connection With UE Lost | The action is requested due to losing the radio connection to the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Failure in the Radio Interface Procedure | Radio interface procedure has failed. In the current version of this specification applicable for Dual Connectivity only. |
| Bearer Option not Supported | The requested bearer option is not supported by the sending node. In the current version of this specification applicable for Dual Connectivity only. |
| UP integrity protection not possible | The PDU session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU session cannot be accepted according to the required user plane confidentiality protection policy. |
| Resources not available for the slice(s) | The requested resources are not available for the slice(s). |
| UE Maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum data rate for integrity protection supported by the UE. |
| CP Integrity Protection Failure | The request is not accepted due to failed control plane integrity protection. |
| UP Integrity Protection Failure | The procedure is initiated because the SN (hosting node) detected an Integrity Protection failure in the UL PDU coming from the MN. |
| Slice(s) not supported by NG-RAN | The failure is due to slice(s) not supported by the NG-RAN node. |
| MN Mobility | The procedure is initiated due to relocation of the M-NG-RAN node UE context. |
| SN Mobility | The procedure is initiated due to relocation of the S-NG-RAN node UE context. |
| Count reaches max value, | Indicates the PDCP COUNT for UL or DL reached the max value and the bearer may be released. |
| Unknown Old NG-RAN node UE XnAP ID | The action failed because the Old NG-RAN node UE XnAP ID or the S-NG-RAN node UE XnAP ID is unknown. |
| PDCP Overload | The procedure is initiated due to PDCP resource limitation. |
| DRB ID not available | The action failed because the M-NG-RAN node is not able to provide additional DRB IDs to the S-NG-RAN node. |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| UE Context ID not known | The context retrieval procedure cannot be performed because the UE context cannot be identified. |
| Non-relocation of context | The context retrieval procedure is not performed because the old RAN node has decided not to relocate the UE context. |
| CHO-CPC resources to be changed | The prepared resources for CHO or CPC for a UE are to be changed. |
| RSN not available for the UP | The redundant user plane resources are not available. |
| NPN Access denied | Access denied, or release is required, due to NPN reasons. |
| Report Characteristics Empty | The action failed because there is no measurement object in the report characteristics. |
| Existing Measurement ID | The action failed because the measurement ID is already used. |
| Measurement Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object. |
| Measurement not Supported For The Object | At least one of the concerned object(s) does not support the requested measurement. |
| UE Power Saving | The procedure is initiated to accommodate the preference indicated by UE to release the S-NG-RAN node for UE power saving purpose. |
| Not existing NG-RAN node2 Measurement ID | The action failed because the NG-RAN node2 Measurement ID is not used. |
| Insufficient UE Capabilities | The procedure can’t proceed due to insufficient UE capabilities. |
| Normal Release | The release is due to normal reasons. |
| SCG activation deactivation failure | The action failed due to rejection of the SCG activation deactivation request. |
| SCG deactivation failure due to data transmission | The SCG deactivation failure due to ongoing or arriving data transmission. |
| Handover Desirable for inference-based load balancing | The reason for requesting this action is inference-based load balancing strategies. |
| Measurement not available with requested reporting periodicity | The measurement is not available by the NG-RAN node and/or for the specific cell with the requested reporting periodicity. |
| Measurement not available with the specific requested prediction time | The measurement is not available by the NG-RAN node and/or for the specific cell with the specific requested prediction time. |
| Measurement not supported with requested reporting periodicity | The measurement is not supported by the NG-RAN node and/or for the specific cell with the requested reporting periodicity. |
| Measurement not supported with the specific requested prediction time | The measurement is not supported by the NG-RAN node and/or for the specific cell with the specific requested prediction time. |