3GPP TSG-RAN WG3 Meeting #121-bis	R3-235500

Xiamen, China, 9th – 13th  October, 2023

Agenda Item:	23.2.2

Source:	Huawei

Title:	(TP BL  38.xxx) Discussion on LPHAP

Document for:	Approval

# 1	Introduction

In RAN3#121, the following was concluded for LPHAP [1]:

WA: LMF provides the validity area as a list of cells, and SRS recommendation to the serving gNB. The serving gNB replies with a single SRS configuration (as in legacy positioning information exchange procedure).

The details of the SRS recommendation are FFS e.g. SRS configuration or requested SRS transmission characteristics with additional information.

FFS on whether the serving gNB includes a list of cells identifying a modification of the validity area in the response message to the LMF.

This contribution provides our view on positioning enhancements for Rel-18. The contribution expresses the Huawei’s preference, but the final proposal is a possible compromised as way forward for the LPHAP.

# 2	Discussion

## 2.1 SRS Configuration Procedure

### 2.1.1 Area-specific Resource Allocation (Validity Area)

The following agreements was made during RAN2#122. From RAN2 perspective, the LMF is able to determine the area-specific SRS configuration. In principle, The LMF controls the positioning sessions but the gNB should be the node to allocate resources. From our understanding, the intention of the agreement is that the LMF can send SRS configurations for recommendation and the serving gNB can finally decide, which is aligned with RAN3 WA during the last meeting.

Agreements:

RAN2 consider that the LMF should determine the area-specific SRS configuration.  Details are up to RAN3.

LS to RAN3 to indicate this conclusion, including RAN1 to prompt them for parameters.  To be included in the LS from [AT122][415].

A LS is received from RAN1 [2] on LPHAP with area-specific parameters for SRS:

- RAN1 has agreed the following area-specific parameters for SRS for positioning configurations in a validity area: - inactivePosSRS-TimeAlignmentTimer - inactivePosSRS-RSRP-ChangeThreshold - BWP parameters - locationAndBandwidth - subcarrierSpacing - cyclicPrefix - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - p0 and alpha - pathlossReferenceRS-Pos - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping - resourceType - sequenceID - spatialRelationInfoPos - In addition, from RAN1’s perspective, the area-specific parameters should also include the following: - A list of PCIs defining the positioning area - autonomous TA adjustment enabler

RAN1 has decided on the area-specific parameters for SRS, and they think positioning area can be defined by a list of PCIs. Therefore, the following agreements can be made based on the WA:

Proposal 1: LMF provides a list of cells as positioning validity area for SRS to the serving gNB, and the cells can be defined by a list of PCIs.

The last RAN3 meeting decision leads to two options for the following FFS:

The details of the SRS recommendation are FFS e.g. SRS configuration or requested SRS transmission characteristics with additional information.

Option 1: The LMF reuses the exiting SRS characteristics to the serving gNB, and the serving gNB allocates the SRS according to that information, just as legacy procedure.

Option 2: LMF recommends SRS configuration(s) and validity area to the serving gNB, the serving gNB further determines the SRS configuration and provides it to LMF.

For option 1, the legacy is reused that the serving gNB configures UE with the information carried by Requested SRS Transmission Characteristics IE in Positioning Information Request message, which includes the expected periodicity, bandwidth, etc. In this case, the SRS configuration is totally decided by the serving gNB, within a larger scale. The problem is the SRS configuration determined by the serving gNB may have been occupied in other cells of the expected validity area. If the LMF request for SRS reservation in the cells of the expected validity area, the neighborhood gNBs may refuse it, which may fail configuration. Or the LMF needs to send the updated validity area to the serving gNB which leads to a complex procedure, but also the UE may have been released to RRC_Inactive state as illustrated by the following figure.

Figure 1, the existing SRS characteristics are provided to the serving gNB, option 1

Observation 1. Serving gNB cannot decide on the cross-cell configuration only based on the legacy Requested SRS Transmission Characteristics IE, because the determined SRS configuration may have been occupied in other cells of the expected validity area.

For option 2, the difference is that the LMF sends area-specific SRS configuration(s) to the serving gNB as a recommendation, which is aligned with RAN2 agreements. The serving gNB can reply with the finally determined SRS configuration to the LMF.

Figure 2, LMF recommend SRS configurations to serving gNB, option 2

Observation 2. The option that LMF sends area-specific SRS configuration to the serving gNB is aligned with RAN2 agreements.

Above the discussion, we therefore proposal the following:

Proposal 2. LMF can send SRS configuration(s) to the serving gNB as recommendation, and the serving gNB replies with the finally determined SRS configuration.

Then what configurations should be recommended by the LMF? The purpose of SRS recommendation is to avoid resource conflicts over the cells of the expected validity area. A feasible method is to provide the full SRS configurations to the serving gNB, e.g. reusing SRS configuration IE.  In this case, the LMF may find a list of available SRS configurations in the validity area and can send the list to the serving gNB, which can allow more flexibility and improve the probability of successful configuration.

An alternative is to only provide some parameters, e.g. the list of area-specific parameters for SRS provided by RAN1 in [2].

Proposal 3. LMF recommends the full SRS configurations or only a list of parameters, e.g. the parameter list provided by the RAN1, to the serving gNB.

Another issue is how the LMF determines the recommended area-specific SRS configurations to the serving gNB. There are also two potential solutions based on the discussion:

Solution 1. LMF coordinate SRS via NRPPa

Solution 2. LMF coordinate SRS via OAM

Solution 1 is a straightforward solution preferred by Huawei, that the LMF can collect the available SRS configurations from the gNBs, e.g. via TRP Information Exchange procedure. While solution 2 is a solution with less spec. impact, as a cost based on the assumption that a bunch of SRS configurations are pre-reserved or pre-configured only for the LPHAP uses and the LMF can coordinate the resources as a central node. Considering the limited time of Rel-18 and controversy of the discussion, we suggest to put this topic on hold for now then how the LMF obtains the SRS configurations can be left to implementation, and the LMF sends Positioning Information Request message to the serving gNB with a suggested validity area and available SRS configurations in order to configure the UE.

Observation 3: The LMF can determine the recommended area-specific SRS configuration via NRPPa signalling or via OAM.

Proposal 4. The LMF determines the recommended area-specific SRS configurations by implementation at the current stage.

A remaining issue is whether the serving gNB can modify the validity area configuration, i.e. whether to response a list of cells. We are not against this possibility, but in last meeting discussion and contributions, we did not notice convincing argumentation which will make the gNB updating the validity area, we also did not see any proposal how the gNB could be aware of some SRS configuration not efficient in the validity area… We then would like to stop this discussion for now, with the possibility to re-open it, and have a future proof encoding that will allow such implementation in the future.

Proposal 5. The serving gNB should not be able to modify the validly area cell list in this release.

### 2.1.2 Resource Reservation

Since the cross-cell SRS configuration is expected to be used in the validity area, i.e. the SRS configuration should not be allocated to other UEs in the cells of the validity area. The LMF should coordinate the gNBs in the validity area to reserve the allocated SRS resources. Naturally, if the SRS configurations are no longer occupied by the target UE, the previous reservation should be cancelled/released.

Proposal 6. When a UE is configured with SRS configurations associated with a validity area, the LMF notify the gNBs in this area to reserve/release the allocated SRS configurations to avoid interference.

Then which message can be used for the reservation? Since the resource reservation should be applicable to all gNBs in the validity area, the existing procedures, e.g., Positioning Information Exchange, are not appropriate; they are UE associated, the validity area contains several gNB(s) not connected to the UE.  Therefore, it is proposed to define a new non-UE associated NRPPa procedure for SRS resource reserve/release for all gNB(s) in the validity area.

Proposal 7. Define a new non-UE associated NRPPa procedure for SRS resource reservation in the positioning validity area.

It is proposed to adopt the following as the baseline procedure for SRS configuration;

Proposal 8. Adopt the following as the baseline procedure for SRS configuration:

Figure 3. SRS Configuration Procedure Step 1. LMF sends POSITIONING INFORMATION REQUEST message to the serving gNB with a suggested validity area and available SRS configuration(s) in the area. The assumption is the LMF has obtained the prior knowledge of available SRS configurations in the cells by implementation, e.g. via coordination over NRPPa or via OAM. Step 2. The serving gNB determines the SRS configuration and sends it to the LMF in POSITIONING INFORMATION RESPONSE message. Step 3. The serving gNB sends the SRS configuration and the associated validity area to the UE. Step 4. The LMF sends the SRS configuration obtained in step 2 to other gNBs in the validity area for reservation.

## 2.2 SRS Configuration update when the UE leaves the validity area

As discussed previously, the SRS configuration is invalid when the UE moves out of the validity area. Based on RAN2 agreements as follows, when the UE reselects to another cell out of the Validity Area, the UE may send RRCResumeRequest message to the network for SRS configuration request, and the updated SRS configuration and validity area is delivered to the UE via RRC Release message.

Agreements:

When the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request. The SRS configuration request is sent in the RRC message RRCResumeRequest (18/18).

WA: A new resume cause is introduced for the above use case.

In Rel-17, Xn-AP has been enhanced to ensure the continuity of positioning services. When receiving the RRC Resume Request, the receiving gNB initiates Retrieve UE Context procedure to request for UE context relocation. If positioning information, i.e. Requested SRS Transmission Characteristics and Routing ID, is included in the UE context, the receiving gNB configures the UE with SRS transmission for UL positioning.

In legacy procedure for SRS update, the new serving gNB itself decides the SRS configuration configured to the UE in the new cell. However, if the new validity area is expected to be configured, the LMF should be involved to recommend new SRS configurations.

In addition, the LMF should decide whether to continue the positioning service if the UE moves out of the validity area. For example, if the UE moves to a cell out of the target area, e.g. a warehouse/factory/assembly, there may be no need to continue the positioning service or configure the SRS validity area; only the LMF is aware about this!

Observation 4: If the new validity area is expected to be configured, LMF should involve in the update of SRS configuration with validity area.

Observation 5. If UE moves out of a validity area, LMF should decide on the service continuity.

Therefore, a new procedure for SRS update is needed for LPHAP, in which the NG-RAN should send the SRS configuration update request to the LMF. Subsequently, the LMF can initiate another Positioning Information Exchange procedure to configure the UE with new SRS and validity area. Instead of introducing a new message for the request, the existing Positioning Information Update message can be reused.

There are two options: either the receiving gNB or the last serving gNB can the SRS configuration update request to the LMF. we slightly prefer the second option that the last serving gNB to send the request, because the receiving gNB can only send the Positioning Information Update message after anchor relocation and path switch, which has a larger latency. Besides, in the case of without anchor relocation, the serving gNB cannot send the Positioning Information Update message to the LMF which is UE-associated.

Proposal 9. The last serving gNB sends Positioning Information Update message to the LMF for SRS configuration update.

For both options, the cell ID should be carried in the Positioning Information Update message which can help LMF to configure the validity area. Therefore, we proposed the following:

Proposal 10. Cell ID is carried in Positioning Information Update message for SRS configuration update.

Above all, a new SRS configuration update procedure should be introduced for LPHAP and it is proposed to adopt the following as the baseline.

Proposal 11. Adopt the following as the baseline procedure for SRS configuration update procedure.

Figure 4. SRS Configuration Update procedure

## 2.3	Pre-configured SRS (WI task)

According to the WID, the pre-configuration of one or multiple SRS for positioning configurations should be discussed. From our understanding, if a UE is pre-configured with SRS e.g. before a positioning session, the UE can send the activation request to the receiving gNB when an event is detected, and the receiving gNB can directly activate the SRS by only updating the TA information without LMF involvement, which reduces latency and saves signaling overhead.

We noted at this stage that any change of the SRS Configuration will need to re-establish the Validity Area over gNB(s). Therefore, the SRS configuration should also be reserved by the gNBs in the validity area upon pre-configuration. Otherwise, the activation request should be always forwarded to the LMF to check with gNBs if the pre-configured SRS is still available and then pre-configuration brings no benefit.

Observation 6. The SRS configuration should also be reserved by the gNBs in the validity area upon pre-configuration. Otherwise, the activation request should be always forwarded to the LMF to check with gNBs if the pre-configured SRS is still available and then pre-configuration brings no benefit.

During the last RAN2 meeting, the following agreement was made for pre-configured SRS [9]:

RAN2 do not further consider providing pre-configured SRS via system information in Rel-18.

For the activation indication and/or request for preconfigured SRSs, RRCResumeRequest message is used, and 1-bit indication in the RRCResumeRequest is introduced for this use.

WA: The resume cause introduced for the SRS configuration request can be reused for the activation indication of the pre-configuration SRS.

The RAN2 WA is that RRCResumeRequest is used for activation, and the resume cause is the same with SRS configuration request. Therefore, in order to activate SRS transmission in the UE, the receiving gNB should be aware of the pre-configuration as well as the pre-configured SRS of the target UE.

Observation 7: The receiving gNB should be aware of the pre-configured SRS resources when receiving the activation request from the UE.

To enable it, the reservation for pre-configured SRS can be bind with the target UE. However, as discussed in section 2.2, non-UE associated NRPPa message should be used for SRS reservation. A simple solution is to carry an association identifier in this message for the reservation for pre-configured SRS of the target UE. For example, I-RNTI can be can candidate for the associate identifier because it is always included in RRCResumeRequest message.

Proposal 12: A association identifier, e.g. I-RNTI, can be carried in the non-UE associated NRPPa message for the reservation for pre-configured SRS of the target UE.

Pre-configuration of multiple SRS means multiple instances of SRS configuration can be pre-configured to the UE, each configuration is associated with a validity area. If the UE moves out of the current validity area, it can request to activate another configuration. Based on the analysis, the SRS configurations should be reserved upon pre-configuration, which may result in unnecessary capacity loss if the number of instances is large. Therefore, it is proposed that the pre-configuration of multiple SRS is deprioritized in Rel-18.

Proposal 13. Pre-configured multiple SRS is deprioritized in R18 from RAN3 perspective. LS RAN2 on RAN3’s concern.

## 2.4 SRS/PRS periodicity

The following agreements was made by RAN1 [10]:

Agreement From RAN1 perspective, candidate values larger than 10240 ms for PRS and/or SRS periodicity, e.g., 20480 ms, can be introduced. - FFS: specification impact on PRS/SRS configuration. - Send LS to RAN2 asking them to work on the higher layer signalling details (e.g., specific values of periodicity, hyper SFN information in the configuration, etc.)

RAN1 thinks the periodicity larger than 10240 ms for PRS and/or SRS can be introduced for LPHAP. From our understanding, the positioning interval for the studied use case of LPHAP is 15-30s, therefore extended PRS/SRS periodicities 20480ms can be introduced. To avoid duplicate discussion, RAN3 can wait RAN2 to decide on the specific values.

Proposal 14. An extended value of PRS/SRS periodicity, e.g., 20480ms, can be introduced. Wait RAN2 to decide on the specific values.

# 3	Conclusion

This document proposes:

- On SRS Configuration Procedure for LPHAP:

Proposal 1: LMF provides a list of cells as positioning validity area for SRS to the serving gNB, and the cells can be defined by a list of PCIs.

Observation 1. Serving gNB cannot decide on the cross-cell configuration only based on the legacy Requested SRS Transmission Characteristics IE, because the determined SRS configuration may have been occupied in other cells of the expected validity area.

Observation 2. The option that LMF sends area-specific SRS configuration to the serving gNB is aligned with RAN2 agreements.

Proposal 2. LMF can send SRS configuration(s) to the serving gNB as recommendation, and the serving gNB replies with the finally determined SRS configuration.

Proposal 3. LMF recommends the full SRS configurations or only a list of parameters, e.g. the parameter list provided by the RAN1, to the serving gNB.

Observation 3: The LMF can determine the recommended area-specific SRS configuration via NRPPa signalling or via OAM.

Proposal 4. The LMF determines the recommended area-specific SRS configurations by implementation at the current stage.

Proposal 5. The serving gNB should not be able to modify the validly area cell list in this release.

Proposal 6. When a UE is configured with SRS configurations associated with a validity area, the LMF notify the gNBs in this area to reserve/release the allocated SRS configurations to avoid interference.

Proposal 7. Define a new non-UE associated NRPPa procedure for SRS resource reservation in the positioning validity area.

Proposal 8. Adopt figure 3 as the baseline procedure for SRS configuration:

- SRS Configuration update when the UE leaves the validity area

Observation 4: If the new validity area is expected to be configured, LMF should involve in the update of SRS configuration with validity area.

Observation 5. If UE moves out of a validity area, LMF should decide on the service continuity.

Proposal 9. The last serving gNB sends Positioning Information Update message to the LMF for SRS configuration update.

Proposal 10. Cell ID is carried in Positioning Information Update message for SRS configuration update.

Proposal 11. Adopt figure 4 as the baseline procedure for SRS configuration update procedure.

- Pre-configured SRS (WI task)

Proposal 12: A association identifier, e.g. I-RNTI, can be carried in the non-UE associated NRPPa message for the reservation for pre-configured SRS of the target UE.

Proposal 13. Pre-configured multiple SRS is deprioritized in R18 from RAN3 perspective. LS RAN2 on RAN3’s concern.

- SRS/PRS periodicity

Proposal 14. An extended value of PRS/SRS periodicity, e.g., 20480ms, can be introduced. Wait RAN2 to decide on the specific values.

The associated stage 3 Text Proposal on SRS Configuration Procedure for LPHAP are propose in Annex for information and can be re-use as needed.

# 4	References

- RAN3-120 Chair’s Notes

- R3-235006,  Reply LS on LPHAP

- RAN2-122 Chair’s Notes

- R3-226165, S2-2209591, LS on LPHAP information delivery to RAN

- R3-230008, R1-2212725, Reply LS on LPHAP information delivery to RAN

- R3-230021, R2-2213327, Reply LS on LPHAP information delivery to RAN

- RAN1-112 Chair’s Notes

- RAN1-113 Chair’s Notes

- RAN2-123 Chair’s Notes

- R3-235007, R1-2308571, LS on the longer PRS/SRS periodicity for LPHAP

# Annex A. TP for 38.455 17.5.0

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-    SRS Information Transfer. This function allows the LMF to exchange SRS related information with the NG-RAN node for the purpose of positioning with validity area mechanism.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| SRS Information Transfer | Positioning Resource Reservation |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

# 8	NRPPa procedures

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and CSlass 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| Positioning Resource Reservation | POSITIONING RESOURCE RESERVATION |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

## 8.2	Location Information Transfer Procedures

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.6	Positioning Information Exchange

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the NG-RAN node shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If the UE TEG Information Request IE is included in the POSITIONING INFORMATION REQUEST message and set to "onDemand", the NG-RAN node shall, if supported, provide the UE Tx TEG association in the POSITIONING INFORMATION RESPONSE message.

If the UE TEG Information Request IE is set to "periodic", the NG-RAN node shall, if supported, reply with the POSITIONING INFORMATION RESPONSE message without including any UE Tx TEG association in this message. The NG-RAN node shall then take the UE TEG Reporting Periodicity IE into account when configuring the UE’s periodic UE Tx TEG association reporting and initiate the Positioning Information Update procedure for reporting the UE Tx TEG association received from the UE, if any.

If Positioning Validity Area Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node shall, if supported, take this information into account for configuring SRS transmissions for the UE in the configured validty area, and it shall, if supported include SRS Configuration IE and the SFN initialisation time IE in the POSITIONING INFORMATION RESPONSE message.

#### 8.2.6.3	Unsuccessful Operation

Figure 8.2.6.3-1: Positioning Information Exchange procedure, unsuccessful operation

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message and the NG-RAN node is unable to configure any SRS transmissions for the UE, it shall respond with a POSITIONING INFORMATION FAILURE message. If a handover of the target UE has been triggered, the NG-RAN node shall send a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

If the NG-RAN node is unable to provide any of the requested information, the NG-RAN node shall respond with a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

#### 8.2.6.4	Abnormal Conditions

Void.

### 8.2.7	Positioning Information Update

#### 8.2.7.1	General

The Positioning Information Update procedure is initiated by the NG-RAN node to indicate to the LMF that a change has occurred in the SRS configuration or in the UE Tx TEG association. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: Positioning Information Update procedure, successful operation

The NG-RAN node initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the LMF. If the SRS Configuration IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the updated SRS Configuration for the UE. If the SFN Initialisation Time IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the SFN Initialisation Time associated to the SRS Configuration.

If the UE Tx TEG Association List IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider it as the UE Tx TEG association for the SRS resources that have changed their TEG association during the latest reporting interval.

If the SRS Transmission Status IE is included in the POSITIONING INFORMATION UPDATE message and set to "stopped", the LMF shall consider that the SRS transmission has stopped.

If the Cell ID IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider it as the update request for SRS with validity area and initiates another Positioning Information Exchange procedure to configure SRS transmissions for the UE in a new validty area.

#### 8.2.7.3	Unsuccessful Operation

Not Applicable.

#### 8.2.7.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.x	Positioning Resource Reservation

#### 8.2.x.1	General

The purpose of the Positioning Resrouce Reservation procedure is to allow the LMF to request the NG-RAN node in the positioning validity area to reserve or release SRS resources.

#### 8.2.x.2	Successful Operation

Figure 8.2.x.2-1: Positioning Resource Reservation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING RESOURCE RESERVATION message to the NG-RAN node.

If SRS Reservation Request IE is set to “reserve”, the NG-RAN node should reserve the SRS configurations for LPHAP. If SRS Reservation Request IE is set to “release”, the NG-RAN node should release the previous reservation.

#### 8.2.x.3	Unsuccessful Operation

Not Applicable.

#### 8.2.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

# 9	Elements for NRPPa Communication

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Location Information Transfer Procedures

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.70 |  | YES | ignore |
| UE TEG Information Request | O |  | ENUMERATED(onDemand, periodic, stop, …) |  | YES | ignore |
| UE TEG Reporting Periodicity | C-ifUeTegInfoReqPeriodic |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | YES | reject |
| Positioning Validity Area Information | O |  | 9.2.z2 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifUeTegInfoReqPeriodic | This IE shall be present if the UE TEG Information Request IE is set to the value "periodic". |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.13	POSITIONING INFORMATION UPDATE

This message is sent by the NG-RAN node to indicate that a change in the SRS configuration or UE Tx TEG association has occurred.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| UE Tx TEG Association List | O |  | 9.2.78 |  | YES | ignore |
| SRS Transmission Status | O |  | ENUMERATED (stopped, ...) |  | YES | ignore |
| Cell ID | O |  |  |  | YES | ignore |
| >NR PCI | O |  | INTEGER (0..1007) |  |  |  |
| >NR CGI | O |  | 9.2.9 |  |  |  |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.y1	POSITIONING RESOURCE RESERVATION

This message is sent by the LMF to request the NG-RAN node to reserve or release SRS resources.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Reservation Request | M |  | ENUMERATED(reserve, release, …) |  | YES | ignore |
| SRS Configuration | M |  | 9.2.28 |  | YES | ignore |
| Validity Area Cells | M |  | 9.2.z3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

## 9.2	Information Element definitions

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.2.z1 SRS Configuration List

This IE is used to indicate a list of SRS configurations.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS Configuration List |  | 1 |  |  |
| > SRS Configuration Item |  | 1 .. <maxnoSRSConfig> |  |  |
| >>SRS Configuration | M |  | 9.2.28 |  |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.2.z2  Positioning Validity Area Information

This IE is used to indicate the validity area and the available SRS configurations in the area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Validity Area Cells | M |  | 9.2.z3 |  |
| Available SRS Configuration | M |  | 9.2.z1 |  |

### 9.2.z3  Positioning Validity Area Information Cells

This IE is used to indicate the cells belong to the validity area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning Validity Area Cell List |  | 1 |  |  |
| >Positioning Validity Area Cell Item |  | 1 .. <maxnoPACell> |  |  |
| >>NR CGI FFS | O |  | 9.2.6 |  |
| >>NR PCI | O |  | INTEGER (0..1007) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoPACell | FFS |

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

# Annex B. TP for 38.473 V17.5.0

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |
| Positioning Resource Reservation | POSITIONING RESOURCE RESERVATION |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

## 8.13	Positioning Procedures

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.13.9	Positioning Information Exchange

#### 8.13.9.1	General

The Positioning Information Exchange procedure is initiated by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals and to retrieve the SRS configuration from the gNB-DU. The procedure uses UE-associated signalling.

#### 8.13.9.2	Successful Operation

Figure 8.13.9.2-1: Positioning Information Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the gNB-DU.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message. If the SRS Positioning INACTIVE Query Indication IE is also included in the POSITIONING INFORMATION REQUEST message and set to ‘true’, the gNB-DU shall, if supported, include the SRS-PosRRC-InactiveConfig IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the gNB-DU shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If Positioning Validity Area Information IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU, if supported, shall take this information into account for configuring SRS transmissions for the UE in the configured validty area, and it shall, if supported,  include SRS Configuration IE and the SFN initialisation time IE in the POSITIONING INFORMATION RESPONSE message.

Interaction with the UE Context Modification Required (gNB-DU initiated) procedure:

The UE Context Modification Required (gNB-DU initiated) procedure may be performed before the POSITIONING INFORMATION RESPONSE message.

#### 8.13.9.3	Unsuccessful Operation

Figure 8.13.9.3-1: Positioning Information Exchange procedure, unsuccessful operation

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message and the gNB-DU is unable to configure any SRS transmissions for the UE, the gNB-DU shall respond with a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

If the gNB-DU is unable to provide any of the requested information, the gNB-DU shall respond with a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.13.16	Positioning Information Update

#### 8.13.16.1	General

The Positioning Information Update procedure is initiated by the gNB-DU to indicate to the gNB-CU that a change has occurred in the SRS configuration. The procedure uses UE-associated signalling.

#### 8.13.16.2	Successful Operation

Figure 8.13.16.2-1: Positioning Information Update procedure, successful operation

The gNB-DU initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the gNB-CU.

If the SRS Configuration IE is included in the POSITIONING INFORMATION UPDATE message, the gNB-CU shall consider this information as the updated SRS Configuration for the UE. If the SFN Initialisation Time IE is included in the POSITIONING INFORMATION UPDATE message, the gNB-CU shall consider this information as the SFN Initialisation Time associated to the SRS Configuration.

If the Cell ID IE is included in the POSITIONING INFORMATION UPDATE message, the gNB-CU shall consider it as the update request for SRS with validity area and initiates another Positioning Information Exchange procedure to configure SRS transmissions for the UE in a new validty area.

#### 8.13.16.3	Unsuccessful Operation

Not Applicable.

#### 8.13.16.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.13.a	Positioning Resource Reservation

#### 8.13.a.1	General

The purpose of the Positioning Resrouce Reservation procedure is to allow the gNB-CU to request the gNB-DU to reserve or release SRS resources.

#### 8.13.a.2	Successful Operation

Figure 8.13.a.2-1: Positioning Resource Reservation procedure, successful operation

The gNB-CU initiates the procedure by sending a POSITIONING RESOURCE RESERVATION message to the gNB-DU.

If SRS Reservation Request IE is set to “reserve”, the gNB-DU should reserve the SRS configurations for LPHAP. If SRS Reservation Request IE is set to “release”, the gNB-DU should release the previous reservation.

#### 8.13.a.3	Unsuccessful Operation

Not Applicable.

#### 8.13.a.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

# 9	Elements for F1AP Communication

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

## 9.2	Message Functional Definition and Content

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.12.13	POSITIONING INFORMATION REQUEST

This message is sent by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals for uplink positioning measurement and also to retrieve the SRS configuration from the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Requested SRS Transmission Characteristics | O |  | 9.3.1.175 |  | YES | ignore |
| UE Reporting Information | O |  | 9.3.1.255 |  | YES | ignore |
| SRS Positioning INACTIVE Query Indication | O |  | ENUMERATED (true, ...) | Applicable only if the Requested SRS Transmission Characteristics IE is present | YES | ignore |
| Positioning Validity Area Information | O |  | 9.2.c2 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.12.26	POSITIONING INFORMATION UPDATE

This message is sent by the gNB-DU to indicate that a change in the SRS configuration has occurred.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRS configuration | O |  | 9.3.1.192 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 |  | YES | ignore |
| Cell ID | O |  |  |  | YES | ignore |
| >NR PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >NR CGI (FFS) | O |  | 9.3.1.12 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.12.b1	POSITIONING RESOURCE RESERVATION

This message is sent by a gNB-CU to request a gNB-DU to reserve or release SRS resources.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| SRS Reservation Request | M |  | ENUMERATED(reserve, release, …) |  | YES | ignore |
| SRS Configuration | M |  | 9.3.1.129 |  | YES | ignore |
| Validity Area Cells | M |  | 9.3.1.c3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

## 9.3	Information Element Definitions

#### 9.3.1.c1 SRS Configuration List

This IE is used to indicate a list of SRS configurations.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS Configuration List |  | 1 |  |  |
| > SRS Configuration Item |  | 1 .. <maxnoSRSConfig> |  |  |
| >>SRS Configuration | M |  | 9.3.1.129 |  |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.c2  Positioning Validity Area Information

This IE is used to indicate the validity area and the available SRS configurations in the area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Validity Area Cells | M |  | 9.3.1.c3 |  |
| Available SRS Configuration | M |  | 9.3.1.c1 |  |

#### 9.3.1.c3  Positioning Validity Area Information Cells

This IE is used to indicate the cells belong to the validity area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning Validity Area Cell List |  | 1 |  |  |
| >Positioning Validity Area Cell Item |  | 1 .. <maxnoPACell> |  |  |
| >>NG CGI | O |  | 9.3.1.12 |  |
| >>NG PCI | O |  | INTEGER (0..1007) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoPACell | FFS |

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

# Annex C. TP for 38.470 V17.5.0

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

### 5.2.11	Positioning function

This function allows to transfer location management messages between gNB-CU and gNB-DU. With this function, gNB-CU request TRP information from gNB-DU, and gNB-DU response to gNB-CU with the TRP information. With this function, gNB-CU request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements.

The function allows the gNB-CU to

-	transfer the positioning assistance data to gNB-DU. The gNB-DU is responsible for broadcasting the positioning assistance data according to the scheduling parameters available.

-	request the gNB-DU to configure SRS transmissions for UE.

-	request the gNB-DU to configure PRS transmissions.

-	request the gNB-DU to configure measurement gap or PRS processing window.

-	request the gNB-DU to activate or deactivate preconfigured measurement gap or PRS processing window.

-	request the gNB-DU to broadcast positioning system information.

-    request the gNB-DU to reserve or release SRS configurations.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

# Annex D. TP for 38.413 V17.5.0

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

## 8.10	NRPPa Transport Procedures

### 8.10.1	General

The purpose of the NRPPa Transport procedures is to carry NRPPa signalling (defined in TS 38.455 [19]) between the NG-RAN node and the LMF over the NG interface.

The Downlink UE Associated NRPPa Transport procedure and the Uplink UE Associated NRPPa Transport procedure use UE-associated signalling. The UE-associated signalling is used to support E-CID Location Information Transfer, Positioning Information Transfer, Measurement Preconfiguration Information Transfer, and Reporting of General Error Situations due to reception of an NRPPa message that utilized UE-associated signalling.

The Downlink Non UE Associated NRPPa Transport procedure and the Uplink Non UE Associated NRPPa Transport procedure use non-UE associated signalling. The non-UE associated signalling is used to support OTDOA Information Transfer, Assistance Information Transfer, TRP Information Transfer, Measurement Information Transfer, PRS Information Transfer, SRS Information Transfer and Reporting of General Error Situations due to reception of an NRPPa message that utilized non-UE associated signalling.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>