3GPP TSG-RAN WG3 #121bis										R3-235513

9th – 13th Oct 2023

Xiamen, China

Agenda item:	13.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on incoming LS

Document for:	Discussion and Decision

# Introduction

This contribution discusses the LS received in RAN3.

# LS on UE RACH-less handover for mobile IAB

This LS ([1]) states:

RAN2 has discussed the UE RACH-less handover in mobile IAB and achieved the following agreements:

- RAN2#121bis meeting agreements:

Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB).

RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration:

- Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details.

- Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command).

RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO.

- RAN2#122 meeting agreements:

RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured).

For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report).

for the UL grant and HO completion in RACH-less HO:

- 1. Both type-1 configured grant and dynamic grant are supported

- 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI).

Send LS to RAN3 to check whether there are issues / feasibility concerns

RAN2 would like to ask RAN3 to take those agreements into account and provide feedbacks if there are any issues or feasibility concerns.

The main technical issue discussed in RAN2 has been the beam indication, whether a “blind” beam selection can be done or should the beam selection be based on UE measurements. “Blind” selection requires that there is a clear one-to-one mapping between source DU and target DU beams so that target beam would correspond to the beam the UE used in the source cell. However, it is to be confirmed if such one-to-one mapping can be done considering the earlier agreement to use different radio resource by the two logical DUs. Additionally, the mapping should also be known by the target CU to be able to indicate that in the HO command which would require enhancements to current specifications, obviously for both F1AP and XnAP.

If the beam selection is relying on UE measurements, it would require additional signalling to configure UEs to measure and report the new cell of the second logical DU. This will slow down the procedure and would require also appropriate measurement gaps to be specified for UEs in order to be able to measure the target cell on different radio resources. This would hence reduce the potential benefits of the RACH-less HO making it less attractive as part of DU migration.

As stated in the RAN2 agreements, “R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report)”. However, considering the above, this may need still confirmation.

In general, it can be noted that RAN2 is still discussing the benefits and need to specify mIAB specific RACH-less operation. This is indicated in the agreements by stating “RACH-less, if supported”. Furthermore, NTN WI is specifying RACH-less HO which could be applicable for mobile IAB and could become an implementation option. Hence, RAN3 could still wait for RAN2 final conclusions on mIAB RACH-less whether it will be specified for Rel.18.

The other open issues, like handling of supervision timer, are more within the RAN2 scope and can wait for RAN2 progress.

Observation 1: The discussions and agreements on RACH-less HO for mobile IAB are still pending in RAN2

Proposal 1: For LS (R3-233713), RAN3 wait for further progress in RAN2, and no need to reply in this meeting.

# LS on CAG solution for mobile IAB

This LS ([2]) states

RAN2 has discussed the solution agreed by SA2 on UE access to MBSR using CAG function.

RAN2 kindly asks SA2 to provide more details using CAG for mIAB, in order to determine AS impacts, if any.

RAN3 need to wait for SA2 on the detail using CAG for mIAB. It seems there is no impact to RAN3 at current stage. The LS can be noted.

Proposal 2: NOTE the LS R3-233717.

# LS On IAB-node De-authorization handling

This LS ([3]) states

SA2 is currently discussing the MBSR (i.e., mobile IAB-node) authorization state changed from authorized to non-authorized (e.g., due to subscription data update for the MBSR). The AMF notifies the Mobile IAB-node authorization status via the N2 message. SA2 noticed that when the AMF indicates the mobile IAB node is authorized, the mobile IAB-node integrates into the network. The donor-gNB communicates with IAB-MT via RRC messages to setup the backhaul, F1-C connection (i.e., as specified in TS 38.401 clause 8.12).

However, when an MBSR is no longer authorized as indicated to the gNB by AMF by N2 message, it’s not clear what is the gNB behaviour.

SA2 would like to check whether this is a correct understanding:

- The gNB attempts to perform handover of all the UEs served by the MBSR

- When the step 1 completes, the gNB releases the F1-C

Q1: is the understanding above correct?

Q2: Whether and how is the Mobile IAB-DU indicated to refrain from setting up a F1-C connection to the donor CU immediately after the F1-C release (is there an indication in the F1-C release instructing the DU to not try again)?

When the MBSR is no long authorized, the IAB-donor need to take action to ensure the IAB node will not serve any UE. One of the action is the IAB-donor initiate the handover for the connected UEs to another cell not belonging to the affected MBSR.

The MBSR-MT’s IAB-donor can release the BH RLC CH, release the previously assigned TNL address, etc. Without the BH RLC CH or TNL address, the IAB-DU cannot initiate the F1-Connection setup immediately. So there is no need to indicate the IAB-DU to refrain from setting up a F1-C connection to the donor-CU immediately.

So the answer to SA2 questions are

- For Q1, It is up to the gNB implementation to perform handover of all the UEs served by the MBSR. It is correct that gNB can release F1-C after the handover of connected UEs are completed.

- For Q2, after the MBSR authorization is changed to “not authorized”, the IAB-donor can release the BH RLC CH, and the previously assigned TNL address. Without BH RLC CH or TNL address, the IAB-DU cannot initiate F1 Setup.

Proposal 3-1: For LS (R3-233730), Reply to SA2 with following answers:

- For Q1, It is up to the gNB implementation to perform handover of all the UEs served by the MBSR. It is correct that gNB can release F1-C after the handover of connected UEs are completed.

- For Q2, after the MBSR authorization is changed to “not authorized”, the IAB-donor can release the BH RLC CH, and the previously assigned TNL address. Without BH RLC CH or TNL address, the IAB-DU cannot initiate F1 Setup.

When the MBSR-MT’s RRC and MBSR-DU’s F1 are terminated at the different IAB-donor, it is the MBSR-MT’s IAB-donor receive the “MBSR not authorized” indication from the AMF. This brings a question on how MBSR-DU’s IAB-donor know the MBSR authorization is changed to “not authorized”, so it can initiate the handover for the connected UEs.

Possible Options:

- Option 1: MBSR-MT’s IAB-donor inform MBSR-DU’s IAB-donor for the MBSR authorized information via a new XnAP MBSR Authorized Information (e.g. IAB Transport Migration Modification procedure)

Upon the reception of the “MBSR not authorized” indication, the MBSR-DU’s IAB-donor can initiate the handover for the connected UEs. This requires a new XnAP IE.

- Option 2: MBSR-MT inform the co-located MBSR-DU, and the MBSR-DU inform its IAB-donor, so the MBSR-DU’s IAB-donor initiate the handover for the connected UEs.

This option should be avoided, since the MBSR may be not trustable. The IAB-donor should take an action based on the information received from a trust node (e.g. AMF, or another IAB-donor).

- Option 3: MBSR-MT’s IAB-donor initiate the IAB Transport Migration Modification procedure to inform MBSR-DU’s IAB-donor to release all traffic/TNL address.

Unlike Option 1 requiring a new XnAP MBSR Authorized Information, the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message does not require changes. The REQUEST message can include the Traffic To Be Released Information IE (e.g. the All Traffic Indication IE in the Traffic to Be Released Information IE is set to “true”) and the IAB TNL Address To Be Released IE, so the MBSR-DU’s IAB-donor can know there will be no BH RLC CH available. Then the MBSR-DU’s IAB-donor can initiate the handover for the connected UEs.

Option 3 is preferred.

Proposal 3-2: when the MBSR-MT’s IAB-donor is different to the MBSR-DU’s IAB-donor, the MBSR-MT’s IAB-donor can initiate the IAB Transport Migration Modification procedure to inform MBSR-DU’s IAB-donor for releasing all traffic/TNL address.

# Conclusions

For the multiple LSs received in RAN3, our proposals are:

Observation 1: The discussions and agreements on RACH-less HO for mobile IAB are still pending in RAN2

Proposal 1: For LS (R3-233713), RAN3 wait for further progress in RAN2, and no need to reply in this meeting.

Proposal 2: NOTE the LS R3-233717.

Proposal 3-1: For LS (R3-233730), Reply to SA2 with following answers:

- For Q1, It is up to the gNB implementation to perform handover of all the UEs served by the MBSR. It is correct that gNB can release F1-C after the handover of connected UEs are completed.

- For Q2, after the MBSR authorization is changed to “not authorized”, the IAB-donor can release the BH RLC CH, and the previously assigned TNL address. Without BH RLC CH or TNL address, the IAB-DU cannot initiate F1 Setup.

Proposal 3-2: when the MBSR-MT’s IAB-donor is different to the MBSR-DU’s IAB-donor, the MBSR-MT’s IAB-donor can initiate the IAB Transport Migration Modification procedure to inform MBSR-DU’s IAB-donor for releasing all traffic/TNL address.

# References

- R3-233713	LS on UE RACH-less handover for mobile IAB	RAN2

- R3-233717	LS on CAG solution for mobile IAB	RAN2

- R3-233730	LS On IAB-node De-authorization handling	SA2