3GPP TSG-RAN WG3 #121bis						               									R3-235515

9th – 13th Oct 2023

Xiamen, China

Agenda item:	16.4

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS38.401 BL CR) discussion on the support for multi-path

Document for:	Discussion and Decision

# 1	Introduction

In last meeting, RAN3 agreed to further discussion following issues:

Check with SA2 (possible via an LS) to get feedback related to the handling of the UE location information in the multi-path operation.

Discuss which node to decide the DL RRC messages sent over split SRB is transmitted over the direct path or the indirect path.

Whether to reuse the existing SpCell ID for path addition in the F1AP signaling

Setup the tunnels for different paths, focus on stage-3 details

How to support and capture Scenario 2 in the BL CRs, e.g., stage-3 details

Continue working on path addition/release

This contribution analyses the above issues.

# 2	Discussion

## 2.1 		User Location Information

In DC, the gNB can report the PSCell via the NGAP User Location Inform IE.

Current CN is unaware of the indirect path. The direct path and indirect path may use different cell from same or different gNB-DU. It may be necessary for the CN know the serving cell for both direct path and the serving cell for indirect path. This needs to be confirmed by SA2. It is suggested to send a LS to SA2 asking whether need to report both the serving cell ID for direct path, and the serving cell ID for indirect path to CN.

Proposal 1: RAN3 send a LS to SA2 asking whether need to report both the serving cell ID for direct path, and the serving cell ID for indirect path to CN.

In the SA2 confirm the need, ULI IE may need to be updated, since current ULI IE can only report PSCell, but it is not applicable to multi-path.

## 2.2 		RB duplication

From the perspective of the gNB-DU (i.e. the gNB-DU for the direct path, or the gNB-DU for the indirect path), the required behavior is similar to Rel-17, e.g. setting the 2nd path via F1AP UE Context Setup procedure.

Previous meeting agreed turn the following WA into agreement:

WA: For inter-DU case, legacy DC based data split/duplication mechanism can be reused as baseline for split DRB/SRB.

The gNB-CU can request the 2nd gNB-DU to setup the 2nd path and use it for split DRB. Separate F1-U tunnels terminated at different gNB-DU are used for direct path and indirect path. Similarl to DC, the gNB-CU is responsible for data split/duplication. For SRB, separate F1-C connection is established between the gNB-CU and the gNB-DU for direct path, and between the gNB-CU and the other gNB-DU for indirect path. This is also similar  to DC. It may require some small modification to the specification, since current F1AP text only describes the DC based duplication or CA based duplication. But this can be handled later during the Stage-3 CR discussion.

Proposal 2-1: Enhance F1AP to capture the RB duplication in multi-path.

Case 2: direct path and indirect path use same gNB-DU.

Last RAN3 meeting agreed the F1AP TP to support the multi-path.

#### 9.3.1.x1	Path Addition Information

This IE provides information for path addition.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Path Addition Information | M |  |  |  |
| >Indirect Path Addition |  |  |  |  |
| >>Target Relay UE ID | M |  | BIT STRING (SIZE(24)) | Corresponds to information provided in the targetRelayUE-Identity contained in the SL-PathSwitchConfig IE, defined in TS 38.331 [8] |
| >>Remote UE Local ID | M |  | 9.3.1.267 |  |
| >>T420-like (FFS on the name) | M |  | FFS | FFS |
| >Direct Path Addition |  |  |  |  |
| >>PCell ID | M |  | NR CGI 9.3.1.12 |  |

Last RAN3 meeting did not discuss how to identify a path to be modified/released.

To modify/release only one path, gNB-CU initiates UE Context Modification procedure towards the gNB-DU. When same DRB ID is used for direct path and indirect path, the F1AP UE CONTEXT MODIFICATION REQUEST message needs to indicate the information on modification/removal of a path. For example,

+ when need to modify one path but not affect the other path, gNB-CU need to inform gNB-DU on which path is to be modified.

+ when need to remove one path but keep the other path, the gNB-CU initiate UE Context Modification procedure. gNB-CU need to inform gNB-DU on which path is to be removed. In case gNB-CU release both paths, the gNB-CU initiate the UE Context Release procedure.

In case only release indirect path, current F1AP UE Context Modification procedure can be used to release the PC5 RLC channel, thus release the indirect path.

In case only release direct path, it can be similar to direct path addition to indirect path. The gNB initiates the procedure to change the PCell from the indirect path to direct path (in case of addition), and change the PCell from direct path to indirect path (in case of removal). There is no enhancement needed.

Proposal 2-2: Legacy procedure can be used to release one path (i.e. direct path or indirect path) but keep another path (i.e. indirect path or direct path).

## Split SRB/DRB via multi-path

For inter-DU case, separate gNB-DU is used for direct path and indirect path. The indirect path can be setup via Rel-17 signaling. DC based mechanism can be reused. There is no Stage-3 impact to RAN3.

For intra-DU case,

- For DRB, RAN3 agreed “For intra-DU case, two F1-U tunnels are setup between CU and DU for a split DRB. ” The gNB-CU decide whether send DL data via one F1-U tunnel, or via two F1-U tunnels. For UL, the UE decide whether direct path or indirect path or both will be used based on the configuration. No Stage-3 impact to RAN3.

- For SRB, the gNB-CU send the RRC message to the gNB-DU, and gNB-DU need to decide how to send the RRC message to the UE.  There are two options

+ Option 1: gNB-CU indicate whether the RRC message is sent via direct path, or indirect path, or duplication over both paths (via existing Execute Duplication IE). The gNB-DU does not need to know which path is Primary path.

This option aligns with DRB that the gNB-CU explicitly indicate the path to be used for DRB/SRB. In case future release introduces multiple indirect path, the gNB-CU may need to indicate the specific path, e.g. via the Path ID. This may be another reason why path ID is needed.

+ Option 2: gNB-DU know which path is primary path during the direct/indirect path setup. Later, when gNB-CU send a RRC message to gNB-DU, gNB-CU indicate whether duplication is needed. In case no duplication, gNB-DU only send the RRC message via primary path.

It is preferred to adopt Option 1 that gNB-CU indicate whether the RRC message is sent via direct path, or indirect path, or duplication over both paths.

Proposal 2-3: gNB-CU inform gNB-DU on how to send the RRC message via direct path, or indirect path, or duplication over both paths

## 2.3	Mode-1 in inter-DU multi-path

Last meeting agreed following WA

WA: RAN3 agrees to limit mode 1 resource allocation scheme for U2N remote UE under only in intra-DU scenario under multi-path relay.

Considering the complexity to support mode-1 in inter-DU case, it is preferred to not support mode-1 in inter-DU case.

Proposal 3-1: turn following WA into agreement

WA: RAN3 agrees to limit mode 1 resource allocation scheme for U2N remote UE under only in intra-DU scenario under multi-path relay.

It is necessary to capture this agreement in the specification. This may be similar to the sidelink configuration in MR-DC that “NR Sidelink Communication, V2X Sidelink Communication and NR Sidelink Discovery cannot be configured in MR-DC in this release.” Is captured in TS 37.340. In this case, it is better to capture it in TS 38.401 clauses related to inter-DU multi-path. For example, adding “Mode 1 resource configuration cannot be configured in inter-gNB-DU multi-path in this release.” in below TS38.401 clauses.

### 8.xx.1	Inter-DU direct path addition on top of indirect path

### 8.xx.2	Inter-DU indirect path addition on top of direct path

An example for the proposed TP can be found in Annex – TP for TS38.401 BL CR

Proposal 3-2: update TS38.401 BL CR to clarify mode 1 resource configuration cannot be configured in inter-gNB-DU multi-path.

# 3	Conclusion

In this contribution, we briefly analyzed the RAN3 impact to support multi-path. Our proposals are:

Proposal 1: RAN3 send a LS to SA2 asking whether need to report both the serving cell ID for direct path, and the serving cell ID for indirect path to CN.

Proposal 2-1: Enhance F1AP to capture the RB duplication in multi-path.

Proposal 2-2: Legacy procedure can be used to release one path (i.e. direct path or indirect path) but keep another path (i.e. indirect path or direct path).

Proposal 2-3: gNB-CU inform gNB-DU on how to send the RRC message via direct path, or indirect path, or duplication over both paths

Proposal 3-1: turn following WA into agreement

WA: RAN3 agrees to limit mode 1 resource allocation scheme for U2N remote UE under only in intra-DU scenario under multi-path relay.

Proposal 3-2: update TS38.401 BL CR to clarify mode 1 resource configuration cannot be configured in inter-gNB-DU multi-path.

# References

- R3-234579, Summary of offline for SL Relay Multi-path

# Annex – TP for TS38.401 BL CR

## 8.xx	Overall procedures for multi-path support

### 8.xx.1	Inter-DU direct path addition on top of indirect path

The signalling flow for inter-DU direct path addition is shown in Figure 8.xx.1-1.

Figure 8.xx.1-1: Signalling procedure of inter-DU direct path addition on top of indirect path

1.	The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the direct path to remote UE under a different gNB-DU (i.e., gNB-DU1).

3.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU1, which contains at least the direct path configuration.

4.	The gNB-DU1 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

5.	The gNB-CU sends an RRCReconfiguration message to the relay UE to update the indirect path configuration if necessary.

6.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for remote UE by including the RRCReconfiguration message to the gNB-DU2. The contents in the RRCReconfiguration message may include at least direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).

7.	The gNB-DU2 sends the RRCReconfiguration message to the remote UE.

8.	The gNB-DU2 sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

9.	The remote UE performs random access procedure at the gNB-DU1.

10.	The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU2.

Editor’s Note: FFS whether the remote UE1 sends the RRCReconfigurationComplete message to the gNB-CU via old path (indirect path) or new path (direct path).

11.	The gNB-DU2 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

Mode 1 resource configuration cannot be configured in inter-gNB-DU multi-path in this release.

### 8.xx.2	Inter-DU indirect path addition on top of direct path

The signalling flow for inter-DU indirect path addition is shown in Figure 8.xx.2-1.

Figure 8.xx.2-1 Signalling procedure of inter-DU indirect path addition on top of direct path

1. The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the indirect path via relay UE to remote UE under a different gNB-DU (i.e., gNB-DU2).

3.	The reconfiguration to relay UE is performed among relay UE, gNB-DU2 and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped.

4.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU2, which contains the indirect path configuration at least.

5.	The gNB-DU2 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU1. The contents in the RRCReconfiguration message may include at least indirect path addition configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s).

7.	gNB-DU1 sends the RRCReconfiguration message to the remote UE.

8.	The remote UE establishes PC5 connection with relay UE.

9.	The remote UE may complete the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU1.

Editor’s Note: FFS whether the remote UE sends the RRCReconfigurationComplete message to the gNB-CU via old path (direct path) or new path (indirect path).

Editor’s Note: FFS: In case the relay UE is in RRC_IDLE/ INACTIVE state, how to trigger the relay UE enter RRC_CONNECTED state, RAN3 wait for RAN2’s further progress.

10.	The gNB-DU1 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

Mode 1 resource configuration cannot be configured in inter-gNB-DU multi-path in this release.