3GPP TSG-RAN WG3 #121bis						               									R3-235522

9th – 13th Oct 2023

Xiamen, China

Agenda item:	25.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on ECN marking for NR XR

Document for:	Discussion and Decision

# 1	Introduction

This contribution analyses the RAN3 enhancements to support ECN marking and Congestion Monitoring.

# 2	Discussion

## 2.1	ECN marking in NG-RAN

TS23.501 defines:

In 5G System, ECN marking for L4S may be supported. ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows. ECN marking for the L4S in the IP header is supported in either the NG-RAN (see clause 5.37.3.2 and TS 38.300 [27]), or in the PSA UPF (see clause 5.37.3.3).

In the case of ECN marking for L4S by PSA UPF, the NG-RAN is instructed to perform congestion information monitoring.

…

In the case of inter NG-RAN UE mobility, if the ECN marking for L4S has been enabled on source NG-RAN, but the target NG-RAN does not support ECN marking for L4S, then the SMF may, if supported, enable ECN marking for L4S in PSA UPF as defined in clause 5.37.3.3.

TS23.502 defines:

For each QoS Flow, an ECN marking for L4S indicator to (R)AN in the case of ECN marking for L4S in RAN as described in clause 5.37.3 of TS 23.501 [2] or,

-	a QoS monitoring configuration for congestion information as described in clause 5.45.3 of TS 23.501 [2], in the case of ECN marking for L4S by PSA UPF as described in clause 5.37.3 of TS 23.501 [2] or QoS monitoring for congestion information as described in clause 5.45.3 of TS 23.501 [2].

….

14.	[Conditional] (R)AN to AMF: N2 Request Ack (List of PDU Sessions To Be Established with N2 SM information (AN Tunnel Info, List of accepted QoS Flows for the PDU Sessions whose UP connections are activated, List of rejected QoS Flows for the PDU Sessions whose UP connections are activated, established QoS Flows status (active/not active) for QoS monitoring configuration for congestion information, established QoS Flows status (active/not active) for ECN marking for L4S, List of PDU Sessions that failed to be established with the failure cause given in the N2 SM information element).

In case of ECN marking performed in NG-RAN, the following enhancements are needed:

- NGAP:

- SMF informs NG-RAN on the QoS flow(s) requiring ECN marking.

The ECN marking is per QoS Flow. The SMF can request the ECN marking via a new ECN Marking Request IE in the QoS Flow Level QoS Parameters IE.

Up to the deployment, the ECN marking can be performed in the NG-RAN node or in the UPF. In case the NG-RAN node does not support ECN marking, SMF can enable ECN marking in UPF. So SMF need to be aware whether the QoS Flow is established with ECN marking, during the QoS flow establishment procedure.

- During NG/Xn-HO, target NG-RAN node informs SMF whether ECN marking is supported in target NG-RAN.

The required NGAP change to inform SMF is similar to the indication related to PDU Set QoS parameters.

In a summary, following NGAP enhancements are needed:

- Add the ECN Marking Request in the NGAP QoS Flow Level QoS Parameters IE.

- For NG-RAN node to indicate whether the QoS Flow is established with ECN marking activated, RAN3 adopt similar method as the indication to indicate the QoS Flow is successfully established with PDU Set QoS handling.

- XnAP:

- During NG/Xn-HO, source NG-RAN informs target NG-RAN on the QoS flow(s) requiring ECN marking

- During Xn Retrieve UE Context procedure, source NG-RAN informs target NG-RAN of the QoS flow(s) requiring ECN marking

To support ECN marking in NG-RAN, RAN3 consider and study following enhancements to XnAP:

- Add the ECN Marking Request in the XnAP QoS Flow Level QoS Parameters IE.

- F1-C (F1AP)

The impact to F1 depends on which node performs the ECN marking.

For uplink:

When the ECN marking is performed in NG-RAN, there are two options on which node performs the ECN marking.

- Option 1: ECN marking is performed in gNB-CU-UP, based on the congestion info from gNB-DU.

- Option 2: ECN marking is performed in gNB-DU (in F1-U IP header or F1-U GTP-U header, then gNB-CU-UP copy it to the NG-U)

Option 1 has the benefit that the gNB-CU-UP has the better knowledge, e.g. based on the congestion information received from the gNB-DU via F1-U, and the buffer status in the gNB-CU-UP.

Option 2 may be complex since the gNB-DU need to convey the ECN marking to the gNB-CU-UP. There may be two further options. 1) gNB-DU add the ECN marking in the F1-U header, then the gNB-CU-UP performs the ECN marking. Or 2) gNB-DU add the ECN marking in the IP header of the F1-U packet, then the gNB-CU-UP copy it when gNB-CU-UP performs the ECN marking.

It may simpler that the gNB-DU convey the congestion information to gNB-CU-UP, then the gNB-CU-UP performs the ECN marking.  For the congestion information provided from gNB-DU to gNB-CU, it can be same as the one used in the network exposure of 5GS information.

For downlink:

It may be similar as uplink. The gNB-DU report the congestion information to gNB-CU-UP, then the gNB-CU-UP performs ECN marking in the PDU IP header of the DL packet.

To support ECN marking in NG-RAN, RAN3 consider and study following enhancements to F1AP:

- Add the Congestion Monitoring Request in the F1AP QoS Flow Level QoS Parameters IE.

- F1-U:

After a QoS flow is established with ECN marking, gNB-DU informs gNB-CU of the congestion information (i.e. a percentage of packets that NG-RAN uses for ECN marking for L4S), then the CU-UP makes the ECN marking.

To support ECN marking in NG-RAN, RAN3 consider and study following enhancements to F1-U:

- The gNB-DU report the per QoS Flow congestion indication to gNB-CU.

- E1AP

The gNB-CU-UP need to know whether it needs to perform the ECN marking, so it is necessary that the gNB-CU-CP informs the gNB-CU-UP when a QoS flow requiring ECN marking is to be established. One possible option is to add the ECN Marking Request in the E1AP QoS Flow Level QoS Parameters List IE.

To support ECN marking in NG-RAN, RAN3 consider and study following enhancements to E1AP:

- Add the ECN Marking Request in the E1AP QoS Flow Level QoS Parameters List IE.

- Add the feedback indication to inform gNB-CU-CP when a QoS flow is established with ECN marking in gNB-CU-UP.

Proposal 1-1: RAN3 consider and study following enhancements to NGAP, XnAP, F1AP and E1AP:

- Add the ECN Marking Request in the QoS Flow Level QoS Parameters IE.

- For NG-RAN node to indicate whether the QoS Flow is established with ECN marking request accepted, RAN3 adopt similar method as the indication to indicate the QoS Flow with PDU Set QoS parameters is accepted.

- Add the feedback indication to inform gNB-CU-CP when a QoS flow is established with ECN marking in gNB-CU-UP.

Proposal 1-2: gNB-DU report the per QoS Flow congestion indication to gNB-CU via F1-U, so the gNB-CU performs ECN marking.

## 2.2	ECN marking in UPF

In case of ECN marking performed in UPF, the following enhancements are needed:

- NGAP:

- SMF informs NG-RAN on the QoS flow requiring congestion information reporting.

- During NG/Xn-HO, NG-RAN informs SMF that congestion information reporting is not supported in target NG-RAN.

The required NGAP change is similar to the PDU Set QoS parameters.

- XnAP:

- During NG/Xn-HO, source NG-RAN informs target NG-RAN on the QoS flow(s) requiring congestion information reporting

- During Xn Retrieve UE Context procedure, source NG-RAN informs target NG-RAN on the QoS flow(s) requiring congestion information reporting.

- F1-C (F1AP)

- gNB-CU informs gNB-DU on the QoS Flow(s) requiring congestion information reporting.

- F1-U:

- gNB-DU informs gNB-CU on the congestion information (i.e. a percentage of packets to be ECN marked). GNB-CU then forwards the congestion information to the UPF in NG-U.

- E1AP

The gNB-CU-UP need to know whether it needs to provide the congestion indication to UPF, so it is necessary that the gNB-CU-CP informs the gNB-CU-UP when a QoS flow requiring Congestion Monitoring is to be established. One possible option is to add the Congestion Monitoring Marking Request in the E1AP QoS Flow Level QoS Parameters List IE.

- NG-U: The NG-RAN provide the UL and/or DL QoS Flow congestion information to the UPF (i.e., a percentage of congestion level for exposure or ECN marking by the UPF)

Proposal 2-1: To support ECN marking in UPF, RAN3 consider and study following enhancements to NGAP, XnAP, F1AP, and E1AP:

- Add the Congestion Monitoring Request in the QoS Flow Level QoS Parameters IE.

- For NG-RAN node to indicate whether the QoS Flow is established with Congestion Monitoring request accepted. RAN3 adopt similar method as the indication to indicate the QoS Flow with PDU Set QoS parameters is accepted.

Proposal 2-2: To support ECN marking in UPF, RAN3 consider and study following enhancements to NG-U and F1-U: add the per QoS Flow congestion indication in NG-U and F1-U.

## 2.3	Network information exposure

TS23.501 states:

For the UL and/or DL congestion information monitoring (see clause 5.45.3), based on the PCC rule from PCF, the SMF requests the NG-RAN to report the information via GTP-U header to PSA UPF This NG-RAN reported information is common to support congestion information exposure and to support ECN marking for L4S in PSA UPF as described in clause 5.37.3.3.

This requires following enhancements to RAN3 interfaces:

- NGAP

The SMF may request the NG-RAN node to report the congestion information. This maybe similar to the ECN marking in the UPF which requires the NG-RAN to report the congestion information.

Similar to ECN marking, the target NG-RAN node need to know the request for congestion information during the NG-handover.

- NG-U

The NG-RAN node needs to report the congestion information via GTP-U. This maybe similar to the ECN marking in the UPF which requires the NG-RAN to report the congestion information.

- XnAP

Similar to ECN marking, the target NG-RAN node need to know the request for congestion information during the Xn handover preparation procedure or Xn Retrieve UE Context Procedure.

- F1AP

The gNB-CU can request the gNB-DU to report the congestion information. This maybe similar to the ECN marking in the NG-RAN or UPF which requires the gNB-DU to report the congestion information.

- F1-U

The gNB-DU node needs to report the congestion information via GTP-U to gNB-CU-UP. This maybe similar to the ECN marking in the NG-RAN or UPF which requires the gNB-DU to report the congestion information.

- E1AP

The gNB-CU-CP can request the gNB-CU-UP to report the congestion information to UPF via GTP-U. This maybe similar to the ECN marking in the UPF which requires the gNB-CU-UP to report the congestion information.

The required enhancement in RAN3 is same as the required enhancement to support ECN marking in UPF.

Proposal 3-1: To support network information exposure, the required enhancements is the same as the required enhancement to support ECN marking in UPF.

## 2.4	Stage-3

TS23.501 specifies:

Only one of ECN marking for L4S (in the case of ECN marking for L4S in RAN as described in clause 5.37.3) or QoS monitoring of congestion information may be requested to NG-RAN for a QoS Flow. They are mutually exclusive, therefore, measurements of Congestion information on a QoS Flow are not provided in QoS Monitoring reports if SMF enables ECN marking for L4S in RAN (see clause 5.37.3).

So the request to the NG-RAN node should be a CHOICE structure including

- ECN marking (in NG-RAN) request

- Congestion Monitoring request

For example, the NGAP IE can be defined as

| CHOICE ECN and Congestion Monitoring Request | M |  |  |  | - |  |
| --- | --- | --- | --- | --- | --- | --- |
| >ECN Marking |  |  |  |  |  |  |
| >>ECN Marking Request | M |  | ENUMERATED (UL, DL, Both, …, stop) |  | - |  |
| >Congestion Monitoring |  |  |  |  |  |  |
| >>Congestion Monitoring Request | M |  | ENUMERATED (UL, DL, Both, …, stop) |  | - |  |

Proposal 4-1: define a CHOICE structure to transfer a request for ECN marking in NG-RAN, or request for congestion monitoring.

Regarding the Stage-3 design for the congestion information, one option is to define it as a percentage, i.e. from 0 to 100%. However, this is really insufficient and would allow L4S flow to only have a smooth throughput up to 99Mbps (rule of thumb is r ~= 1/p - 1[Mbps]). All higher rates would start getting get a sawthooth, needing 1 report of 1% and then several reports of 0%. Using the full scale of the 8bit integer would allow a minimum percentage of 1/255, and a rate up to r ~= 1/p - 1[Mbps] ~= 254Mbps, so still not very future proof.

A future proof option is to encode it as a 8 bits floating-point fraction, in hexadecimal representation, with a 4 bits exponent (most significant bits, i.e. bits 5 to 8) and 4 bits mantissa (least significant bits, i.e. bits 1 to 4), Both exponent and mantissa are binary integers with a value within the range 0 to 15. The value that is represented needs to be interpreted as a fraction p = (16 + m) / 31 * 2^(e - 15) and where the binary value e=1111,m=1111 represents fraction value p = 1. The resulting fraction represents the ratio of packets requested (by the NG-RAN) to be used for ECN marking for L4S and needs to be multiplied by 100 to get the percentage value. As a single exception, the binary value e=0000, m=0000 needs to be interpreted as fraction 0 or 0%, which means no marking at all.

Each hexadecimal character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the exponent shall appear first in the string, and the character representing the mantissa shall appear last in the string.

Examples:

"FF" (e=1111, m=1111) => p=100%

"FE" (e=1111, m=1110) => p=96.77%

…

"EF" (e=1110, m=1111) => p=50%

…

"94" (e=1001, m=0100) => p=1.008%

…

"02" (e=0000, m=0010) => p=0.001772%

"01" (e=0000, m=0001) => p=0.001674%

"00" (e=0000, m=0000) => p=0%

Proposal 4-2: Define the congestion information as a 8-bit floating-point fraction, in hexadecimal representation.

# 3	Conclusion

In this contribution, we briefly analyzed the required enhancements in RAN3 to support ECN marking and congestion monitoring. Our proposals are:

Proposal 1-1: RAN3 consider and study following enhancements to NGAP, XnAP, F1AP and E1AP:

- Add the ECN Marking Request in the QoS Flow Level QoS Parameters IE.

- For NG-RAN node to indicate whether the QoS Flow is established with ECN marking request accepted, RAN3 adopt similar method as the indication to indicate the QoS Flow with PDU Set QoS parameters is accepted.

- Add the feedback indication to inform gNB-CU-CP when a QoS flow is established with ECN marking in gNB-CU-UP.

Proposal 1-2: gNB-DU report the per QoS Flow congestion indication to gNB-CU via F1-U, so the gNB-CU performs ECN marking.

Proposal 2-1: To support ECN marking in UPF, RAN3 consider and study following enhancements to NGAP, XnAP, F1AP, and E1AP:

- Add the Congestion Monitoring Request in the QoS Flow Level QoS Parameters IE.

- For NG-RAN node to indicate whether the QoS Flow is established with Congestion Monitoring request accepted. RAN3 adopt similar method as the indication to indicate the QoS Flow with PDU Set QoS parameters is accepted.

Proposal 2-2: To support ECN marking in UPF, RAN3 consider and study following enhancements to NG-U and F1-U: add the per QoS Flow congestion indication in NG-U and F1-U.

Proposal 3-1: To support network information exposure, the required enhancements is the same as the required enhancement to support ECN marking in UPF.

Proposal 4-1: define a CHOICE structure to transfer a request for ECN marking in NG-RAN, or request for congestion monitoring.

Proposal 4-2: Define the congestion information as a 8-bit floating-point fraction, in hexadecimal representation.

The draft NGAP TP can be found in ([5]).

# References

- RP-230786, WID for NR XR

- TS 23.501