3GPP TSG-RAN WG3 Meeting #121bis	R3-235573

Xiamen, P.R. China, 9th – 13th, October 2023

Agenda Item:	24.2

Source:	Ericsson

Title:	Text Proposal on XnAP: Introduction of Network Energy Saving

Document for:	Discussion, Decision

# 1	Information

This paper contains the text proposal to XnAP Baseline CR. It includes three parts:

- A new procedure for Cell DTRX information exchange

- Including Cell DRX and Cell DTX information in Dual Connectivity procedure

# 2	Text Proposal on XnAP

************************ 1st Change********************************

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the QoS Flow Level QoS Parameters IE for each QoS flow shall follow the principles specified for the PDU Session Resource Setup procedure in TS 38.413 [5].

************************ Skip the unchanged ********************************

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC procedure.

If the S-NG-RAN node UE Slice Maximum Bit Rate IE for a specific S-NSSAI is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use the received S-NG-RAN node UE Slice Maximum Bit Rate for all PDU sessions associated with the S-NSSAI for the concerned UE as defined in TS 23.501 [7].

If the Cell DRX Configuration IE and the Cell DTX Configuration IE are included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider to use the same Cell DRX and Cell DTX Configuration for the UE in the cell.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: S-NG-RAN node Addition Preparation, unsuccessful operation

If the S-NG-RAN node is not able to accept any of the bearers or a failure occurs during the S-NG-RAN node Addition Preparation, the S-NG-RAN node sends the S-NODE ADDITION REQUEST REJECT message with an appropriate cause value to the M-NG-RAN node.

#### 8.3.1.4	Abnormal Conditions

If the S-NG-RAN node receives an S-NODE ADDITION REQUEST message containing in a PDU Session Resource To Be Added Item IE neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE, the S-NG-RAN node shall fail the S-NG-RAN node Addition Preparation procedure indicating an appropriate cause.

************************ Skip to the next change********************************

### 8.4.x	Resource Status Exchange

#### 8.4.x.1	General

The purpose of the Resource Status Exchange procedure is to request and report the Resource Status, e.g.

network energy saving related information over the Xn-C interface, in order to assist the NG-RAN nodes to enhance the resource utilization.

The procedure uses non UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x: Resource Status Exchange, successful operation

The NG-RAN node1 initiates the procedure by sending the RESOURCE STATUS EXCHANGE REQUEST message to a peer NG-RAN node2.

On receipt of the RESOURCE STATUS EXCHANGE REQUEST message the NG-RAN node2 shall perform the requested resource status reporting and updates.

The Resource Status Reporting Request Type IE included in the RESOURCE STATUS EXCHANGE REQUEST message indicates to the NG-RAN node2 whether:

-	to report directly;

-	to stop reporting;

-	to report periodically;

-	to report when NG-RAN node2 starts to use NES

If the Interested Cell List IE is included in the Resource Status Reporting Request Type IE within the RESOURCE STATUS EXCHANGE REQUEST message, the NG-RAN node2 shall store this information and use it in the Resource Status updating procedure for the specified cells in the list.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

Void.

### 8.4.y	Resource Status Update

#### 8.4.y.1	General

The purpose of the Resource Status Update procedure is to report the Resource Status Update, e.g. network energy saving related information over the Xn-C interface

The procedure uses non UE-associated signalling.

#### 8.4.y.2	Successful Operation

Figure 8.4.y: Resource Status Update successful operation

The NG-RAN node2.initiates the procedure by sending the RESOURCE STATUS UPDATE message to a peer NG-RAN node1

#### 8.4.y.3	Unsuccessful Operation

Not applicable.

#### 8.4.y.4	Abnormal Conditions

Void.

************************ Skip to the next change********************************

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Source NG-RAN Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source NG-RAN node or the source SN. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| PSCell Change History | O |  | ENUMERATED (reporting full history, ...) |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| CHO Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| UE Cell DRX Configuration | O |  | OCTET STRING | Includes the CellDRX-Config IE as specified in TS 38.331[10] This IE indicates the Cell DRX configuration for the UE | YES | ignore |
| UE Cell DTX Configuration | O |  | OCTET STRING | Includes the CellDTX-Config IE as specified in TS 38.331[10] This IE indicates the Cell DTX configuration for the UE | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

************************ Skip to the next change********************************

#### 9.1.3.x	RESOURCE STATUS EXCHANGE REQUEST

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to request Resource Status for an Xn-C interface instance.

Direction: NG-RAN node1 à NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Resource Status Reporting Request Type | M |  | 9.2.3.x |  | YES | ignore |

#### 9.1.3.x+1	RESOURCE STATUS EXCHANGE RESPONSE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to responde to the request Resource Status for an Xn-C interface instance.

Direction: NG-RAN node2 à NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Resource Status Exchange Feedback | M |  | 9.2.3.x+1 |  | YES | ignore |

#### 9.1.3.x+2	RESOURCE STATUS UPDATE

This message is sent by a neighbouring NG-RAN node to a peer node to report the Resource Status Update.

Direction: NG-RAN node2 à NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Resource Status Scope | M |  |  |  | YES | ignore |
| >Cell Level |  |  |  |  |  |  |
| >>Cell DTRX Information | M |  | 9.2.3.y |  | – |  |

************************ Skip to the next change********************************

#### 9.2.3.x	Resource Status Reporting Request Type

This IE indicates the type of Resource Status Reporting Request to be handled by the NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED (direct, stop reporting, report periodically, report upon using NES, ...) |  |
| Report scope | M |  | ENUMERATED (cell, …) |  |
| Interested Cell List |  | 0..1 |  | Indicates the Cells that the resource status are interested by the sending NG-RAN node. |
| >Interested Cell Item |  | 1..<maxnoofCellsInNG-RAN node> |  |  |
| >>Cell ID | M |  | NR CGI 9.2.2.7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.2.3.x+1	Resource Status Exchange feedback

This IE provides the feedback of the Resource Status Exchange Request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Feedback | M |  | ENUMERATED (not using NES, …) |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  |

#### 9.2.3.y	Cell DTRX Information

This IE contains the Cell DTRX Information to be reported.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Cell List |  | 1 .. < maxnoofCellsinNG-RAN node> |  |  |
| >Cell ID | M |  | NR CGI 9.2.2.7 |  |
| >onDuration usage probablity | M |  | INTEGER (1..100) | Probability that the resource is used during the onDuration. |
| >offDuration usage probablity | M |  | INTEGER (1..100) | Probability that the resource is used during the offDuration. |
| >Duration | M |  | ENUMERATED {ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, ...} | Unit: milliSeconds |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

************************ Skip to the next change********************************

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextConfirm,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart,

HandoverSuccess,

ConditionalHandoverCancel,

EarlyStatusTransfer,

FailureIndication,

HandoverReport,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

AccessAndMobilityIndication,

CellTrafficTrace,

RANMulticastGroupPaging,

ScgFailureInformationReport,

ScgFailureTransfer,

F1CTrafficTransfer,

IABTransportMigrationManagementRequest,

IABTransportMigrationManagementResponse,

IABTransportMigrationManagementReject,

IABTransportMigrationModificationRequest,

IABTransportMigrationModificationResponse,

IABResourceCoordinationRequest,

IABResourceCoordinationResponse,

CPCCancel,

PartialUEContextTransfer,

PartialUEContextTransferAcknowledge,

PartialUEContextTransferFailure,

ResourceStatusExchangeRequest,

ResourceStatusExchangeResponse,

ResourceStatusUpdate

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-cellTrafficTrace,

id-RANMulticastGroupPaging,

id-scgFailureInformationReport,

id-scgFailureTransfer,

id-f1CTrafficTransfer,

id-iABTransportMigrationManagement,

id-iABTransportMigrationModification,

id-iABResourceCoordination,

id-retrieveUEContextConfirm,

id-cPCCancel,

id-partialUEContextTransfer,

id-resourceStatusExchange,

id-resourceStatusUpdate

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

XnAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnRemoval												|

xnSetup													|

nGRANnodeConfigurationUpdate							|

e-UTRA-NR-CellResourceCoordination						|

cellActivation											|

reset													|

resourceStatusReportingInitiation						|

mobilitySettingsChange									|

iABTransportMigrationManagement							|

iABTransportMigrationModification						|

iABResourceCoordination									|

partialUEContextTransfer								|

resourceStatusExchange,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport 			|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				|

cellTrafficTrace							|

rANMulticastGroupPaging					|

scgFailureInformationReport				|

scgFailureTransfer						|

f1CTrafficTransfer						|

retrieveUEContextConfirm				|

cPCCancel								|

resourceStatusUpdate,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

************************ Skip to the next change********************************

resourceStatusExchange	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusExchangeRequest

SUCCESSFUL OUTCOME		ResourceStatusExchangeResponse

PROCEDURE CODE			id-resourceStatusExchange

CRITICALITY				ignore

}

resourceStatusUpdate	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusUpdate

CRITICALITY				ignore

}

************************ Skip to the next change********************************

************************ Skip to the next change********************************

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

************************ Skip to the next change********************************

MDTPLMNModificationList,

F1-terminatingIAB-donorIndicator,

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

HashedUEIdentityIndexValue,

ResourceStatusExchangeRequestType,

ResourceStatusExchangeFeedback,

ResourceStatusScope

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

id-ActivationIDforCellActivation,

id-AdditionalDRBIDs,

************************ Skip to the next change********************************

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-HashedUEIdentityIndexValue,

id-UECellDRXConfiguration,

id-UECellDTXConfiguration,

id-ResourceStatusExchangeRequestType,

id-ResourceStatusExchangeFeedback,

id-ResourceStatusScope

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells

FROM XnAP-Constants;

************************ Skip to the next change********************************

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST

--

-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-UESecurityCapabilities				CRITICALITY reject		TYPE UESecurityCapabilities						PRESENCE mandatory}|

{ ID id-s-ng-RANnode-SecurityKey			CRITICALITY reject		TYPE S-NG-RANnode-SecurityKey					PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUE-AMBR					CRITICALITY reject		TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-selectedPLMN						CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList				CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-indexToRatFrequSelectionPriority	CRITICALITY reject		TYPE RFSP-Index										PRESENCE optional }|

{ ID id-PDUSessionToBeAddedAddReq			CRITICALITY reject		TYPE PDUSessionToBeAddedAddReq					PRESENCE mandatory}|

{ ID id-MN-to-SN-Container					CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE optional }|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore		TYPE ExpectedUEBehaviour							PRESENCE optional }|

{ ID id-requestedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-PCellID								CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-DesiredActNotificationLevel			CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AvailableDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE conditional}

-- The IE shall be present if there is at least one  PDUSessionResourceSetupInfo-SNterminated included --|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting		CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern					CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional }|

{ ID id-S-NG-RANnode-Addition-Trigger-Ind	CRITICALITY reject		TYPE S-NG-RANnode-Addition-Trigger-Ind		PRESENCE optional }|

{ ID id-TraceActivation						CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-UERadioCapabilityID					CRITICALITY reject		TYPE UERadioCapabilityID							PRESENCE optional }|

{ ID id-SourceNG-RAN-node-ID				CRITICALITY ignore		TYPE GlobalNG-RANNode-ID							PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY ignore		TYPE MDTPLMNList									PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore		TYPE UEHistoryInformation							PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore		TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-PSCellChangeHistory					CRITICALITY ignore		TYPE PSCellChangeHistory							PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject		TYPE IABNodeIndication								PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore		TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-CHOinformation-AddReq				CRITICALITY reject		TYPE CHOinformation-AddReq						PRESENCE optional }|

{ ID id-SCGActivationRequest				CRITICALITY ignore		TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CPAInformationRequest				CRITICALITY reject		TYPE CPAInformationRequest						PRESENCE optional }|

{ ID id-S-NG-RANnodeUE-Slice-MBR				CRITICALITY reject		TYPE UESliceMaximumBitRateList					PRESENCE optional }|

{ ID id-F1-terminatingIAB-donorIndicator		CRITICALITY reject		TYPE F1-terminatingIAB-donorIndicator			PRESENCE optional }|

{ ID id-UECellDRXConfiguration				CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-UECellDTXConfiguration				CRITICALITY ignore		TYPE OCTET STRING							PRESENCE optional },

...

}

************************ Skip to the next change********************************

-- **************************************************************

--

#### -- RESOURCE STATUS EXCHANGE REQUEST

--

-- **************************************************************

ResourceStatusExchangeRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResourceStatusExchangeRequest-IEs}},

...

}

ResourceStatusExchangeRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ResourceStatusExchangeRequestType		CRITICALITY ignore		TYPE ResourceStatusExchangeRequestType		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS EXCHANGE RESPONSE

--

-- **************************************************************

ResourceStatusExchangeResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResourceStatusExchangeResponse-IEs}},

...

}

ResourceStatusExchangeResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ResourceStatusExchangeFeedback		CRITICALITY ignore		TYPE ResourceStatusExchangeFeedback		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS EXCHANGE UPDATE

--

-- **************************************************************

ResourceStatusExchangeUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResourceStatusExchangeUpdate-IEs}},

...

}

ResourceStatusExchangeUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ResourceStatusScope		CRITICALITY ignore		TYPE ResourceStatusScope		PRESENCE optional },

...

}

************************ Skip to the next change********************************

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

************************ Skip to the next change********************************

CellDTRXInformation ::= SEQUENCE {

cell-List				CellDTRXInformationCellList,

iE-Extensions			ProtocolExtensionContainer {CellDTRXInformation-ExtIEs} }	OPTIONAL,

...

}

CellDTRXInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellDTRXInformationCellList ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellDTRXInformationCellList-Item

ResourceStatusInterestedCell-Item ::= SEQUENCE

cell-ID				NR-CGI,

onDurationUsageProbability		INTEGER (1..100),

offDurationUsageProbability		INTEGER (1..100),

duration						ENUMERATED {ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,ms80, ms100, ms200, ms300,

ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, ...}

iE-Extensions		ProtocolExtensionContainer { {ResourceStatusInterestedCellItem-ExtIEs} }	OPTIONAL,

...

}

ResourceStatusInterestedCellItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

************************ Skip to the next change********************************

ResourceStatusExchangeFeedback ::= SEQUENCE {

feedback				ENUMERATED {not-using-NES, ...},

criticalityDiagnostics	CriticalityDiagnostics		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ResourceStatusExchangeFeedback-ExtIEs} }	OPTIONAL,

...

}

ResourceStatusExchangeFeedback-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceStatusExchangeRequestType ::= SEQUENCE {

event-Type			ENUMERATED {direct, stop-reporting, report-periodically, report-upon-using-NES, ...},

report-scope		ENUMERATED {cell, ...},

interesteCellList	SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF ResourceStatusInterestedCell-Item		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ResourceStatusExchangeRequest-ExtIEs} }	OPTIONAL,

...

}

ResourceStatusExchangeRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceStatusInterestedCell-Item ::= SEQUENCE

cell-ID				NR-CGI,

iE-Extensions		ProtocolExtensionContainer { {ResourceStatusInterestedCell-Item-ExtIEs} }	OPTIONAL,

...

}

ResourceStatusInterestedCell-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceStatusScope ::= CHOICE {

cell-level				CellDTRXInformation,

choice-extension		ProtocolIE-Single-Container { {ResourceStatusScope-ExtIEs} }

}

ResourceStatusScope-ExtIEs XNAP-PROTOCOL-IES ::= {

...

************************ Skip to the next change********************************

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-cellTrafficTrace 											ProcedureCode ::= 38

id-RANMulticastGroupPaging										ProcedureCode ::= 39

id-scgFailureInformationReport									ProcedureCode ::= 40

id-ProcedureCode41-NotToBeUsed									ProcedureCode ::= 41

id-scgFailureTransfer											ProcedureCode ::= 42

id-f1CTrafficTransfer											ProcedureCode ::= 43

id-iABTransportMigrationManagement								ProcedureCode ::= 44

id-iABTransportMigrationModification							ProcedureCode ::= 45

id-iABResourceCoordination										ProcedureCode ::= 46

id-retrieveUEContextConfirm										ProcedureCode ::= 47

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

id-resourceStatusExchange										ProcedureCode ::= 90 -- to be allocated

id-resourceStatusUpdate											ProcedureCode ::= 91 -- to be allocated

************************ Skip to the next change********************************

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-HashedUEIdentityIndexValue																		ProtocolIE-ID ::= 372

id-QosFlowMappingIndication																			ProtocolIE-ID ::= 373

id-Full-and-Short-I-RNTI-Profile-List																ProtocolIE-ID ::= 374

id-UECellDRXConfiguration																			ProtocolIE-ID ::= 999 -- to be allocated

id-UECellDTXConfiguration																			ProtocolIE-ID ::= 998 -- to be allocated

id-ResourceStatusExchangeRequestType																ProtocolIE-ID ::= 997 -- to be allocated

id-ResourceStatusExchangeFeedback																	ProtocolIE-ID ::= 996 -- to be allocated

id-ResourceStatusScope																				ProtocolIE-ID ::= 995 -- to be allocated

END

-- ASN1STOP