3GPP TSG-RAN WG3 Meeting #121bis-e	R3-235584

Xiamen, China, 09 – 13 October 2023

Agenda Item:	23.2.2

Source:	Ericsson

Title:	Way forward on LPHAP

Document for:	Discussion, Other

# Introduction

In last meeting, the following agreements were taken:

RAN3#121: WA: LMF provides the validity area as a list of cells, and SRS recommendation to the serving gNB. The serving gNB replies with a single SRS configuration (as in legacy positioning information exchange procedure). The details of the SRS recommendation are FFS e.g. SRS configuration or requested SRS transmission characteristics with additional information. FFS on whether the serving gNB includes a list of cells identifying a modification of the validity area in the response message to the LMF.

The assumption is that LMF will signal a list of cells and SRS assistance information. It is FFS whether the assistance information is the LMF-constructed SRS configuration to endorse by serving gNB, or the SRS recommendation with some additional information.

We propose a way forward based on RAN1’s latest agreement and RAN2 progress from last meeting.

# Discussion

The following agreement was made by RAN1 in last meeting and communicated via LS to RAN2, cc: RAN3 in [1]:

Regarding the parameters for area-specific SRS configuration, - RAN1 has agreed the following area-specific parameters for SRS for positioning configurations in a validity area: - inactivePosSRS-TimeAlignmentTimer - inactivePosSRS-RSRP-ChangeThreshold - BWP parameters - locationAndBandwidth - subcarrierSpacing - cyclicPrefix - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - p0 and alpha - pathlossReferenceRS-Pos - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping - resourceType - sequenceID - spatialRelationInfoPos - In addition, from RAN1’s perspective, the area-specific parameters should also include the following: - A list of PCIs defining the positioning area - autonomous TA adjustment enabler

From the above list, it can be construed that the information discloses the LPHAP SRS parameters that need to be part of the SRS configuration sent by the serving gNB to UE in the RRC Release message to be valid across the validity area. In fact, the first main bullets are information related to RRC_INACTIVE configuration in green, which are legacy parameters. These are not recommended by LMF nor sent back by gNB to LMF.

In the RAN2 RRC baseline CR [2], we can see the structure of the RRC message that contains the LPHAP SRS configuration:

RRCRelease message -- ASN1START -- TAG-RRCRELEASE-START […] SRS-PosRRC-Inactive-r17 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveConfig-r17) SRS-PosRRC-InactiveValidityArea-r18 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveValidtyAreaConfig-r18) SRS-PosRRC-InactiveConfig-r17 ::= SEQUENCE { srs-PosConfigNUL-r17 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosConfigSUL-r17 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r17 BWP OPTIONAL, -- Need S bwp-SUL-r17 BWP OPTIONAL, -- Need S inactivePosSRS-TimeAlignmentTimer-r17 TimeAlignmentTimer OPTIONAL, -- Need M inactivePosSRS-RSRP-ChangeThreshold-r17 RSRP-ChangeThreshold-r17 OPTIONAL -- Need M } SRS-PosRRC-InactiveValidityAreaConfig-r18 ::= SEQUENCE { srs-PosRRC-InactiveValidityArea-r18 SEQUENCE (SIZE(1..maxNrOfCellIsInVA-r18)) OF CellIdentity, srs-PosConfigNUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosConfigSUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r18 BWP OPTIONAL, -- Need S bwp-SUL-r18 BWP OPTIONAL, -- Need S inactivePosSRS-ValidityAreaTAT-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity}, inactivePosSRS-ValidityAreaRSRP-r18 RSRP-ChangeThreshold-r17 OPTIONAL -- Need M ... } Editor’s Note: FFS on timer value

Taking past Rel-17 work into consideration, we note that some enhancement to F1AP Positioning Information Exchange procedure is needed to add support for the SRS-PosRRC-InactiveValidityAreaConfig request from the CU to DU, and for the DU to provide the configuration to CU, so that CU transmits it to UE in the RRC Release message. Example to TS 38.473 impact is provided below:

#### 9.2.12.13 POSITIONING INFORMATION REQUEST This message is sent by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals for uplink positioning measurement and also to retrieve the SRS configuration from the gNB-DU. Direction: gNB-CU gNB-DU. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. gNB-CU UE F1AP ID; M; 9.3.1.4; YES; reject. gNB-DU UE F1AP ID; M; 9.3.1.5; YES; reject. Requested SRS Transmission Characteristics; O; 9.3.1.175; YES; ignore. UE Reporting Information; O; 9.3.1.255; YES; ignore. SRS Positioning INACTIVE Query Indication; O; ENUMERATED (true, ...); Applicable only if the Requested SRS Transmission Characteristics IE is present; YES; ignore. SRS Positioning INACTIVE Validity Area Query Indication; O; ENUMERATED (true, ...); Applicable only if the Requested SRS Transmission Characteristics IE is present; YES; ignore. #### 9.2.12.14 POSITIONING INFORMATION RESPONSE This message is sent by the gNB-DU to provide the configured SRS information to the gNB-CU. Direction: gNB-DU gNB-CU. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. gNB-CU UE F1AP ID; M; 9.3.1.4; YES; reject. gNB-DU UE F1AP ID; M; 9.3.1.5; YES; reject. SRS Configuration; O; 9.3.1.192; YES; ignore. SFN Initialisation Time; O; Relative Time 1900 9.3.1.183; YES; ignore. Criticality Diagnostics; O; 9.3.1.3; YES; ignore. SRS-PosRRC-InactiveConfig; O; OCTET STRING; Includes the SRS-PosRRC-InactiveConfig IE as defined in TS 38.331 [8]; YES; ignore. SRS-PosRRC-InactiveValidityAreaConfig; O; OCTET STRING; Includes the SRS-PosRRC-InactiveValidityAreaConfig IE as defined in TS 38.331 [8]; YES; ignore.

Proposal 1: Include the SRS Positioning INACTIVE Validity Area Query Indication IE in the F1 POSITIONING INFORMATION REQUEST message and the SRS-PosRRC-InactiveValidityAreaConfig IE in the F1 POSITIONING INFORMATION RESPONSE

Additionally, to be able to query for the SRS-PosRRC-InactiveValidityAreaConfig, the gNB-CU must have received a recommendation from LMF that contains LPHAP assistance information. To do so, we can define a new LPHAP Assistance Information IE within the Requested SRS Transmission Characteristics IE signalled in the NRPPA POSITIONING INFORMATION REQUEST message, which contains Validity Area list, defined as a list of CGIs, and Time Alignment timer for VA. The value of the TA timer follows RAN2 RRC definition.

Proposal 2: The LMF signals a new LPHAP Assistance Information IE within the Requested SRS Transmission Characteristics IE in the NRPPA POSITIONING INFORMATION REQUEST message, that contains Validity Area list, defined as a list of NR CGIs, and Time Alignment timer for VA.

Note: the LPHAP indication defined in TS 23.273 can be implicitly signalled from the name of the LPHAP Assistance Information IE.

Since the VA timer and list of CGI are CU-level information, the F1 Requested SRS Transmission Characteristics IE does not need to be impacted.

On the rationale of signalling the VA timer from LMF, it is so that the serving gNB-CU includes this value in the TA command sent to UE during the RRC Release message. But the serving gNB-CU may end up deciding on another value of the TA timer for VA, based on the final VA size, available SRS resource pool, and positioning requirements to mitigate interference. Therefore, the serving gNB should also inform of final VA specific TA timer value to LMF. The LMF needs to be aware of the final value of the TA VA timer so that it knows for how long the VA LPHAP resources are valid before it can send a NRPPA POSITIONING DEACTIVATION message to end the LPHAP session in the network.

Observation 2: the VA specific TA timer value in the RRC Release message is tied, among others, to VA size, available SRS resource pool, and positioning requirements to mitigate interference. The LMF needs to be aware of the final value of the TA VA timer so that it knows for how long the VA LPHAP resources are valid before it can send a NRPPA POSITIONING DEACTIVATION message to end the LPHAP session in the network.

Proposal 3: In order for LMF to know the time validity of the LPHAP session, the serving gNB must signal the final VA specific TA timer value to LMF together with the SRS-PosRRC-InactiveValidityAreaConfig.

Once the LMF receives the VA-dedicated SRS configuration and the VA timer from the serving gNB, it notifies the other gNBs in the VA.

Furthermore, it is an open question whether the serving gNB includes a list of cells identifying a modification of the validity area in the NRPPA POSITIONING INFORMATION RESPONSE message to the LMF. Based on the discussion from last meeting, two options can be considered:

- the gNB notifies the LMF of a modification of the validity area, for instance, if the SRS configuration could only be associated to a subset of the list of cells signaled initially from LMF.

- The gNB either successfully configures or fails the SRS LPHAP configuration for the indicated fixed validity area. In this case a new cause value is needed for the failure.

In both cases, an enhancement is required, either to include the list of cells associated with the SRS LPHAP configuration in the NRPPA POSITIONING INFORMATION RESPONSE message, or to include a new cause value, e.g. "LPHAP Validity Area not supported" in the ERROR INDICATION message.

Given that the failure scenario should be avoided, otherwise it may increase the number of trial and error between LMF and gNB, and that it is within the gNB's remit to modify the validity area, there is no identified problem preventing the gNB from pruning the validity area size based on the gNB's resources to configure an associated LPHAP SRS configuration. Therefore, we propose to support option 1.

Proposal 4: The serving gNB notifies the LMF in the NRPPA POSITIONING INFORMATION RESPONSE message of a modification of the validity area, for instance, that the SRS LPHAP configuration could only be associated to a subset of the list of cells signaled initially from LMF.

In the scenario when UE resumes out of VA, the LMF must be timely informed of UE’s new cell ID to make decision. Whether the decision is to update LPHAP VA or to terminate LPHAP and start legacy positioning is up to LMF implementation. It is proposed that the new gNB sends a POSITIONING INFORMATION UPDATE message with new Cell ID IE to the LMF.

For the sake of progress, the aspects related to the above two scenarios of notification from LMF to neighbor gNBs, and of UE leaving the VA can be further discussed, once the interaction between the LMF and serving gNB is in better shape for LPHAP configuration..

# Conclusion

In this document we made the following observations and proposals:

Proposal 1: Include the SRS Positioning INACTIVE Validity Area Query Indication IE in the F1 POSITIONING INFORMATION REQUEST message and the SRS-PosRRC-InactiveValidityAreaConfig IE in the F1 POSITIONING INFORMATION RESPONSE

Proposal 2: The LMF signals a new LPHAP Assistance Information IE within the Requested SRS Transmission Characteristics IE in the NRPPA POSITIONING INFORMATION REQUEST message, that contains Validity Area list, defined as a list of NR CGIs, and Time Alignment timer for VA.

Observation 2: the VA specific TA timer value in the RRC Release message is tied, among others, to VA size, available SRS resource pool, and positioning requirements to mitigate interference. The LMF needs to be aware of the final value of the TA VA timer so that it knows for how long the VA LPHAP resources are valid before it can send a NRPPA POSITIONING DEACTIVATION message to end the LPHAP session in the network.

Proposal 3: In order for LMF to know the validity of the LPHAP session, the serving gNB must signal the final VA specific TA timer value to LMF together with the SRS-PosRRC-InactiveValidityAreaConfig.

Proposal 4: The serving gNB notifies the LMF in the NRPPA POSITIONING INFORMATION RESPONSE message of a modification of the validity area, for instance, that the SRS LPHAP configuration could only be associated to a subset of the list of cells signaled initially from LMF.

# References

[1]	R1-2308349. Reply LS on LPHAP, RAN1#114

[2]	R2-2309182, Rapporteur CR for Positioning RRC Changes

# Annex 1: TP to NRPPA BL CR

<<<<<<<<<<<<<<<<<<<< Beginning of changes >>>>>>>>>>>>>>>>>>>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

CG-SDT	Configured Grant Small Data Transmission

CID	Cell-ID (positioning method)

DL-PRS	Downlink Positioning Reference Signal

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

LMF	Location Management Function

LPHAP	Low Power High Accuracy Positioning

LPP	LTE Positioning Protocol

MSAS	Multi-functional Satellite Augmentation System

NavIC	NAVigation with Indian Constellation

NRPPa	NR Positioning Protocol A

OTDOA	Observed Time Difference of Arrival

posSIB	Positioning SIB

PRS	Positioning Reference Signal (for E-UTRA)

QZSS	Quasi-Zenith Satellite System

RSRP	Reference Signal Received Power

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SBAS	Space Based Augmentation System

SRS	Sounding Reference Signal

TEG	Timing Error group

TRP	Transmission-Reception Point

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RSCP	UL Reference Signal Carrier Phase

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL SRS-RSRPP	UL SRS reference signal received path power

WAAS	Wide Area Augmentation System

Z-AoA	Zenith Angles of Arrival

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.6	Positioning Information Exchange

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the NG-RAN node shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If the UE TEG Information Request IE is included in the POSITIONING INFORMATION REQUEST message and set to "onDemand", the NG-RAN node shall, if supported, provide the UE Tx TEG association in the POSITIONING INFORMATION RESPONSE message.

If the UE TEG Information Request IE is set to "periodic", the NG-RAN node shall, if supported, reply with the POSITIONING INFORMATION RESPONSE message without including any UE Tx TEG association in this message. The NG-RAN node shall then take the UE TEG Reporting Periodicity IE into account when configuring the UE’s periodic UE Tx TEG association reporting and initiate the Positioning Information Update procedure for reporting the UE Tx TEG association received from the UE, if any.

If the Time Window Information for SRS IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node shall, if supported, configure the UE to start transmitting its UL SRS transmission at the indicated time instance.

If the LPHAP Assistance Information IE is included in the Requested SRS Transmission Characteristics IE within the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE in LPHAP session, and it shall include the SRS LPHAP Configuration Information IE in the POSITIONING INFORMATION RESPONSE message.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.11	POSITIONING INFORMATION RESPONSE

This message is sent by the NG-RAN node to provide positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| UE Tx TEG Association List | O |  | 9.2.78 |  | YES | ignore |
| SRS LPHAP Configuration Information | O |  | 9.2.A |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. |  |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  |  |  |
| CHOICE Bandwidth | M |  |  |  |  |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ...) |  |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  |  |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  |  |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. |  |  |
| >>Periodicity List |  | 0.. 1 |  |  |  |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |  |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds |  |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. |  |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  |  |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  |  |  |
| SSB Information | O |  | 9.2.54 |  |  |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| LPHAP Assistance Information | O |  | 9.2.B |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.A	SRS LPHAP Configuration Information

This IE contains the SRS LPHAP configuration information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS LPHAP Configuration | M |  | OCTET STRING | Includes the SRS-PosRRC-InactiveValidityAreaConfig IE as defined in TS 38.331 [13] |
| Validity Area specific Timer | M |  | ENUMERATED (ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity), |  |
| List of determined cells for VA determination |  | 0 .. <maxnoOfCells> |  |  |
| >NR CGI | M |  | 9.2.9 |  |

### 9.2.B	LPHAP Assistance Information

This IE contains the LPHAP assistance information to help gNB determine the SRS LPHAP configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| List of recommended cells for VA determination |  | 1 .. <maxnoOfCells> |  |  |
| >NR CGI | M |  | 9.2.9 |  |
| Validity Area specific Timer | M |  | ENUMERATED (ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity), | Suggested VA specific TA timer value |

# Annex 1: TP to F1AP BL CR

<<<<<<<<<<<<<<<<<<<< Beginning of changes >>>>>>>>>>>>>>>>>>>>

### 8.13.9	Positioning Information Exchange

#### 8.13.9.1	General

The Positioning Information Exchange procedure is initiated by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals and to retrieve the SRS configuration from the gNB-DU. The procedure uses UE-associated signalling.

#### 8.13.9.2	Successful Operation

Figure 8.13.9.2-1: Positioning Information Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the gNB-DU.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message. If the SRS Positioning INACTIVE Query Indication IE is also included in the POSITIONING INFORMATION REQUEST message and set to ‘true’, the gNB-DU shall, if supported, include the SRS-PosRRC-InactiveConfig IE in the POSITIONING INFORMATION RESPONSE message. If the SRS Positioning INACTIVE Validity Area Query Indication IE is also included in the POSITIONING INFORMATION REQUEST message and set to ‘true’, the gNB-DU shall, if supported, include the SRS-PosRRC-InactiveValidityAreaConfig IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the gNB-DU shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU may take this information into account for allocating proper CG-SDT resources when positioning a UE.

Interaction with the UE Context Modification Required (gNB-DU initiated) procedure:

The UE Context Modification Required (gNB-DU initiated) procedure may be performed before the POSITIONING INFORMATION RESPONSE message.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.12.13	POSITIONING INFORMATION REQUEST

This message is sent by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals for uplink positioning measurement and also to retrieve the SRS configuration from the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Requested SRS Transmission Characteristics | O |  | 9.3.1.175 |  | YES | ignore |
| UE Reporting Information | O |  | 9.3.1.255 |  | YES | ignore |
| SRS Positioning INACTIVE Query Indication | O |  | ENUMERATED (true, ...) | Applicable only if the Requested SRS Transmission Characteristics IE is present | YES | ignore |
| SRS Positioning INACTIVE Validity Area Query Indication | O |  | ENUMERATED (true, ...) | Applicable only if the Requested SRS Transmission Characteristics IE is present | YES | ignore |

#### 9.2.12.14	POSITIONING INFORMATION RESPONSE

This message is sent by the gNB-DU to provide the configured SRS information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| SRS-PosRRC-InactiveConfig | O |  | OCTET STRING | Includes the SRS-PosRRC-InactiveConfig IE as defined in TS 38.331 [8] | YES | ignore |
| SRS-PosRRC-InactiveValidityAreaConfig | O |  | OCTET STRING | Includes the SRS-PosRRC-InactiveValidityAreaConfig IE as defined in TS 38.331 [8] | YES | ignore |