3GPP TSG-RAN WG3 Meeting #121bis	R3-235598

Xiamen, China, October 09 – 13, 2023

Title:	[draft] Reply LS on the user consent for trace reporting

Response to:

Release:	Rel-18

Work Item:	NR_ENDC_SON_MDT_enh-Core

Source:	Ericsson (to be RAN3)

To:	SA3, SA5, RAN2

Cc:	SA1, RAN

Contact person:	Angelo Centonza

Angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	To be added

# 1	Overall description

RAN3 thanks SA3 for the answers provided in their LS on user consent for trace reporting. The answers provided by SA3 are listed below:

A1: The existing user consent mechanism is only intended for internal use within the 3GPP operators (controllers) domain for collection MDT measurements at the RAN and reporting them to the Trace Collection Entity.

A2: Further, user consent is given to the operator so that the 3GPP system can be provisioned/configured based on the operator-subscriber agreed permissions stored in UDM to make it feasible for the 3GPP system to comply with local laws and regulations.

Whether the RAN needs to check if user consent is required for a specific type of information/data of a subscriber for a particular purpose can be configured by the OAM. Such configuration is done based on local regulations, which is likely to change infrequently. From the UDM, per UE basis, the RAN receives the yes/no information on whether a user has given consent for the information/data configured by the OAM to be used by the RAN for a particular purpose.

The steps described above include the method detailed in Q2. Hence the method is feasible.

RAN3 would like to inform SA3 and SA5 that the requirements on user consent and the workflow for a user consent solution described by SA3 have been adopted by RAN3, entailing amending the current solution on user consent for MDT. RAN3 converged to a solution on user consent that follows the steps below:

1)	The OAM configures the RAN with a list of MDT measurements that are subject to user consent.

2)	MDT measurements not listed in such OAM configuration can be collected by the RAN and reported to the TCE without consent from the user.

3)	The current MDT PLMN List IE can be reused to indicate to the RAN the PLMNs for which collection and distribution of MDT measurements configured to the RAN by the OAM is allowed, according to the list in 1)

RAN3 has amended their specifications according to the solution described above, as per attached CR.

# 2	Actions

To: SA3

ACTION: 	RAN3 kindly asks SA3 to take the above progress into account.

To: SA5 and RAN2

ACTION: 	RAN3 kindly asks SA5 and RAN2 to take the above progress into account and to amend their specifications so to align with the guidance provided by SA3 and with the solution agreed by RAN3.

# 3	Dates of next TSG RAN WG 3 meetings

Please check the RAN3 calendar here