3GPP TSG-RAN3 #121-bis	R3-235603

9th–13th October 2023

Xiamen, P.R. China

Title: 	(TP for AI/ML BLCR to TS38.423) AI/ML Network Energy Saving

Source: 	Ericsson

Agenda item: 	12.2.2.3

Document Type:	Discussion and Decision

# Introduction

In this paper, a TP is presented reflecting the proposals in R3-235602.

# TP for TS 38.423

<<<<<< Start of CHANGES >>>>>>

#### 9.1.3.FF	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item(FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| UE Associated Info Result |  | 0..1 |  |  | YES | FFS |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >> UE Assistant Identifier (FFS on the name) | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1 | – |  |
| >>UE Performance | O |  | 9.2.3.Y |  | – |  |
| >>UE trajectory | O |  | FFS |  | – |  |
| Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates zero measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. Energy Consumption is measured on a linear scale. (FFS) | YES | FFS |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

<<<<<< End of CHANGES >>>>>