3GPP TSG-RAN WG3 Meeting #121-bis	R3-235610

Xiamen, China, 09 – 13 October, 2023

Agenda item:	12.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to TS 38.300) Stage 2 Updates

Document for:	Text Proposal

# 1	Introduction

In RAN3 #117 we agreed the following:

Not to capture the flow charts right now, can be considered after the standard impacts are identified.

In RAN3 #119 also the following was agreed:

The agreed class1 procedure (AI/ML INFORMATION REQUEST/RESPONSE, the name needs further discussion) is used to configure UE performance feedback reporting.

Introduce into the agreed new request message (AI/ML INFORMATION REQUEST, the name needs further discussion), an indication that UE performance feedback is provided after handover event. Whether the indication is in implicit or explicit way needs to be further discussed.

Introduce a trigger indication in the HO request message to indicate that UE performance feedback is requested after HO completion. The details of indication need to be discussed.

The agreed new class2 non-UE associated procedure (AI/ML INFORMATION UPDATE, which name is FFS) is used for UE performance feedback reporting.

In RAN3 #121 we agreed the following with respect to UE Trajectory Information:

The measurement object to request the report of the UE Trajectory IE should be introduced in the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message.

The AI/ML Measurement ID in HANDOVER REQUEST from source gNB is reused to trigger the target node feedback the UE trajectory information to the source gNB.

AI/ML INFORMATION UPDATE from target node is used to convey the UE trajectory information.

The Measured UE Trajectory reporting is one-time report.

The conditions triggering the one-time reporting are at least:

- Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE);

- Condition B: UE leaves the target node;

- Condition C: Expiration of the report time duration;

- Condition D: The configured number of hand-overs within the same target node is reached.

Report time duration and configured number of intra-NG-RAN node inter-cell handovers are included in DATA COLLECTION REQUEST message.

In this contribution, we proceed to capture some stage 2 aspects and add missing flow charts details related to the interaction of the Data Collection procedure and a Xn Handover. We also add Stage2 text related to agreements on the measured UE Trajectory.

# 2	Discussion

In the last meeting we made some agreements related to the measured UE Trajectory, which haven’t been captured in Stage2 text.

Also, Data Collection Initiation and Data Collection Reporting procedures are non UE-associated procedures. When it comes to the measurements of UE Performance Feedback and measured UE Trajectory, they are configured through the Data Collection Initiation procedure, but they are triggered, in a UE-associated way, through a Handover. We think that this interaction between the Data Collection Initiation and Handover procedures needs to be documented in Stage2 for Handover-triggered measurements, since no sufficient details are provided in Stage3.

Proposal: Discuss and agree the TP to BLCR to TS 38.300 provided in the Annex.

# 3	Conclusion

In this contribution we make the following proposal:

Proposal: Discuss and agree the TP to BLCR to TS 38.300 provided in the Annex.

# References

[1]		R3-234785, Draft CR to 38.300 on AI/ML for NG-RAN

# Annex	- TP for TS 38.300

This TP is based on [1].

******************************************Start of 1st Change****************************************

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

******Unchanged Text Omitted******

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

ML	Machine Learning

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

******************************************End of 1st Change****************************************

******************************************Start of 2nd Change***************************************

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

The predicted Resource Status Information as well as the UE performance feedback can be configured to be reported once or periodically. This information can be used for all use cases. The reporting is configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

UE Performance feedback is configured to be reported within a time duration, that starts at the time of successful handover completion. UE Performance feedback measurement collection terminates when UE moves to RRC_IDLE or RRC_INACTIVE states, when UE is handed over to another cell or at expiration of the configured time duration.

Measured UE Trajectory information can be configured to be reported once, within a configured time duration. The reporting is also configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure. Measured UE Trajectory reporting is triggered when UE moves to RRC_IDLE or RRC_INACTIVE states, when UE leaves the target node, when a configured number of handovers within the same target node is reached or at expiration of the configured time duration.

The cell-based UE trajectory prediction, which can be used e.g. for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Request message to provide information for e.g. subsequent mobility decisions.

The metric of Energy Cost (EC), which can be used e.g. for the Network Energy Saving use case, is introduced and can be exchanged with the neighbouring NG-RAN nodes upon request.

******************************************End of Change******************************************

******************************************Start of 3rd Change**************************************

### X.X.3 Handover-Triggered Data Collection Reporting

- NG-RAN node 1 initiates the Data Collection Initiation procedure towards NG-RAN node 2, associating an AI/ML measurement ID to a number of handover-triggered measurements. NG-RAN node 1 also indicates a time window during which the handover-triggered measurements shall be reported by NG-RAN node 2.

Note: Handover-triggered measurements include UE Performance Feedback and UE Trajectory information.

2.	NG-RAN node 1 hands over a UE by indicating the AI/ML measurement ID (FFS on the name) of the requested handover-triggered measurements configured in step 1. Handover may correspond to normal Handover or to conditional Handover.

3.	Upon successful completion of the Handover for which an AI/ML measurement ID (FFS on the name) is indicated, NG-RAN node 2 starts the corresponding measurements and reports those to the NG-RAN node 1 within the indicated time window by initiating the Data Collection Reporting procedure.

******************************************Start of 3rd Change**************************************