3GPP TSG-RAN WG3 Meeting #121-bis	R3-235613

Xiamen, China, 09 – 13 October, 2023

Agenda item:	12.2.2.3

Source:	Nokia, Nokia Shanghai Bell, Orange

Title:	(TP to TS 38.423) Energy Efficiency aspects related to SA5 LS and Energy Cost Description

Document for:	Text Proposal

# 1	Introduction

In RAN3 #120, RAN3 sent an LS to SA5 in [1], in which RAN3 informed SA5 about the introduction of a new Energy Cost metric, that is encoded as an index, normalized by rules provided by OAM., and which provides a representation of the energy consumption at an NG-RAN node. In the LS, RAN3 indicated that it assumes that the rules used for the normalization shall be the same at least for all neighbouring NG-RAN nodes within the area where a request on Energy Cost reporting is triggered by an NG-RAN node in order to map the Energy Cost value of an NG-RAN node to a measurement of consumed energy, ensuring normalization of the exchanged Energy Cost information. RAN3 in the LS asked SA5 to provide feedback on the feasibility of the above and to provide feedback back to RAN3, if needed.

In [2], SA5 indicated the understanding of SA5 on the problem namely that:

- NG-RAN nodes (i.e. gNBs and ng-eNBs) should exchange information about their respective energy consumption. However, instead of exchanging their actual energy consumption (e.g. in Kw/h), RAN3 expects the actual energy consumption be converted into an ‘Energy Cost’, i.e. an index which reflects the energy consumption of the NG-RAN Nodes.

- The rule to convert the actual energy consumption of the NG-RAN nodes into the Energy Cost is provisioned via OAM into the NG-RAN nodes and is unified across the Operator’s network.

Additionally, SA5 in the response LS asked RAN3 additional questions for clarification:

- Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other?

- Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)?

- Should the Energy Cost be calculated in near-real time or non-real time (see above)?

- What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

In this contribution, we provide our views to the questions from SA5 related to the original LS sent by RAN3 [1].

On another aspect, in RAN3 #121 we made an agreement not to consider a predicted Energy Cost in Rel-18 namely:

The definition and signaling over RAN interfaces of the Additional Load as well as Inferred EC are not pursued in Rel-18.

and we captured a semantics description, with details still FFS, of an Energy Cost metric in R3-234752. In this contribution, we additionally provide our views on the description of the Energy Cost on a node level as opposed to a delta value.

# 2	Discussion

We agree with the understanding of SA5 on the problem that the rule to convert the actual energy consumption of the NG-RAN nodes into the Energy Cost is provisioned via OAM into the NG-RAN nodes and is unified across the Operator’s network. However, we would like to further clarify our views on the matter.

Considering how to map energy consumption to an Energy Cost through a rule based on a single conversion factor is too rigid and does not account for situations where conditions related to energy consumption may vary, e.g. during the day. We therefore believe that operator could configure to NG-RAN nodes a condition or a set of different conditions and associated weight(s),that should be used by an NG-RAN node to evaluate and measure its energy consumption. In a simple case, a single condition can be configured at an NG-RAN node to map hardware-related energy consumption with a unitary weight, in which case this would amount to an Energy Cost based on pure (hardware-related) energy consumption. In another alternative, multiple conditions can be configured at an NG-RAN node that could be related to hardware energy consumption (static component) as well as to a traffic distribution of UEs with an associated weight (dynamic component). In the second case, the Energy Cost will not only account for the pure hardware-related Energy Consumption, but also on the possible impacts of UE Traffic to a node depending on whether it is at the cell-border or cell-center.

Proposal 1: The rule to convert energy consumption of an NG-RAN node into an Energy Cost can be scaled differently depending on conditions that are satisfied at an NG-RAN node.

As a simple example, a conversion rule involving more than one condition could take into account the type of power supply used to power the node. But the enhanced flexibility can also help to take into account other conditions like traffic distribution and constraints linked to load balancing. Therefore we think that the Energy Cost should correspond to a normalized Energy Consumption. We provide a Text Proposal to BL CR to TS 38.423 in the Annex to reflect an update in the Energy Cost semantics.

Proposal 2: Discuss and agree the TP to BL CR to TS 38.423 in the Annex.

## 2.1 Addressing the questions from SA5

Next, we proceed to provide answers to the questions for clarification raised by SA5.

- Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other?

The unified mapping rule should be configured to the nodes by the operator. As mentioned above, the operator may define a rule involving more than one scaling factor, each corresponding to a given condition (or set of conditions). Each condition or set of conditions can be associated with an operator-configured weight, that allows to scale accordingly the energy consumption at an NG-RAN node when a condition is experienced by the node.

Therefore, our proposed answer is the following:

Answer 1: The unified mapping rule should be defined by the operator, who may define a rule involving more than one scaling factor, each of which corresponds to a given condition or set of conditions satisfied at the NG-RAN node.

- Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)?

To our understanding Energy Cost should be based on an energy consumption measurement (in kWh) made over a period of time. A possible measurement of the Energy Cost based on instantaneous power would have large oscillations, which would make it hard to utilize it for Energy Saving purposes. Thus, we think that energy consumption measurements should be collected over a period of time, which will help to smoothen the measurement information.

But the Energy Cost should be unitless. Therefore, we target an averaged power consumption without time unit configured by operator. Time duration for the average can depend on the implementation. Our proposed answer is the following:

Answer 2: Energy Cost calculation should be based on a unitless average. The time duration over which the average is calculated is up to implementation.

- Should the Energy Cost be calculated in near-real time or non-real time (see above)?

In our view, we do not expect SA5 specification to address this aspect, as discussed also in question 2. Therefore, our proposed answer is the following:

Answer 3: We do not anticipate SA5 specification to address this aspect. Also the terms “near-real time” and “non-real time” do not seem relevant for an average cost.

- What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

In case of split architecture, we think that the normalization of the Energy Cost and encoding needs to be performed in the gNB-DU. Since gNB-DU is the location where the actual Energy Consumption can be measured, providing normalization rules in the gNB-DU allows higher flexibility. The gNB-CU can receive individual Energy Costs reported by the different gNB-DUs it manages and can sum up the individual Energy Costs to obtain the total NG-RAN node Energy Cost.

Answer 4: In case of split architecture, normalization of the Energy Cost and encoding needs to be performed in the gNB-DU. The gNB-CU receives individual Energy Costs reported by the different gNB-DUs it manages and sums up the individual Energy Costs to obtain the total NG-RAN node Energy Cost.

Therefore we propose the following:

Proposal 3: Send a reply LS to SA5, addressing the questions for clarification as follows:

- Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other?

Answer 1: The unified mapping rule should be defined by the operator, who may define a rule involving more than one scaling factor, each of which corresponds to a given condition or set of conditions satisfied at the NG-RAN node.

- Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)?

Answer 2: Energy Cost calculation should be based on a unitless average. The time duration over which the average is calculated is up to implementation.

- Should the Energy Cost be calculated in near-real time or non-real time (see above)?

Answer 3: We do not anticipate SA5 specification to address this aspect. Also the terms “near-real time” and “non-real time” do not seem relevant for an average cost.

- What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

Answer 4: In case of split architecture, normalization of the Energy Cost and encoding needs to be performed in the gNB-DU. The gNB-CU receives individual Energy Costs reported by the different gNB-DUs it manages and sums up the individual Energy Costs to obtain the total NG-RAN node Energy Cost.

## 2.2	Energy Cost: Overall Node Level versus Delta Reporting

The Energy Cost measurement should aim to determine a cost with respect to energy originating by a certain offloading action. There are two options to enable an NG-RAN node 1 determine what is the Energy Cost at a neighbouring node NG-RAN node 2 corresponding to its offloading action to the neighbour.

- Option 1: NG-RAN node 1 may ask NG-RAN node 2 to report Energy Cost before and after an offloading action and NG-RAN node 1 will take the difference of the two to determine the corresponding Energy Cost of  the offloading.

- Option 2: NG-RAN node 1 may request Energy Cost reporting from NG-RAN node 2. NG-RAN node 2 may subsequently calculate the delta increase of the Energy Cost corresponding to the offloaded traffic by NG-RAN node 1 by comparing the Energy Cost before and after the offloading action and may send the (delta) difference to NG-RAN node 1.

Option 1 would be accurate only when a single offloading action takes place at a given time. In case of concurrent offloading actions, NG-RAN node 1 will not be able to determine if the reported Energy Cost values by a neighbouring NG-RAN node 2 correspond only to offloading actions of NG-RAN node 1 or by actions taken also by other neighbouring nodes.

Observation 1: If an NG-RAN node 2 reports a total node level Energy Cost, requested by a neighbouring NG-RAN node 1, the latter cannot know if the reported Energy Cost value corresponds only to offloading actions of NG-RAN node 1 or also to offloading actions by other neighbouring NG-RAN nodes.

For Option 1 to become meaningful, the reporting node NG-RAN node 2 would also need to indicate together with the reported Energy Cost to NG-RAN node 1, additional information about other concurrent offloadings that could possibly pollute the reported Energy Cost measurement, which could even further complexify the solution in case Energy Cost is reported periodically.

Observation 2: If an NG-RAN node reports the total node level Energy Cost to a neighbour, it would also need to provide information indicating whether the reported Energy Cost is polluted through offloading actions of other neighbouring nodes.

On the other hand, option 2 is simpler since it allows NG-RAN node 2 to isolate and report only the proportion of the Energy Cost that corresponds to an offloading action of the requesting NG-RAN node. For the same reason, the reported value may more accurately reflect impacts of a certain offloading to the reported Energy Cost value and requires less signalling since it is up to the target node receiving the offloaded traffic to calculate the difference.

Proposal 4: When a node is requested to provide an Energy Cost corresponding to an offloading action, it calculates the delta increase of the Energy Cost corresponding to the offloaded traffic and reports this information to the requesting node.

If on the other hand, companies prefer to exchange the total node level Energy Cost value, then we need to define mechanisms to reflect if the reported node level Energy Cost information is impacted due to offloading actions by other NG-RAN nodes and if so, to identify the amount of this impact.

Proposal 5: If companies prefer to exchange the total Energy Cost on a node level as opposed to a delta Energy Cost corresponding to an offloading action, we need to identify mechanisms to reflect the amount by which the reported Energy Cost information is polluted by offloading actions of other NG-RAN nodes, not related to the original offloading.

# 3	Conclusion

In this contribution, we make the following observations and proposals:

Proposal 1: The rule to convert energy consumption of an NG-RAN node into an Energy Cost can be scaled differently depending on conditions that are satisfied at an NG-RAN node.

Proposal 2: Discuss and agree the TP to BL CR to TS 38.423 in the Annex.

Proposal 3: Send a reply LS to SA5, addressing the questions for clarification, as follows:

- Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other?

Answer 1: The unified mapping rule should be defined by the operator, who may define a rule involving more than one scaling factor, each of which corresponds to a given condition or set of conditions satisfied at the NG-RAN node.

- Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)?

Answer 2: Energy Cost calculation should be based on a unitless average. The time duration over which the average is calculated is up to implementation.

- Should the Energy Cost be calculated in near-real time or non-real time (see above)?

Answer 3: We do not anticipate SA5 specification to address this aspect. Also the terms “near-real time” and “non-real time” do not seem relevant for an average cost.

- What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

Answer 4: In case of split architecture, normalization of the Energy Cost and encoding needs to be performed in the gNB-DU. The gNB-CU receives individual Energy Costs reported by the different gNB-DUs it manages and sums up the individual Energy Costs to obtain the total NG-RAN node Energy Cost.

Observation 1: If an NG-RAN node 2 reports a total node level Energy Cost, requested by a neighbouring NG-RAN node 1, the latter cannot know if the reported Energy Cost value corresponds only to offloading actions of NG-RAN node 1 or also to offloading actions by other neighbouring NG-RAN nodes.

Observation 2: If an NG-RAN node reports the total node level Energy Cost to a neighbour, it would also need to provide information indicating whether the reported Energy Cost is polluted through offloading actions of other neighbouring nodes.

Proposal 4: When a node is requested to provide an Energy Cost corresponding to an offloading action, it calculates the delta increase of the Energy Cost corresponding to the offloaded traffic and reports this information to the requesting node.

Proposal 5: If companies prefer to exchange the total Energy Cost on a node level as opposed to a delta Energy Cost corresponding to an offloading action, we need to identify mechanisms to reflect the amount by which the reported Energy Cost information is polluted by offloading actions of other NG-RAN nodes, not related to the original offloading.

# References

[1] R3-233515, LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[2] S5-235777, Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[3] R3-234786, (BLCR to 38.423) for AI/ML for NG-RAN

# Annex	- TP for TS 38.423

This TP is based on [1].

<<< start of changes >>>

#### 9.1.3.FF	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item(FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| UE Associated Info Result |  | 0..1 |  |  | YES | FFS |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >> UE Assistant Identifier (FFS on the name) | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1 | – |  |
| >>UE Performance | O |  | 9.2.3.Y |  | – |  |
| >>UE trajectory | O |  | FFS |  | – |  |
| Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured normalized Energy Consumption index. Value 0 indicates the minimum normalized measured Energy Consumption and 10000 indicates the maximum normalized measured Energy Consumption. Energy Consumption is measured on a linear scale. | YES | FFS |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

<<< end of changes >>>