3GPP TSG-RAN3 #121-bis	R3-235618

Xiamen, CN  October 9th – October 13th 2023

Title: 	Timing configuration for UE performance feedback reporting

Source: 	Ericsson, InterDigital, Deutshce Telekom

Agenda item:	12.2.2.1

Document Type:	Discussion and Decision

# Introduction

Considering the agreements taken and the discussions marked as “to be continued” at RAN3#121, in this paper, we further discuss and bring TPs for XnAP Baseline CR for UE performance feedback reporting duration and UE performance feedback periodic reporting.

# Discussion

UE performance feedback reporting duration

For the reporting duration of UE performance feedback, the following agreement and discussion “to be continued” were captured.

A new IE is introduced in the Data Collection Request, indicating the Reporting Duration for the UE Performance Feedback, starting at successful Handover Execution. Any UE Performance Feedback reporting should occur no later than the expiration of the Reporting Duration.

If the UE moves to Idle/Inactive or it hands over to another cell during the Reporting Duration, collection of UE Performance Feedback is terminated. Whether an explicit or implicit indication on the latter events or any other events is signalled to the requesting node needs further discussions.

Focusing first on the discussion to be continued above, we acknowledge that collection of UE Performance Feedback needs to be terminated once the UE moves to RRC_Idle or RRC_Inactive or if the UE hands over to a different cell. However, such termination conditions do not need to be explicitly indicated in the signalling, but can be described as “exit conditions” in the procedure text.

One reason not to require an explicit indication of UE Performance Feedback termination due to mobility to a different node is that it is possible for a source NG-RAN node to request the UE trajectory from the target NG-RAN node. With that it would be possible already to see if and when the UE performs mobility to a different cell. Therefore adding an explicit UE Performance Feedback terminating condition due to the occurrence of a hand over would mean to duplicate a piece of information already existing in the signalling.

Proposal 1: Agree that in case were UE performance feedback collection is interrupted before the expiration of the reporting duration due to UE RRC state change or due to mobility, no explicit signaling of those events to the requesting node is needed.

In addition to the discussion above, at the last RAN3 meeting, some companies proposed the possibility to use a single IE in the DATA COLLECTION REQUEST message in order to configure both, the duration of the UE performance feedback reporting and the duration of the measured UE trajectory reporting.

With respect to such proposals, we would like to stress out that the time scales for UE performance feedback reporting duration and measured UE trajectory reporting duration have a totally different order of magnitude. A plausible reporting duration for the UE performance feedback could be up to a few hundreds of milli seconds or some seconds. This time duration should be enough to collect and report the performance of the UE once it moves to the target NG-RAN node. However, the reporting of the measured UE trajectory should span across several minutes or more, in order to be able to monitor mobility events affecting the UE within the target NG-RAN node.

Thus, if a gNB wanted to use the same DATA COLLECTION REQUEST message to simultaneously configure the reporting of the UE performance feedback and the reporting of the measured UE trajectory, it would be very constraining, if not impossible, for the requesting node to define a single value for the reporting duration which is suitable for both reports.

We also note that the semantics of these two IEs, the UE Performance Feedback reporting duration and the time window for reporting the UE Trajectory, are totally different and uncorrelated. The only thing the two IEs have in common is that the time they report is measured starting from the HO execution. However, the first IE measures a time window during which UE Performance Feedback is measured, while the UE is within the same cell; the second IE measures a time window during which mobility events are recorded, while the UE is within the same target node. Therefore, reducing these two IEs to one seems inappropriate.

Proposal 2: Agree that the new IE introduced in the DATA COLLECTION REQUEST message for the configuration of the reporting duration for the UE performance feedback is used exclusively for this purpose.

Periodic reporting of UE performance feedback

For the periodic UE performance feedback reporting, the following agreement was made at RAN3#121:

- Periodic and one time UE Performance Feedback Reporting are supported.

To mirror this agreement in the specifications, two options are possible:

Option A: use the existing Reporting Periodicity,

Option B: add a new IE for the UE performance feedback reporting periodicity.

We believe that Option A is sufficient for the defined purpose and that there is no need to add further optimization by defining a new IE for the reporting periodicity of the UE performance feedback reporting. One reason for this is that, if a new IE, representing the reporting periodicity of the UE Performance Feedback, is defined and is present in the same DATA COLLECTION REQUEST message together with the existing Reporting Periodicity, there is the possibility of generating two sequences of DATA COLLECTION UPDATE messages, each following different periodicities. We believe that this increases considerably signalling load and complexity for the solution.

Proposal 3: For the reporting periodicity of the UE performance feedback periodic reporting, we propose to use the existing Reporting Periodicity IE in the DATA COLLECTION REQUEST message. Absence of the Reporting Periodicity implies that UE Performance Feedback is collected only once.

# Conclusion

In this paper we discuss the Reporting Duration and the Reporting Periodicity for the UE performance feedback and we make the following proposals:

Proposal 1: Agree that in case were UE performance feedback collection is interrupted before the expiration of the reporting duration due to UE RRC state change or due to mobility, no explicit signaling of those events to the requesting node is needed.

Proposal 2: Agree that the new IE introduced in the DATA COLLECTION REQUEST message for the configuration of the reporting duration for the UE performance feedback is used exclusively for this purpose.

Proposal 3: For the reporting periodicity of the UE performance feedback periodic reporting, we propose to use the existing Reporting Periodicity IE in the DATA COLLECTION REQUEST message. Absence of the Reporting Periodicity implies that UE Performance Feedback is collected only once.

A TP mirroring the above proposals is available in R3-235619