3GPP TSG-RAN WG3 #121-bis	R3-235621

Xiamen, 9 – 13 Oct. 2023

Agenda Item:	14.2. Signaling Support for L1/L2 based Inter-Cell Mobility

Source:	NTT DOCOMO INC.

Title:	Further discussion on LTM

Document for:	Discussion, agreement

Introduction

In the previous meeting, RAN3#121, many agreements were achieved, however, I’m raising some of them related to our proposals in this contribution.

TCI state configuration

The gNB-CU uses the UE Context Setup procedure to collect the TCI state configurations of candidate cells in LTM configuration phase for inter-DU LTM.

After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE.

FFS on the need of an explicit request from CU

Selected beam transfer

The LTM Cell Change Notification procedure is reused to transfer the selected beam from the source gNB-DU to the target gNB-DU via gNB-CU.

FFS for the detail of selected beam information.

FFS on the procedure from CU to target DU to inform the selected beam information and target cell ID.

Option 1: Two new class 2 procedures with different name.

Option 2: same new class 2 procedure

Option 3: class 1 procedure.

Down selection among those 3 options  at next meeting.

TA acquisition

Source DU is in charge of checking the TA validity.

S-DU indicates the valid TA values to target -DU after LTM for subsequent LTM

Reference Configuration

Prioritize the reference configuration on F1 at next meeting.

Discussion

Execution of inter-DU LTM cell switch

RAN3 has considered two approaches [1] to the cell switch execution phase in inter-DU LTM:

Approach 1:   the serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch.

Approach 2:   the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE.

In the reply LS from RAN2 [2], they stated that Approach 1 is the baseline and the signaling of Approach 2 is possible depending on the NW implementation. This aligns with RAN3's understanding, so for stage2/3 CR, RAN3 can agree to this.

Proposal 1:	Approach 1 shall be the baseline, while the support of Approach 2 could be up to the network implementation without any specification impact.

TCI state configuration

According to the LS from RAN1 [3], they agreed to make the TCI state index in the cell switch mandatory. On the other hand, it is still FFS for the behavior of the UE in RACH-based HO.

- A field to indicate 1 joint or 1 pair of UL and DL unified TCI State index for the target cell field is always present in the cell switch command.

- FFS UE behaviour for the beam indication field for the RACH-based handover scenario after cell switch command

From the RAN3 perspective, this is a matter of explicitly indication for the TCI state configuration request (same as PRACH resources) in step 3 of the procedure in the TP [4] agreed in the previous RAN3 meeting. Considering the above agreements, it could be beneficial to make it mandatory for the CU to request TCI state configuration from candidate DU(s) in LTM preparation, even in the case of RACH-based HO. However, we prefer to wait for RAN1 decision for the FFS.

Proposal 2:	In Inter-LTM preparation, we can wait for the RAN1 decision on whether the CU explicitly requests the TCI state configuration.

Selected beam transfer

After the cell switch of the inter-DU LTM, the target DU needs to know the target cell ID and target TCI state configuration selected by the source DU. The TCI state configuration associated with the candidate cell corresponding to the target DU is generated by itself and configured to the UE via the CU. Since the TCI state ID is uniquely assigned among the candidate cells in the candidate DU, it is sufficient for the target DU to know only the TCI state ID selected by the source DU.

Proposal 3:	The source DU notifies the target DU via the CU of the selected TCI state ID.

In the previous meeting, the following three options were raised as procedures to notify the selected beam and target cell ID from the source DU to the CU and from the CU to the target DU.

Option 1:   	Two new class 2 procedures with different name.

Option 2:   	same new class 2 procedure

Option 3:   	class 1 procedure.

In LTM cell switch, there is currently no need to signal beam-related information from CU to target DU other than when the source DU notifies the selected target beam information. Therefore, there is no need to separate these procedures, i.e., Option 2.

Proposal 4:	The CU sends the selected cell ID and beam information to the target DU in the same procedure as the LTM CELL CHANGE NOTIFICATION.

TA management

TA validity is managed by the source DU, however in a subsequent LTM, the source DU is changing every execution of cell switch. Therefore, the source DU needs to transfer the valid TA values to the target DU. There are two possible timing for signaling:

Option 1: Right after cell switch trigger (i.e., LTM Cell Change Notification)

Option 2: After the UE establishes a connection to the target cell

Both options are feasible. However in Option 2, after the cell switch is completed, either the UE context modification procedure is reused, a new TA Information Notify is reused, or a new procedure is introduced. Since either of these options may result in unnecessary round trips, thus Option 1 is more straight-forward.

Proposal 5:	Source DU forwards valid TA values to target DU via CU with LTM CELL CHANGE NOTIFICATION.

Reference configuration

The structure of the reference configuration depends on RAN2, however which node generates it depends on the RAN3 perspective. Which node generates it has an impact on the load to generate the configuration on the node, but does not have a significant impact on the signaling overhead. This is because it does not make a big difference between that the full configuration is sent in F1 signaling and that the reference configuration and delta configuration are sent separately. The procedure and analysis are described below:

- When DU generates reference config:

DU generates reference config and delta config and sends them to the CU in UE Context Setup/Modification Response. The CU sends the reference config and delta config received from the DU to the UE as RRC Configuration. In inter-DU LTM, when the source DU generates the reference config and the candidate DUs generate the delta config, or when the candidate DU generates the reference config and the source DU and other candidate DUs generates the delta config (i.e. part of the candidate cell belongs to the source DU), the reference config needs to be sent through the CU to the candidate DUs and/or source DU on F1 interface. A load on the CU to generate config is small, a load on the DUs to generate config are larger.

- When CU generates reference config:

One way is for all DUs to send the full configuration to the CU in the UE Context Setup/Modification Response, and the CU generates the reference config and delta config from the received config. In another way, the CU first generates the reference config (e.g. based on the serving cell config) and sends it to the DUs in the UE Context Setup/Modification Request, and the DUs send the delta config in the Response message. A load on the CU to generate config is large, a load on the DU to generate config is small.

Observation 1:	Which node generates the reference configuration affects the load on the NW node:

        - When DU generates reference config: CU's load to generate config is smaller, DU's load to generate config is larger.

        - When CU generates reference config: CU's load to generate config is larger, DU's load to generate config is smaller.

Observation 2:	Signaling overhead does not much depending on which node generates the reference configuration.

Proposal 6:	RAN3 should support a method to generate reference configurations in either of the following ways:

Option 1: 	The DU generates reference config and delta config, and sends them to the CU. The CU combines those configurations and generates the RRCReconfiguration to be configured to the UE.

Option 2: 	The DU(s) send the full configuration to the CU, and the CU generates the reference config and delta config based on the received config.

Considering the computing power to generate reference configurations, we think it is better that the CU performs such tasks to avoid unnecessary load on the DUs, so we prefer a way to generate reference and delta configurations at the CUs, i.e., option 2.

Proposal 7:	RAN3 should support option 2 to generate reference configuration using legacy F1 messages (e.g. UE Context Setup/Modification messages).

Conclusions and proposals

Our observations and proposals are summarized below.

Proposal 1:	Approach 1 shall be the baseline, while the support of Approach 2 could be up to the network implementation without any specification impact.

Proposal 2:	In Inter-LTM preparation, we can wait for the RAN1 decision on whether the CU explicitly requests the TCI state configuration.

Proposal 3:	The source DU notifies the target DU via the CU of the selected TCI state ID.

Proposal 4:	The CU sends the selected cell ID and beam information to the target DU in the same procedure as the LTM CELL CHANGE NOTIFICATION.

Proposal 5:	Source DU forwards valid TA values to target DU via CU with LTM CELL CHANGE NOTIFICATION.

Observation 1:	Which node generates the reference configuration affects the load on the NW node:

        - When DU generates reference config: CU's load to generate config is smaller, DU's load to generate config is larger.

        - When CU generates reference config: CU's load to generate config is larger, DU's load to generate config is smaller.

Observation 2:	Signaling overhead does not much depending on which node generates the reference configuration.

Proposal 6:	RAN3 should support a method to generate reference configurations in either of the following ways:

Option 1: 	The DU generates reference config and delta config, and sends them to the CU. The CU combines those configurations and generates the RRCReconfiguration to be configured to the UE.

Option 2: 	The DU(s) send the full configuration to the CU, and the CU generates the reference config and delta config based on the received config.

Proposal 7:	RAN3 should support option 2 to generate reference configuration using legacy F1 messages (e.g. UE Context Setup/Modification messages).

References

- R3-230889, “LS on Approaches during execution for inter-DU LTM (to RAN1)” .

- R3-235014, “Reply LS on Approaches during execution for inter-DU LTM”.

- R3-235008, “LS on L1 measurement and TA management for LTM”.

- R3-234758, “(TP for L1L2Mob BLCR for TS 38.401): Inter-DU LTM procedure update”.