# 3GPP TSG-RAN WG3 Meeting #121bis	R3-235626

# Xiamen, China, Oct.9th - Oct.13th, 2023

Agenda Item:	23.2.3

Source:	ZTE

Title:	[TP for 38.455 & 38.473 BLCR] Discussion on impacts of CPP and Band Aggregation

Document for:	Discussions & Approval

1	Introduction

In the last meeting, companies discussed the carrier phase positioning, SRS band aggregation, and UL ROTA measurement based on RAN4’s progress. And there still some FFS exists for these features.

Currently, RAN3 also receive the other groups’ progress on these features. In this contribution, we provide our views on these R18 positioning features, and corresponding TP for 38.455 and 38.473.

2	Discussion

2.1 Carrier phase positioning

Upon checking the reply LS from RAN1, RAN1 informs SA2 that RAN1 has continued that the discussion on the simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU, and reached the corresponding agreements. In the last meeting, RAN3 has initially agree that LMF request the serving gNB with the indicated time widow configurations, enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the  UL SRS resources from the UE within indicated time window(s), and also support TRP to report UL RSCP in the measurement result to LMF.

Among the latest agreement in RAN1, it has RAN3’s specification impacts as yellow-highlighted part.

RAN1#114 Agreements: Agreement Confirm the following working assumption with modification made in RAN1#113: Working assumption To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of system frame number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement When a LMF requests the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), - the duration of a time window can be configured by one of the following values: - {1, 2, 4, 8, 12} OFDM symbols - {1, 2, 4, 6, 8, 12, 16} slots - FFS: additional values - the number of the time windows can be configured as: - {1, 2, …, 16} Agreement When a LMF requests the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s): - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2, …, 16} Agreement When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2} - FFS: {4, 8}

From RAN3 perspective, with respect to the time window information for SRS configuration, each of the time windows should include the start time, duration, and optional periodicity. The max number of time window is 16. In light of this progress in RAN1, it is proposed to introduce the start time, time duration, and optional periodicity IE in the Time Window Information for SRS IE in the POSITIONING INFORMATION REQUEST message.

Observation 1: Accoording to the agreements in RAN1, multiple time windows can be provided from LMF to the gNB.

Proposal 1: Introduce the Time Window Information List IE in the Time Window Information for SRS IE in the POSITIONING INFORMATION REQUEST message over NRPPa and F1AP, including:

- Start time of the time window, including subframe number, slot offset and symbol index

- Duration of the time window, which is the CHOICE type including number of OFDM symbols or slots

- Periodicity of the time window (optional)

With respect to the time window information for measurement, in the last meeting, it is FFS if a Time window Information for Measurement is needed. From our perspective, since there is multiple time window information, it is proposed to use a new IE for Time Window Information for Measurement.

Proposal 2: Introduce the new Time Window Information for Measurement IE in the Measurement Request message, and remover the Editor Note.

Following the RAN1 agreements, it is proposed to introduce the start time, time duration, and periodicity IE in the Time Window Information for Measurement IE in the Measurement REQUEST message.

Proposal 3: Introduce the Time Window Information List IE in the Time Window Information for SRS IE in the POSITIONING INFORMATION REQUEST message over NRPPa and F1AP, including:

- Start time of the time window, including subframe number, slot offset and symbol index

- Duration of the time window, including number of slots

- Periodicity of the time window (optional)

The corresponding TP for 38.455 and 38.473 TP is provide in the Annex-A&B.

2.2 SRS Band Aggregation

In the last meeting, RAN3 agree that LMF requests the serving gNB to provide SRS resource set that can be aggregated, and the serving gNB decides which SRS resource set(s) can be aggregated and provide associated aggregation ID to LMF.

And there are some remaining issues left:

-	It is FFS if the SRS Resource Set are also impacted by SRS Bandwidth aggregation.

-	It is FFS if the Measurement procedure need to indicate explicit SRS aggregation ID.

In reference to the first bullet point, according to RAN1's understanding, SRS bandwidth aggregation solely takes into account the aspects related to positioning SRS resource sets. We suggest that SRS bandwidth aggregation does not have any impact on the SRS Resource Set.

Since in RAN1, there are no explicit agreements that specify that SRS bandwidth aggregation only pertains to the positioning SRS resource set, RAN3 may consider sending a LS to RAN1 for clarification if there are any concerns or uncertainties regarding this matter.

Proposal 4: SRS bandwidth aggregation does not have any impact on the SRS Resource Set. The LS to RAN1 may be needed for clarification if there are any concerns.

With respect to the second bullet, following the agreement from RAN1 below, the LMF can request the TRP to report the UL measurement from aggregated SRS resources.

RAN1 #112: Agreement Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs

Considering the current progress, the Positioning SRS Resource Aggregation ID IE has been introduced as part of the Positioning Resource Set IE. Furthermore, the Positioning Resource Set IE is included in the SRS configuration IE within the MEASUREMENT REQUEST message. This configuration has allowed the LMF to request the gNB for UL positioning measurements based on aggregated SRS resources.

Proposal 5: LMF can request the gNB for UL measurement aggregated SRS resources through the aggregation ID in SRS configuration IE in the MEASUREMENT REQUEST message.

Based on the following agreement made in RAN1, we think it is common understanding that gNB should report whether the measurement is aggregated, and report the measurement of SRS resources are from which two or three of the carriers. That’s because gNBs may have flexibility to only get measurement from a subset of aggregated carriers.

Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources. Send an LS to RAN2 to confirm the feasibility. Agreement For SRS bandwidth aggregation across carriers, support - Single RSRP or RSRPP is reported - FFS: the single RSRP/RSRPP is based on aggregated SRS resources across aggregated carriers The used SRS resource IDs for the aggregated measurement are shared for RSRP/RSRPP and/or timing measurement results Agreement SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which measurement is aggregated

Therefore, we propose two options below on the indication to LMF which measurement is aggregated:

- -	Option 1: Introduce the Aggregation ID IE in the TRP Measurement Result IE.

- -   Option 2: Introduce the Aggregation Information IE in the TRP Measurement Result IE, encoded as BIT, to indicate the SRS resource measurement is based on which one of the following:

- -> 0bit - single-CC measurement

- -> 1bit - two-CC aggregation, where the SRS resource in the current carrier is aggregated with the SRS resource in the first linked carrier

- -> 2bit - two-CC aggregation, where the SRS resource in the current carrier is aggregated with the SRS resource in the second linked carrier in the case when SRS in three carriers are linked

- -> 3bit - three-CC aggregation

- Proposal 6: RAN3 should discuss following option to support gNB reporting to LMF whether/which the measurement is aggregated:

- -	Option 1: Introduce the Aggregation ID IE in the TRP Measurement Result IE.

- -   Option 2: Introduce the Aggregation Information IE in the TRP Measurement Result IE.

- From our perspective, option1 is preferred for limited specification impacts.

Proposal 7: In the NRPPa/F1AP measurement report message, introduce Aggregation ID IE in the TRP Measurement Result IE.

The corresponding TP for 38.455 and 38.473 TP is provide in the Annex-C&D.

3	Conclusion

Following are the observation and proposal:

CPP Positioning:

Observation 1: According to the agreements in RAN1, multiple time windows can be provided from LMF to the gNB.

Proposal 1: Introduce the Time Window Information List IE in the Time Window Information for SRS IE in the POSITIONING INFORMATION REQUEST message over NRPPa and F1AP, including:

- Start time of the time window, including subframe number, slot offset and symbol index

- Duration of the time window, which is the CHOICE type including number of OFDM symbols or slots

- Periodicity of the time window (optional)

Proposal 2: Introduce the new Time Window Information for Measurement IE in the Measurement Request message, and remover the Editor Note.

Proposal 3: Introduce the Time Window Information List IE in the Time Window Information for SRS IE in the POSITIONING INFORMATION REQUEST message over NRPPa and F1AP, including:

- Start time of the time window, including subframe number, slot offset and symbol index

- Duration of the time window, including number of slots

- Periodicity of the time window (optional)

SRS Bandwidth Aggregation:

Proposal 4: SRS bandwidth aggregation does not have any impact on the SRS Resource Set. The LS to RAN1 may be needed for clarification if there are any concerns.

Proposal 5: LMF can request the gNB for UL measurement aggregated SRS resources through the aggregation ID in SRS configuration IE in the MEASUREMENT REQUEST message.

- Proposal 6: RAN3 should discuss following option to support gNB reporting to LMF whether/which the measurement is aggregated:

- -	Option 1: Introduce the Aggregation ID IE in the TRP Measurement Result IE.

- -   Option 2: Introduce the Aggregation Information IE in the TRP Measurement Result IE.

Proposal 7: In the NRPPa/F1AP measurement report message, introduce Aggregation ID IE in the TRP Measurement Result IE.

4	Reference

[1] R3-235009, Reply LS on PRU Procedures, RAN4

5	Annex-A TP for TS 38.455 BLCR for CPP

## Start of modifications

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.70 |  | YES | ignore |
| UE TEG Information Request | O |  | ENUMERATED(onDemand, periodic, stop, …) |  | YES | ignore |
| UE TEG Reporting Periodicity | C-ifUeTegInfoReqPeriodic |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | YES | reject |
| Time Window Information for SRS | O |  | 9.2.x1 |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifUeTegInfoReqPeriodic | This IE shall be present if the UE TEG Information Request IE is set to the value "periodic". |

## next modification

9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information for Measurement | O |  | 9.2.x2 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

## next modification

9.2.x1	Time Window Information forSRS

This IE contains the time window(s) when UL SRS transmission is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time window Information List |  | 1 |  |  |
| >Time window Information Item |  | 1..<maxnoofTimeWindows> |  |  |
| >> System Frame Number | M |  | INTEGER(0..1023) |  |
| >> Slot Offset | M |  | INTEGER(0..32) |  |
| >> Symbol index | M |  | INTEGER(0..13) |  |
| >> Duration | M |  | 9.2.y1 Duration of Time window |  |
| >> PeriodicitySRS | O |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds |

| Range bound | Explanation |
| --- | --- |
| maxnoofTimeWindows | Maximum no. of Time Windows. Value is 16 |

9.2.x2	Time Window Information of Measurement

This IE contains the time window(s) when UL SRS measurement is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time window Information List |  | 1 |  |  |
| >Time window Information Item |  | 1..<maxnoofTimeWindows> |  |  |
| >> System Frame Number | M |  | INTEGER(0..1023) |  |
| >> Slot Offset | M |  | INTEGER(0..32) |  |
| >> Symbol index | M |  | INTEGER(0..13) |  |
| >> Duration | M |  | 9.2.y1 Duration of Time window |  |
| >> Measurement Periodicity | O |  | ENUMERATED (160ms, 320ms, 640ms, 5120ms, 1280ms, 2560ms, 10240ms, 20480ms, 40960ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTimeWindows | Maximum no. of Time Windows. Value is 16 |

9.2.y1	Duration of Time Window

This IE contains the duration of the time window.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Duration of Time window Configuration |  |  |  |  |
| >Number of Symbols | M |  | ENUMERATED(1,2,4,8,12) |  |
| >Number of Slots | M |  | ENUMERATED(1,2,4, 6,8,12,16) |  |

## End of modifications

6	Annex-B TP for TS 38.473 BLCR for CPP

## Start of modifications

9.2.12.13	POSITIONING INFORMATION REQUEST

This message is sent by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals for uplink positioning measurement and also to retrieve the SRS configuration from the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Requested SRS Transmission Characteristics | O |  | 9.3.1.175 |  | YES | ignore |
| UE Reporting Information | O |  | 9.3.1.255 |  | YES | ignore |
| SRS Positioning INACTIVE Query Indication | O |  | ENUMERATED (true, ...) | Applicable only if the Requested SRS Transmission Characteristics IE is present | YES | ignore |
| Time Window Information for SRS | O |  | 9.3.1.x1 |  | YES | reject |

next modification

9.2.12.3	POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request the gNB-DU to configure a positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.3.1.197 |  |  |  |
| >>Search Window Information | O |  | 9.3.1.204 |  |  |  |
| >>NR CGI | O |  | 9.3.1.12 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.3.1.238 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Positioning Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| Positioning Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, …, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, and 30min are not applicable. | YES | reject |
| Positioning Measurement Quantities |  | 1 |  |  | YES | reject |
| >Positioning Measurement Quantities Item |  | 1..<maxnoofPosMeas> |  |  | EACH |  |
| >>Positioning Measurement Type | M |  | ENUMERATED (gNB RX-TX, UL-SRS-RSRP, UL AoA, UL RTOA, …, Multiple UL AoA, UL SRS-RSRPP, UL-RSCP) |  |  | - |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | TS 38.133 [38] |  |  |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true, ...) | This IE is ignored when the Measurement characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.3.1.242 | This IE is ignored when the Positioning Report Characteristics IE is set to “Periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.3.1.254 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4,…) |  | YES | ignore |
| Positioning Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Positioning Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information for Measurement | O |  | 9.3.1.x2 |  | YES | ignore |

9.3.1.x1	Time Window Information for SRS

This IE contains the time window(s) when UL SRS transmission is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time window Information List |  | 1 |  |  |
| >Time window Information Item |  | 1..<maxnoofTimeWindows> |  |  |
| >> System Frame Number | M |  | INTEGER(0..1023) |  |
| >> Slot Offset | M |  | INTEGER(0..32) |  |
| >> Symbol index | M |  | INTEGER(0..13) |  |
| >> Duration | M |  | 9.3.1.y1 Duration of Time window |  |
| >> PeriodicitySRS | O |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds |

| Range bound | Explanation |
| --- | --- |
| maxnoofTimeWindows | Maximum no. of Time Windows. Value is 16 |

9.3.1.x2	Time Window Information of Measurement

This IE contains the time window(s) when UL SRS measurement is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time window Information List |  | 1 |  |  |
| >Time window Information Item |  | 1..<maxnoofTimeWindows> |  |  |
| >> System Frame Number | M |  | INTEGER(0..1023) |  |
| >> Slot Offset | M |  | INTEGER(0..32) |  |
| >> Symbol index | M |  | INTEGER(0..13) |  |
| >> Duration | M |  | 9.3.1.y1 Duration of Time window |  |
| >> Measurement Periodicity | O |  | ENUMERATED (160ms, 320ms, 640ms, 5120ms, 1280ms, 2560ms, 10240ms, 20480ms, 40960ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTimeWindows | Maximum no. of Time Windows. Value is 16 |

9.3.1.y1	Duration of Time Window

This IE contains the duration of the time window.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Duration of Time window Configuration |  |  |  |  |
| >Number of Symbols | M |  | ENUMERATED(1,2,4,8,12) |  |
| >Number of Slots | M |  | ENUMERATED(1,2,4, 6,8,12,16) |  |

## End of modifications

7	Annex-B TP for TS 38.455 BLCR for SRS BW Aggregation

## Start of modifications

9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |  |  |
| >CHOICE Measured Results Value | M |  |  |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |  | - |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA | M |  | 9.2.39 |  | - |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |  | - |  |
| >>Z-AoA | M |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA | M |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP | M |  | 9.2.72 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Positioning SRS Resource Aggregation ID | O |  | INTEGER(0..16) |  | YES | ignore |

## next modification

### 9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. |  |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  |  |  |
| CHOICE Bandwidth | M |  |  |  |  |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ...) |  |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  |  |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  |  |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. |  |  |
| >>Periodicity List |  | 0.. 1 |  |  |  |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |  |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds |  |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. |  |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  |  |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  |  |  |
| SSB Information | O |  | 9.2.54 |  |  |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| Bandwidth Aggregation Request Information | O |  | ENUMERATED(true, …) |  | YES | ignore |

## next modification

### 9.2.32	Positioning SRS Resource Set

This information element indicates a positioning SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| Positioning SRS Resource ID List |  | 1..<maxnoSRS-PosResourcePerSet> |  |  |
| >Positioning SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Resource Type | M |  |  |  |
| >periodic |  |  |  |  |
| >>PosperiodicSet | M |  | ENUMERATED(true,…) |  |
| >semi-persistent |  |  |  |  |
| >>Possemi-persistentSet | M |  | ENUMERATED(true,…) |  |
| >aperiodic |  |  |  |  |
| >>SRS Resource Trigger | M |  | INTEGER(1..3) |  |
| Positioning SRS Resource Aggregation ID | O |  | INTEGER(0..16) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

## End of modifications

- Annex-B TP for TS 38.473 BLCR for SRS BW Aggregation

## Start of modifications

9.3.1.166	Positioning Measurement Result

The purpose of this information element is to provide the measurement result(s).

| IE/Group Name |  | Presence |  | Range |  | IE Type and Reference |  | Semantics Description |  | Criticality |  |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Positioning Measured Result Item |  |  |  | 1 .. <maxnoofPosMeas> |  |  |  |  |  | - |  |  |  |  |
| >CHOICE Measured Results Value |  | M |  |  |  |  |  |  |  | - |  |  |  |  |
| >>UL Angle of Arrival |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>UL Angle of Arrival |  | M |  |  |  | 9.3.1.167 |  |  |  | - |  |  |  |  |
| >>UL SRS-RSRP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>UL SRS-RSRP |  | M |  |  |  | INTEGER (0..126) |  |  |  | - |  |  |  |  |
| >>UL RTOA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>UL RTOA |  | M |  |  |  | UL RTOA Measurement 9.3.1.168 |  |  |  | - |  |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>gNB Rx-Tx Time Difference |  | M |  |  |  | 9.3.1.170 |  |  |  | - |  |  |  |  |
| >>Zenith Angle of Arrival Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>Zenith Angle of Arrival Information |  | M |  |  |  | 9.3.1.239 |  |  |  | YES |  |  | reject |  |
| >>Multiple UL AoA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>Multiple UL AoA |  | M |  |  |  | 9.3.1.245 |  |  |  | YES |  |  | reject |  |
| >>UL SRS-RSRPP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>UL SRS-RSRPP |  | M |  |  |  | 9.3.1.246 |  |  |  | YES |  |  | reject |  |
| >Time Stamp |  | M |  |  |  | 9.3.1.171 |  |  |  | - |  |  |  |  |
| >Measurement Quality |  | O |  |  |  | TRP Measurement Quality 9.3.1.172 |  |  |  | - |  |  |  |  |
| >Measurement Beam Information |  | O |  |  |  | 9.3.1.173 |  |  |  | - |  |  |  |  |
| >ARP ID |  | O |  |  |  | 9.3.1.244 |  |  |  | YES |  |  | ignore |  |
| >SRS Resource type |  | O |  |  |  | 9.3.1.247 |  |  |  | YES |  |  | ignore |  |
| >LoS/NLoS Information |  | O |  |  |  | 9.3.1.249 |  |  |  | Yes |  |  | ignore |  |
| >Positioning SRS Resource Aggregation ID |  | O |  |  |  | INTEGER(0..16) |  |  |  |  | YES |  | ignore |  |

## next modification

#### 9.3.1.175	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE for positioning purposes.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of SRS transmissions. | - |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  | - |  |
| CHOICE Bandwidth SRS | M |  |  |  | - |  |
| >FR1 |  |  |  |  | - |  |
| >>FR1 Bandwidth | M |  | ENUMERATED (5, 10, 20, 40, 50, 80, 100, ...) |  | - |  |
| >FR2 |  |  |  |  | - |  |
| >>FR2 Bandwidth | M |  | ENUMERATED (50, 100, 200, 400,…,800,1600, 2000) |  | - |  |
| SRS Resource Set List |  | 0.. 1 |  |  | - |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  | - |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. | - |  |
| >>Periodicity List |  | 0.. 1 |  |  | - |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  | - |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds | - |  |
| >>Spatial Relation Information | O |  | 9.3.1.181 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | - |  |
| >>Pathloss Reference Information | O |  | 9.3.1.201 |  | - |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.3.1.210 |  | YES | ignore |
| SSB Information | O |  | 9.3.1.202 |  | - |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| Bandwidth Aggregation Request Information | O |  | Enumerated(true, ...) |  | YES | ignore |

## next modification

9.3.1.196	Positioning SRS Resource Set

This information element indicates a positioning SRS resource set in the UE for UL SRS transmission.

| IE/Group Name |  | Presence |  | Range |  | IE Type and Reference |  | Semantics Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID |  | M |  |  |  | INTEGER(0..15) |  |  |  |
| Positioning SRS Resource ID List |  |  |  | 1..<maxnoSRS-PosResourcePerSet> |  |  |  |  |  |
| >Positioning SRS Resource ID |  | M |  |  |  | INTEGER (0..63, ...) |  |  |  |
| CHOICE Resource Type |  | M |  |  |  |  |  |  |  |
| >Periodic |  |  |  |  |  |  |  |  |  |
| >>PosperiodicSet |  | M |  |  |  | ENUMERATED(true,…) |  |  |  |
| >Semi-persistent |  |  |  |  |  |  |  |  |  |
| >>Possemi-persistentSet |  | M |  |  |  | ENUMERATED(true,…) |  |  |  |
| >Aperiodic |  |  |  |  |  |  |  |  |  |
| >>SRS Resource Trigger List |  | M |  |  |  | INTEGER(1..3) |  |  |  |
| Positioning SRS Resource Aggregation ID |  | O |  |  |  | INTEGER(0..16) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

## End of modifications