3GPP TSG RAN WG3 121bis	R3-235641

Xiamen, China, 9th -13th October 2023

Title:	Draft reply LS on SA5 questions of Energy Cost Index

Response to:	S5-235031 (R3-233515: LS on AI/ML for NG-RAN Energy Saving Energy Cost index)

Release:	3GPP Rel-18

Work Item:	NR_AIML_NGRAN-Core

Source:	CMCC [to be RAN3]

To:	SA5

Contact Person:

#### Name:	Su Gu

#### E-mail Address:	gusu <at> chinamobile <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN3 thanks SA5 for your LS on AI/ML for NG-RAN Energy Saving Energy Cost index and provides answers to the following questions.

- Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other?

- Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)?

- Should the Energy Cost be calculated in near-real time or non-real time (see above)?

- What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

RAN3 answers:

Q1: The unified mapping rule should be defined by 3GPP SA5, while using this mapping rule, Energy Cost index should be normalized and configured by OAM.

Q2: The calculation of the Energy Cost is based on the average power consumption measurement (in W) over a time duration.

Q3: The Energy Cost we required to collect through Xn interface between different NG-RAN nodes should be calculated in near-real time.

Q4: For split NG-RAN nodes, it is suggested to follow the current solution in TR28.913 chapter 4.3 that the EC of a split-gNB is equal to the sum of the EC of all contained gNB-CU(s), gNB-DU(s), gNB-CU-CP(s) (if any), gNB-CU-UP(s) (if any). As any other NFs, gNB-CU(s), gNB-DU(s), gNB-CU-CP(s) and gNB-CU-UP(s) can be composed of PNFs and/or VNFs.

- Actions:

To SA5

Action: RAN3 respectfully asks SA5 to take above information into account.

.

- Date of Next TSG-RAN WG3 Meetings:

RAN3#122	13 – 17 November 2023	Chicago, US