3GPP TSG-RAN WG3 Meeting #121bis	R3-235644

Xiamen, China, 9 Octorber - 13 Octorber 2023

Agenda item:	15.3

Source:	CMCC

Title:	Multicast Reception in RRC_INACTIVE state

Document for:	Discussion

Introduction

During last meeting, the following conclusions are shown as follows:

RAN3#121: Introduce Multicast CU to DU RRC Information IE in F1AP Multicast Context Setup/Modification Request message. Include PDCP configuration and (multicast specific) mtch-neighbourCell in Multicast CU to DU RRC Information IE. gNB-CU indicates Multicast session status (active, deactivated) to gNB-DU via F1AP Multicast Context Setup/Modification Request messages. Enable providing the (multicast specific) MBS Neighbour cell list in F1AP to the DU. CU/CU-CP makes the final decision on whether to enable/disable “Inactive reception” mode for specific multicast session. An indication should be introduced in F1AP Group Paging message to inform DU that inactive reception is allowed for this MBS session and thereby DU could notify UE via Uu Group Paging message.

In this contribution, we will continue to analyse the multicast reception in RRC_INACTIVE state.

Discussion

F1 impact of PTM configuration delivery

Since RAN2 already agreed two ways of PTM configuration delivery via Uu interface, RAN3 needs investigate the impact of providing PTM configurations on F1AP.

If delivering PTM configuration via RRCRelease message, RAN2 reached the conclusion shown as follow:

When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration.

Hence, CU needs to acquire the PTM configurations of the multicast sessions joined by UE. Three options for CU obtaining the latest PTM configurations are prvoided in last meeting.

Option 1: CU retrieves the PTM configuration from DU via CU initiated Multicast context modification procedure.

Option 2: During active MBS multicast sessions, the DU always ensures that the CU is provided with the latest PTM configuration via a new DU initiated Multicast context modification procedure.

Option 3: CU retrieves the PTM configuration from DU via CU initiated UE context modification procedure.

Regard to Option 1 and Option 3, the common part is that gNB-CU initiates to acquire the PTM configuration via non-UE associated signalling or UE-associated signalling. Specifically, CU will request DU to provide the latest PTM configurations using UE CONTEXT MODIFICATION procedure or MULTICAST CONTEXT SETUP/MODIFICATION procedure. For Option 1, since the PTM configurations of MBS session for all UEs are identical, it is unnecessary to request the same PTM configuration from DU to CU in multiple times by UE-associated procedure. For Option 3, it will be inefficient to request the PTM configuration every time gNB-CU decides to release the UE.

Instead, it seems more appropriate to adopt Option 2. The gNB-DU provides the latest configuration towards the gNB-CU as the update of PTM configuration is not a frequent behavior. Existing multicast procedure could not able to fulfil proactive report by DU. Every time PTM configuration updates, DU will send PTM configuration to CU using a new class1 multicast modification procedure triggered by gNB-DU. Option 1 and Option 3 have drawback that CU does not ensure the PTM configuration in CU is the latest PTM configuration when it decides to release a UE into RRC_INACTIVE state and needs to ask the DU for the latest PTM configuration. The solution could be commonly used in the scenario where CU transfers a large amount of UEs receiving multicast into RRC_INACTIVE state in a short time. The CU only needs to request the PTM configuration one time for multiple UEs. However, every time when a new UE moves in a gNB which provides multicast reception in RRC_INACTIVE, CU will request the DU to retrieve the latest configuration. Thus, we prefer Option 2 which reduces the burden on CU without checking the PTM configuration in DU.

Proposal 1: Support the option 2: DU always ensures that the CU is provided with the latest PTM configuration via a new DU initiated Multicast context modification procedure.

The next issue is the granularity of the final decision made by CU/CU-CP on whether to enable/disable “Inactive reception” mode for specific multicast session. The objective of supporting a large number of UEs to receive multicast in RRC_INACTIVE state fulfils the requirements of Mission Critical Services which mitigates the resource utilization in RRC_CONNECTED state. Since the DU has the knowledge of resources utilization information (e.g., the resources for RRC_CONNECTED UEs receiving the multicast service in a cell are overloaded), CU actually does not aware of that information. From our point, it is beneficial to introduce assistance information from DU to help CU make decision on activation of “Inactive reception” for specific multicast session. The decision made on a per DU level or per cell level is feasible, it depends on the size of the congested multicast service area. For per UE level decision, RAN3 has already agreed to support a per UE per MBS session indication from CN to RAN, so obviously we should follow the previous conclusion and support the decision on per UE level as well.

Proposal 2: Introduce assistance information from DU to help CU make decision on activation of “Inactive reception” for specific multicast session.

Proposal 3: The decision on activation of “Inactive reception” for specific multicast session could be based on per UE level, per cell level and per DU level.

Xn impact of MBS inactive reception

During previous RAN2 meeting, they agreed the following conclusion:

The neighbor cell list mechanism for multicast reception in RRC_INACTIVE may be configured e.g. it can be used by UE to resume RRC connection if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell, in some aspects similar to Rel-17 NCL mechanism in MBS broadcast.

In Rel-17, RAN3 does not support exchange ongoing broadcast service between gNBs. In Rel-18, RAN2 concluded that the neighbour cell list for multicast reception in RRC_INACTIVE may be configured. Thus, RAN3 should follow the agreements from RAN2 and introduce the mechanism for exchanging ongoing multicast service which supports inactive state reception. gNB will send to UE after it obtains the information. The UE could use the information to determine whether the service is ongoing in the neighbour cells. Based on the supporting information by neighbour cells, UE can select a suitable target gNB with the capability for handling same multicast service. Hence, it is proposed to exchange ongoing multicast service supporting inactive state reception between NG-RAN nodes in Xn Setup/Modification procedure. Furthermore, the ongoing broadcast service also could be introduced to exchange over XnAP.

Proposal 4: It is proposed to exchange ongoing multicast service supporting inactive state reception and ongoing broadcast service over XnAP.

Group Paging

During previous RAN2 meeting, they agreed the following conclusion:

Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS. No additional enhancement (with regard to enhancements made for ‘deactivation/temp no data’) is needed specifically for enabling UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon session release. Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE.

From the conclusion, it is shown that rel-18 UEs can stay in RRC_INACTIVE state to continue receiving the multicast service after it receives the new indication per tmgi from the enhanced Uu group paging message, which is upon session activation or data transmission resumed. As TS 23.247 specified, group paging is not only used for session activation, but for session release. If the group paging is for session release, no additional enhancement is needed and UE should be transferred to the RRC_CONNECTED state. It is necessary for NG-RAN to distinguish the behaviour of different functions for CN initiated group paging. A new cause value should be introduced in NGAP MULTICAST GROUP PAGING message, indicating the cause of group paging, for session activation or for session release.

Proposal 5: A new cause value should be introduced in NGAP MULTICAST GROUP PAGING message, indicating the cause of group paging, for session activation or for session release.

If the cause value is for session activation, an indicator should be introduced in NGAP to indicate that if UEs are allowed to stay in RRC_INACTIVE state to receive the multicast. If the indicator is presence in message, NG-RAN node does not send the UE back to connected state. Otherwise, the legacy Uu group paging is performed.

Proposal 6: An indicator should be introduced in NGAP to indicate that if UEs are allowed to stay in RRC_INACTIVE state to receive the multicast.

Conclusions

In this paper, we have discussed the multicast reception in RRC_INACTIVE state. The proposals are listed below:

Proposal 1: Support the option 2: DU always ensures that the CU is provided with the latest PTM configuration via a new DU initiated Multicast context modification procedure.

Proposal 2: Introduce assistance information from DU to help CU make decision on activation of “Inactive reception” for specific multicast session.

Proposal 3: The decision on activation of “Inactive reception” for specific multicast session could be based on per UE level, per cell level and per DU level.

Proposal 4: It is proposed to exchange ongoing multicast service supporting inactive state reception and ongoing broadcast service over XnAP.

Proposal 5: A new cause value should be introduced in NGAP MULTICAST GROUP PAGING message, indicating the cause of group paging, for session activation or for session release.

Proposal 6: An indicator should be introduced in NGAP to indicate that if UEs are allowed to stay in RRC_INACTIVE state to receive the multicast.

Reference

- TS 23.247, Architectural enhancements for 5G multicast-broadcast services;