3GPP TSG-RAN WG3 Meeting #121bis                	                                         R3-235648

Xiamen, China, Oct 9- Oct 13, 2023

Agenda Item:	8.3

Source:	CMCC

Title:	Discussion on user consent for trace reporting

Document for:	Discussion

Introduction

During last meeting, we received a reply LS from SA3[1], which follows up the previous LS from RAN3 on the user consent for trace reporting. In the reply LS, SA3 gives their answers regard to the user consent mechanism. If we trace the discussion back to Rel-17, no agreements are made on the user consent for UE trace reporting and the majority’s view in RAN3 is the existing MDT user consent also for the UE location information. In this contribution, we will provide our understanding on the user consent for trace reporting based on the response from SA3.

Discussion

The user consent for trace reporting has been discussed since Rel-16. A previous LS is triggered from SA3, which is noted as follows:

SA3 opines that RAN2, RAN3, and SA5 do not need to make user consent mandatory for RLF/CEF cases but should provide a possibility so that the operator has an option to collect and handle user consent. SA3 also believes it is not required to update previous releases (R15 and prior).

There was a lot of discussions regard to user consent for trace reporting between RAN2/3 and SA3. The lastest LS from SA3 is received in last meeting, providing the replies to two questions:

Q1: Whether user consent should be used to allow/disallow transfer of information from RAN to Trace Collection Entity (TCE), or whether it should also be used to allow/disallow collection of information over the air interface for RAN internal use only. Q2: To provide feedback on feasibility and benefit of a Rel-18 user consent mechanism where an operator can provision, via OAM, which information is subject to user consent, depending on the law and regulations in place. A1: The existing user consent mechanism is only intended for internal use within the 3GPP operators (controllers) domain for collection MDT measurements at the RAN and reporting them to the Trace Collection Entity. A2: Further, user consent is given to the operator so that the 3GPP system can be provisioned/configured based on the operator-subscriber agreed permissions stored in UDM to make it feasible for the 3GPP system to comply with local laws and regulations. Whether the RAN needs to check if user consent is required for a specific type of information/data of a subscriber for a particular purpose can be configured by the OAM. Such configuration is done based on local regulations, which is likely to change infrequently. From the UDM, per UE basis, the RAN receives the yes/no information on whether a user has given consent for the information/data configured by the OAM to be used by the RAN for a particular purpose.

Since existing user consent applies to selection of UE for MDT and location information is included as a part of MDT measurements, RAN can collect the location information and report the information to TCE if UE is selected for MDT. In exisiting user consent, UDM stores user consent information of the specific subscriber, and sends to AMF when UE attaches to the network. Then, AMF stores and transfers the user consent information to RAN node. When the management based MDT activation is sent to gNB, gNB shall check the availability of the Management based MDT PLMN List IE before making the UE selection.

Observation 1: Since existing user consent applies to selection of UE for MDT and location information is included as a part of MDT measurements, RAN can collect the location information and report the information to TCE if UE is selected for MDT.

Followed by the SA3’s answers, OAM totally determines which information/data of a subscriber for a particular purpose is subject to user consent depending on local regulations. In general, operators may control different jurisdictions with different configuration rules. After receiveing the OAM configurations, RAN will know whether to collect the information based on the user consent. If OAM configures RAN does not need to check if the user consent is required for a specific type of information/data of a subscriber for a particular purpose or OAM does not indicate RAN to check, RAN will consider collection of information is not affected by user consent.

Observation 2: OAM totally determines which information/data of a subscriber for a particular purpose is subject to user consent depending on local regulations.

Proposal 1: If OAM configures RAN does not need to check if the user consent is required for a specific type of information/data of a subscriber for a particular purpose or OAM does not indicate RAN to check, RAN will consider collection of information is not affected by user consent.

From our perspective, reusing the existing MDT user consent for acquiring the location information in RLF/CEF report is feasible. If the OAM configures RAN the user consent is required for the collection location information in RLF/CEF report, the main purpose is assumed for user privacy. Compared to location information in MDT which reflects the UE trajectory, the location information in RLF/CEF report indicates an instantaneous position that does not disclose the motion of the UE. The RLF/CEF is only reported when failures happen, so the exposure of location information in RLF/CEF could not be used to track UE and does not bring any privacy issues.

Observation 3: The RLF/CEF is only reported when failures happen, so the exposure of location information in RLF/CEF could not be used to track UE and does not bring any privacy issues.

Proposal 2: Reusing the existing MDT user consent for acquiring the location information in RLF/CEF report is feasible.

Conclusions

In this contribution, we continue the discussion on user consent for trace reporting. The following observations and proposals are listed below:

Observation 1: Since existing user consent applies to selection of UE for MDT and location information is included as a part of MDT measurements, RAN can collect the location information and report the information to TCE if UE is selected for MDT.

Observation 2: OAM totally determines which information/data of a subscriber for a particular purpose is subject to user consent depending on local regulations.

Observation 3: The RLF/CEF is only reported when failures happen, so the exposure of location information in RLF/CEF could not be used to track UE and does not bring any privacy issues.

Proposal 1: If OAM configures RAN does not need to check if the user consent is required for a specific type of information/data of a subscriber for a particular purpose or OAM does not indicate RAN to check, RAN will consider collection of information is not affected by user consent.

Proposal 2: Reusing the existing MDT user consent for acquiring the location information in RLF/CEF report is feasible.

References

- R3-234493, LS on the user consent for trace reporting

- R3-224987, Summary of discussions on CB: # 4_UserConsent