3GPP TSG-RAN WG3 Meeting #121-bis	R3-235654

Xiamen, China,9th – 13th October 2023

Agenda Item:	11.4

Source:	ZTE, China Telecom, CMCC

Title:	Discussion on the other issues of NR QoE enhancement

Document for:	Discussions & Approval

Introduction

This paper provides discussion on the other issues of QoE enhanment in Rel-18.

Discussion

Threshold Trigger

At the RAN3#121 meeting, the following conclusion was captured for the issue of threshold-based RVQoE reporting:

The discussion on Threshold-based triggers for RAN visible QoE is suspend in RAN3 and whether further discussion is needed is pending on progress in RAN2 and SA4.

Same situation is in RAN2, because RAN2 has also decided to wait for RAN3 and SA4 progress.

Now the final conclusion on this issue seems to be relied on SA4 decision. In the latest LS[1] from SA4, they showed negative opinion on definig the threshold-based RVQoE reporting mechanism in the APP layer:

In SA4 reply LS to RAN2 and RAN3 on their LS on buffer level threshold-based RVQoE reporting in S4-231119, SA4 informed that appropriate alternatives for application layer triggering of buffer level threshold-based RVQoE reporting are being considered in SA4. SA4 has not identified any buffer level threshold-based RVQoE reporting mechanism and does not see any value in working further in this direction. SA4 would also like to point to the network assistance feature related to this problem in TS 26.501. RAN2 and RAN3 are requested to take this information in account.

With this LS, it is almost clear that the discussion in threshold-based RVQoE reporting could be closed.

Proposal 1: The threshold-based trigger for RVQoE reporting is not supported in Rel-18.

DU deactivation

During last meeting, there were two camps supporting DU-level and UE-level deactivation of RVQoE reporting over F1 respectively. A compromise was achieved as follows:

DU can deactivate RVQoE reporting over F1 per UE or per DU (which means all the UEs served by DU). The details on the solution, e.g., whether the F1AP procedure for deactivation of RVQoE reporting is UE-associated or non-UE-associated can be further discussed.

In other words, both UE-level and DU-level RVQoE reporting deactivation should be supported over F1. When it comes to the procedure design, we think the non-UE associated procedure with an optional UE list is the best solution. When the UE id list is included in the deactivation message from DU, the CU could deactivate the RVQoE reporting over F1 for these specific UEs. If there is no UE list in the deactivation message, the CU could take it as an indication that the DU wants to deactivate the RVQoE reporting over F1 for all UEs, i.e., in DU level. The procedure could be a class-2 procedure, which does not need the CU to response.

Proposal 2: A non-UE associated class-2 procedure with an optional UE-id list could be defined for the DU to deactivate RVQoE reporting over F1.

Intra 5GC inter-RAT handover

QoE measurement continuity from NR to LTE/5GC and vice versa can be supported by the current Area Scope for QMC IE.

Discuss whether and how a source NR node for handover from NR to LTE/5GC can determine for which service type(s) the UE supports QoE measurements in LTE/5GC.

There is a WA that it should be the source node to select which QoE configuration to keep for UE moves from NR to LTE. However, there is some concern raised by some company that the source node is not aware of the LTE UE capability for QMC support, in which case the source node may select a QMC configuration with the service type that is not supported by the UE. From our point of view, there is no need to make sure the selected QoE configuration is supported in LTE each time. The source node could just select a QoE configuration by implementation and passes to the target node. The target node could decide whether to use the selected configuration or not. If the selected QoE configuration is not supported by the UE in LTE, it could be simply discarded.

Proposal 3: From NR to LTE, the source node selects which QoE configuration to keep by implementation. There is no need for the source node to be aware of the LTE UE capability for QMC.

Assistance information

The assistance information for RAN overload has been pending the SA5 reply previously. At the August meeting, SA5 replied to RAN3 in [2]. The content in the reply is mainly in two aspects which relates to the RAN3 concerns on assistance information:

- Consumer types or characteristics of reporting

It is mentioned in the reply LS that SA5 does not the different of consumers or characteristics of reporting so far. The characteristics such as loop cycle, data size has not been introduced in SA5 and would probably not be defined as assistance information for RAN overload.

- Priority

SA5 has confirmed the feasibility of priority per QoE Reference as assistance information to NG-RAN node.

With the reply from SA5, RAN3 could come to conclusion on this issue, i.e., to introduce priority per QoE Reference as assistance information. The QoE configuration in NGAP and XnAP needs to be updated.

Proposal 4: Introduce priority as assistance information per QoE Reference.

Proposal 5: Send an LS to SA5 to inform that priority is introduced in the QMC configuration per QoE Reference.

A draft LS to SA5 is provided in the Annex.

Conclusions

Proposal 1: The threshold-based trigger for RVQoE reporting is not supported in Rel-18.

Proposal 2: A non-UE associated class-2 procedure with an optional UE-id list could be defined for the DU to deactivate RVQoE reporting over F1.

Proposal 3: From NR to LTE, the source node selects which QoE configuration to keep by implementation. There is no need for the source node to be aware of the LTE UE capability for QMC.

Proposal 4: Introduce priority as assistance information per QoE Reference.

Proposal 5: Send an LS to SA5 to inform that priority is introduced in the QMC configuration per QoE Reference.

References

- R3-235026 Reply LS on buffer level threshold-based RVQoE reporting

- R3-235028 Reply LS on the feasibility of introducing assistance information for handling of QoE reporting during RAN overload

Annex

Title:	LS on the introduction of priority into the QMC configuration

Response to:	-

Release:	Release 18

Work Item:	NR_QoE_enh

Source:	ZTE[to be RAN3]

To:	SA5

Cc:

Contact person:	Man Zhang

zhang.man4@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R3-23xxxx[attach NGAP TP if agreed]

- 1	Overall description

RAN3 thanks SA5 for replying to RAN3 in S5-235542 about questions on assistance information and confirming the feasibility of introducing a priority per QoE configuration, for handling QoE reporting in RAN overload.

After the discussion in RAN3#121bis meeting, RAN3 has taken SA5’s reply into account and made the conclusion that priority could be introduced in the QMC configuration outside the QoE configuration container, in the basis of per QoE Reference. The agreed TP for NGAP is attached as a reference.

- 2	Actions

To RAN3:

ACTION: 	RAN3 would like to ask SA5 to take the above information into account and continue to work on SA5 specifications if needed.

- 3	Dates of next TSG SA WG 5 meetings

RAN3#122		13 - 17 November 2023			Chicago, US

RAN3#123		26 February - 1 March 2024		Athens, Greece