3GPP TSG-RAN WG3 Meeting #121-bis	R3-235655

Xiamen, China,9th – 13th October 2023

Agenda Item:	11.4

Source:	ZTE, Nokia, Nokia Shanghai Bell, China Telecom

Title:	(TPs for BL CR of TS38.401&38.470) Deactivation of RVQoE reporting over F1AP

Document for:	Discussions & Approval

1	Introduction

This paper provides the text proposal to 38.401 and 38.470 based on the discussion paper.

2	Text Proposal for 38.401

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

7.10	Support of RAN visible QoE measurement

The RAN visible QoE measurement function is specified in TS 38.300 [2].

In split gNB architecture, upon the reception of the RAN visible QoE measurement report from the UE, the gNB-CU may forward the corresponding QoE information to the gNB-DU. The QoE information transferred to the gNB-DU may include the RAN visible QoE metrics in the RAN visible QoE measurement result from the UE, along with the corresponding DRB ID(s).

The gNG-DU may deactivate the transmission of RAN visible QoE measurements from the gNB-CU, by sending a deactivation message to the gNB-CU. The deactivation message may include an optional Deactivation Indication List IE, which contains a list of F1AP UE identifiers, in order to allow the deactivation of RVQoE reporting in UE level. Upon receiving the deactivation message, if the UE id list is present, the gNB-CU stops forwarding the RAN visible QoE reports to the specific UEs as indicated by the UE id list. If there is no UE id list received in the deactivation message, the gNB-CU stops forwarding the RAN visible QoE reports to the gNB-DU.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- Text Proposal for 38.470

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6.1.15	QMC procedures

The QMC procedures are listed below:

-	QoE Information Transfer procedure;

-	QoE Information Transfer Deactivation procedure.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>