3GPP TSG-RAN WG3 Meeting #121-bis	R3-235665

Xiamen, China, 9 – 13th October, 2023

Title: 	Further discussions on the user consent for trace reporting

Source: 	Huawei

Agenda item:	8.3

Document Type:	discussion and decision

1. Introduction

RAN3 received further reply LS R3-234493 [1] from SA3 on the user consent for trace reporting, this paper tries to have further discussions on the user consent for trace reporting.

2. Discussion

2.1 Background

This issue triggered by SA3’s request that “SA3 opines that RAN2, RAN3, and SA5 do not need to make user consent mandatory for RLF/CEF cases but should provide a possibility so that the operator has an option to collect and handle user consent if RLF/CEF could be”.

While in the latest reply LS R3-234493 from SA3, SA3 provided feedback to RAN3’s two further questions:

Q1: Whether user consent should be used to allow/disallow transfer of information from RAN to Trace Collection Entity (TCE), or whether it should also be used to allow/disallow collection of information over the air interface for RAN internal use only.

A1: The existing user consent mechanism is only intended for internal use within the 3GPP operators (controllers) domain for collection MDT measurements at the RAN and reporting them to the Trace Collection Entity.

Q2: To provide feedback on feasibility and benefit of a Rel-18 user consent mechanism where an operator can provision, via OAM, which information is subject to user consent, depending on the law and regulations in place.

A2: Further, user consent is given to the operator so that the 3GPP system can be provisioned/configured based on the operator-subscriber agreed permissions stored in UDM to make it feasible for the 3GPP system to comply with local laws and regulations.

Whether the RAN needs to check if user consent is required for a specific type of information/data of a subscriber for a particular purpose can be configured by the OAM. Such configuration is done based on local regulations, which is likely to change infrequently. From the UDM, per UE basis, the RAN receives the yes/no information on whether a user has given consent for the information/data configured by the OAM to be used by the RAN for a particular purpose.

The steps described above include the method detailed in Q2. Hence the method is feasible.

The rest of this paper will discuss if there are any further RAN3 spec impacts according to the latest feedback from SA3.

2.2 Discussion

- As could be seen from the A1, it is a clear understanding that user consent is used to indicate whether a subscriber agrees to perform MDT collection requested by network or not, and in our understanding, this is also in line with RAN3’s understanding.

- Observation 1: SA3 and RAN3 share the same understanding that “existing user consent mechanism is used to indicate whether a subscriber agrees to perform MDT collection request from network/operator or not”.

- As could be seen from the A2, there are two major points:

- Whether the RAN needs to check if user consent is required for a specific type of information/data of a subscriber for a particular purpose can be configured by the OAM. Such configuration is done based on local regulations, which is likely to change infrequently

- The steps described above include the method detailed in Q2. Hence the method is feasible

- With the above two points, it is also quite clear that from SA3 perspective, OAM could configure whether the RAN needs to check if user consent is required for a specific type of information/data of a subscriber for a particular purpose, and the method is feasible.

- Observation 2: SA3 concluded that: “It is feasible that OAM could configure whether the RAN needs to check if user consent is required for a specific type of information/data of a subscriber for a particular purpose”.

- With the above two observations, the further question to RAN3 is, whether RAN3 needs to do additional spec enhancements to support SA3’s conclusion or not.

- Question: whether RAN3 needs to do additional spec enhancements to support SA3’s conclusion or not.

- In order to answer the question above, we need to start from the existing procedures. As we know that OAM uses the Trace procedure to initiate MDT procedure, during which the base station will check the user consent of the selected UE to see if this UE is willing to perform MDT, for management-based MDT task. Here, taking the observation 2 above into account, it is obvious that there is an additional step the base station needs to do, i.e. to check if user consent is required for a specific type of information/data of a subscriber for a particular purpose, which is based on local regulation and is likely to change infrequently.

- To understand above descriptions better, and to take the very original use case about RLF/CEF measurement, if a certain UE indicates its consent as NO, but the local regulation doesn’t require user consent for RLF/CEF collection, the base station just needs to perform additional check if collection of such information could be not affected by the user consent. Here we could name it as template information, and such template information (of no user consent needed for RLF/CEF) should be consistent among the whole network and configured by OAM to the base station. Here it should be noted that the MDT mechanism itself should not be impacted, i.e. the only added behaviour is template information check, if operator doesn’t configure such info, everything sticks to the exiting spec.

- Observation 3: In addition to user consent check, the base station also needs to check the OAM configured template information (whether use consent is needed for a specific type of information/data for MDT purpose, if configured) when selecting UE to perform MDT measurement; meanwhile, the existing MDT mechanism is not impacted.

- With the understanding of Observation 3, it could also be concluded that there will be no direct RAN3 impacts, but maybe some clarification texts needed to further elaborate base station behaviour, such clarification texts could stage 2 or stage 3 or both. Hence, we have the following proposal:

- Proposal: RAN3 to agree observation 3 as a common understanding, and to discuss whether any clarification texts are needed or not.

- In our understanding, since the behaviour of user consent check was already captured in 38.401, some clarification texts could be added to elaborate supplement base station’s behaviour, corresponding CR could be referred to [2], and corresponding reply LS is seen in [3].

3. Conclusion

Based on the discussion in this paper, we propose the following observations and proposals:

- Observation 1: SA3 and RAN3 share the same understanding that “existing user consent mechanism is used to indicate whether a subscriber agrees to perform MDT collection request from network/operator or not”.

- Observation 2: SA3 concluded that: “It is feasible that OAM could configure whether the RAN needs to check if user consent is required for a specific type of information/data of a subscriber for a particular purpose”.

- Observation 3: In addition to user consent check, the base station also needs to check the OAM configured template information (whether use consent is needed for a specific type of information/data for MDT purpose, if configured) when selecting UE to perform MDT measurement; meanwhile, the existing MDT mechanism is not impacted.

- Proposal: RAN3 to agree observation 3 as a common understanding, and to discuss whether any clarification texts are needed or not.

4. Reference

[1]	R3-234493, LS on the user consent for trace reporting, SA3

[2]	R3-235666, Correction to 38.401 on the user consent for trace reporting, Huawei

[3]	R3-235667, [draft] Reply LS on the user consent for trace reporting, Huawei