3GPP TSG-RAN WG3 Meeting #121bis	R3-235685

Xiamen, China, October 09 – 13, 2023

Agenda Item:	25.2

Source:	Ericsson, Orange, Deutsche Telekom, AT&T, Verizon Wireless

Title:	(TP for XR CR for TS 38.415) ECN Marking for L4S

Document for:	Discussion, Decision

# Introduction

This paper contains a TP for the procedure used to exchange information supporting ECN Marking for L4S information.

# TP for TS 38.415

Below, a draft TP is presented where the proposals made in R3-235683 are reflected,

Start of Changes

#### 5.5.2.2	UL PDU SESSION INFORMATION (PDU Type 1)

This frame format is defined to allow the UPF to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective UL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 | 5 |  |  | 4 | 3 |  | 2 |  | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  |  |  | QMP | DL Delay Ind. |  |  | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag |  | QoS Flow Identifier |  |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) |  | New IE Flag 6 |  | New IE Flag 5 |  | New IE Flag 4 | New IE Flag 3 |  | New IE Flag 2 | New IE Flag 1 |  | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  | ECN marking percentage ind. UL |  |  |  |  | ECN marking percentage ind. DL |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| ECN Marking Percentage UL |  |  |  |  |  |  |  |  |  | 0 or 1 |
| ECN Marking Percentage DL |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Padding |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: UL PDU SESSION INFORMATION (PDU Type 1) Format

The New IE Flag in bit 6 of 2nd octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the D1 UL PDCP Delay Result Ind is present (1) or not (0)

Next Change

#### 5.5.3.x1	ECN marking percentage Indication UL

Description: This field indicates the presence of the ECN Marking Percentage UL.

Value range: {0 = ECN Marking Percentage not present, 1= ECN Marking Percentage UL present}.

Field length: 1 bit.

#### 5.5.3.x2	ECN marking percentage Indication DL

Description: This field indicates the presence of the ECN Marking Percentage DL.

Value range: {0 = ECN Marking Percentage DL not present, 1= ECN Marking Percentage DL present}.

Field length: 1 bit.

#### 5.5.3.y	ECN marking percentage UL

Description: This field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked. As an example, value 9574 corresponds to a percentage of 95.74%.

Value range: {0..10000}.

Field length: 1 octet.

#### 5.5.3.z	ECN marking percentage DL

Description: This field indicates the percentage of DL IP packets up to two decimal points that should be ECN marked. As an example, value 9574 corresponds to a percentage of 95.74%.

Value range: {0..10000}.

Field length: 1 octet.

End of Changes