3GPP TSG-RAN WG3 Meeting #121bis	R3-235699

Xiamen, China, 09-13 Oct, 2023

Agenda Item:	8.1

Source:	Huawei

Title:	Summary of offline discussions on CB: # 2_PEIPS

Document for:	Discussion and approval

# 1	Introduction

CB: # 2_PEIPS

- Check TS24.501

- Provide reply LS to CT1 based on group understanding

(moderator - HW)

Summary of offline disc R3-235699.

# 2	For Chairman’s note

Propose the following:

Proposal: Agree the reply LS in R3-235828 (revision of R3-235437)

# 3 Discussion

## 3.1 Background

As discussed contributions and the CT1 LS, two PEI mismatch scenarios are provided below for reference.

Scenario A): the old PEIPS assistance information assigned already

- The AMF has assigned the UE a PEIPS assistance information already;

- The AMF sends a new PEIPS assistance information, and includes it the REGIESTATRION ACCEPT message in subsequent registration procedure;

- But the AMF fails to the reception of REGISTRATION COMPLETE message abnormally

Scenario B): No PEIPS assistance information assigned before

- The AMF has not assigned the UE a PEIPS assistance information before;

- The AMF sends a PEIPS assistance information, and includes it the REGIESTATRION ACCEPT message;

- But the AMF fails to the reception of REGISTRATION COMPLETE message abnormally.

## 3.2 CT1 specification check

During the online discussion, it was required to further check CT1 specification as commented by Samsung. The following provides the excerpt from section 5.5.1.3.8 in TS 24.501.

##### 5.5.1.3.8 Abnormal cases on the network side The following abnormal cases can be identified: a) If a lower layer failure occurs before the message REGISTRATION COMPLETE has been received from the UE and timer T3550 is running, the AMF shall abort the procedure, enter 5GMM-IDLE mode. If a new 5G-GUTI was assigned to the UE in the REGISTRATION ACCEPT message, the AMF shall consider both, the old and new 5G-GUTIs as valid until the old 5G-GUTI can be considered as invalid by the AMF. If a new TAI list was provided in the REGISTRATION ACCEPT message, both the old and new TAI lists shall also be considered valid until the old TAI list can be considered invalid by the AMF. If the old 5G-GUTI was allocated by an AMF other than the current AMF, the current AMF does not need to retain the old 5G-GUTI. Additionally, if the REGISTRATION ACCEPT message includes: 1) Negotiated PEIPS assistance information IE: i) containing a new Paging subgroup ID and the UE is previously assigned a different Paging subgroup ID then, the AMF shall consider both, the old and new Paging subgroup IDs as valid until the old Paging subgroup ID can be considered as invalid by the AMF; or NOTE 0: If the UE was not previously assigned a paging subgroup ID by the AMF, then AMF informs RAN about the new PEIPS information and it is up to RAN how to handle the old information at RAN and new information from AMF so that the paging is not missed. ii) containing no Paging subgroup ID or no Negotiated PEIPS assistance information IE, then the AMF shall delete any old Paging subgroup ID stored in the 5GMM context of the UE. <<<<<<<<<<<<<<<<<<<< Irrelavent Text Omitted >>>>>>>>>>>>>>>>>>>> During this period: <<<<<<<<<<<<<<<<<<<< Irrelevant Text Omitted >>>>>>>>>>>>>>>>>>>> 3) if the UE needs to be paged: i) if in the REGISTRATION ACCEPT message a new Paging subgroup ID in the Negotiated PEIPS assistance information IE is assigned to the UE: - that previously has no Paging subgroup ID assigned then, the AMF shall use the new Paging subgroup ID for paging the UE; - that is same as the old Paging subgroup ID then, the AMF shall use the same Paging subgroup ID for paging the UE; or - that is different than the old Paging subgroup ID then, the AMF may first use the old Paging subgroup ID followed by the new Paging subgroup ID for paging the UE.

As highlighted in “color texts”, it is specified that the AMF may first use the old Paging subgrouping ID in the NGAP paging message, then send the new Paging subgroup ID in the subsequent NGAP paging message.  This is applicable to scenario A in section 3.1

As highlighted in “yellow texts”, it is specified that the AMF shall use the new Paging subgroup ID for paging the UE. This is applicable to scenario B in section 3.1. The moderator understands it can be left to CT1 to consider whether some updates are needed, e.g., the AMF may first send the paging message without paging subgroup ID, then send the paging subgroup ID in the subsequent NGAP paging message.

Proposal 1: For both abnormal scenarios above, leave to AMF implementation (i.e., without NGAP change).

Proposal 2: for scenario B, leave it to CT1 to consider whether there is any issue about the following sentence in TS 24.501

- “-	that previously has no Paging subgroup ID assigned then, the AMF shall use the new Paging subgroup ID for paging the UE;”.

Question 1: Please find your views for the above proposals.

| Company | Comment |
| --- | --- |

Moderator summary:

No further comments are received, see the conclusion in section 2.

## 3.3 Reply LS

The updated reply LS is now in the draft folder, and copied below:

RAN3 thanks to CT1 for their LS on the usage of paging subgrouping information in RAN in case of abnormal scenario. RAN3 discussed the two abnormal scenarios identified by CT1, and concluded that the handling of the abnormal scenarios can be left to the AMF implementation for paging the UE. RAN3 also observed that the AMF behaviour has been specified in section 5.5.1.3.8 of TS 24.501 for paging the UE, as excerpted and highlighted below. RAN3 will leave to CT1 to consider in case of no Paging subgroup ID previously assigned, whether there is need for the AMF first paging the UE without the paging subgroup ID, followed by the new Paging subgroup ID for paging the UE.

Please go directly update the draft reply LS (already in the draft folder) or provide views in the following table.

Question 2: your further views to the draft LS.

| Company | Comment |
| --- | --- |

Moderator summary:

No further comments are received, see the conclusion in section 2.

# 4	References

| Paging Subgrouping |  |  |
| --- | --- | --- |
| R3-235004 | LS on usage of paging subgrouping information in RAN in case of abnormal scenario (CT1(Huawei)) | LS in |
| R3-235435 | The usage of paging subgrouping information in RAN in case of abnormal scenario (Huawei) | discussion |
| R3-235436 | The usage of paging subgrouping information in RAN in case of abnormal scenario (Huawei) | CR1029r, TS 38.413 v17.6.0, Rel-17, Cat. F |
| R3-235628 | The usage of paging subgrouping information in RAN in case of abnormal scenario (Huawei) | CR1100r, TS 38.423 v17.6.0, Rel-17, Cat. F |
| R3-235629 | The usage of paging subgrouping information in RAN in case of abnormal scenario (Huawei) | CR1224r, TS 38.473 v17.6.0, Rel-17, Cat. F |
| R3-235437 | [Draft] Reply LS on the usage of paging subgrouping information in RAN in case of abnormal scenario (Huawei) | LS out To: CT1, SA2 CC: RAN2 |
| R3-235534 | Paging subgrouping in abnormal scenario (Nokia, Nokia Shanghai Bell) | discussion |
| R3-235535 | Correction of Paging subgrouping in abnormal scenario (Nokia, Nokia Shanghai Bell) | CR1031r, TS 38.413 v17.6.0, Rel-18, Cat. F |
| R3-235536 | Reply LS on usage of paging subgrouping information in RAN in case of abnormal scenario (Nokia, Nokia Shanghai Bell) | LS out To: CT1 CC: |
| R3-235577 | Discussion on LS from CT1 on PEIPS usage in case of abnormal scenario (Ericsson) | discussion |
| R3-235578 | [Draft] LS Reply on usage of paging subgrouping information in RAN in case of abnormal scenario (Ericsson) | LS out To: CT1, RAN2 CC: |
| R3-235678 | Discussion on additional PEIPS assistance info supporting (ZTE) | discussion |
| R3-235679 | Correction for F1AP on additional PEIPS assistance information (ZTE) | CR1225r, TS 38.473 v17.6.0, Rel-17, Cat. F |
| R3-235680 | Correction for XnAP on additional PEIPS assistance information (ZTE) | CR1101r, TS 38.423 v17.6.0, Rel-17, Cat. F |
| R3-235681 | Correction for NGAP on additional PEIPS assistance information (ZTE) | CR1032r, TS 38.413 v17.6.0, Rel-17, Cat. F |
| R3-235682 | Correction on additional PEIPS assistance information (ZTE) | draftCR |
| The issue of missing paging when the registration procedure fails due to abnormal scenarios as mentioned by CT1. Whether it can be regarded as corner case? QC: This is the issue to be solved, else there will be mismatch between UE and RAN HW: This is abnormal case will not happen frequently SS: According to TS24.501, 5.5.1.3.8 bullet3, the AMF may first use the old value, it seems CT1 has already had the solution E///: It’s corner case. There are ways of implementation, e.g., AMF implementation or NAS refresh. RAN should be transparent to NAS procedures. ZTE: CT1 sends the LS to both RAN2 and RAN3, any confliction should be avoided Nok: Doubt on the scenario. If we follow TS24.501 section 5.5.1.3.8, we need the CR, needs more time to check the details CATT: It’s the abnormal case. QC: This is the existing issue. CB: # 2_PEIPS - Check TS24.501 - Provide reply LS to CT1 based on group understanding (moderator - HW) Summary of offline disc R3-235699 |  |  |