3GPP TSG-RAN WG3 Meeting #121bis	R3-235827

Xiamen, China, 09-13 Oct, 2023

Title: 	(TP for L1L2Mob BLCR for TS 38.401): LTM stage2 procedure update

Source: 	Huawei, Lenovo, Nokia, Nokia Shanghai Bell, NEC,CATT, Google, ZTE, Ericsson

Agenda item:	14.2

Document Type:	Other

1. Introduction

This document contains a stage 2 TP to capture the agreements in this meeting.

2. TP for LTM BLCR for TS 38.401

---------------------------------------------------------Start of changes -------------------------------------------------------------

#### 8.2.1.X	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU LTM

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate LTM configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing one target candidate cell ID. The gNB-CU requests PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration.

- If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations  (e.g., TCI state configuration, RACH configuration and the RS configuration  ) for the accepted target candidate cell.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- Early synchronization is performed as specified in TS 38.300 [2].

- The UE sends the lower layer measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

- The gNB-DU sends the LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the TCI state ID.

- FFS: How the target gNB-DU detects the UE access.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

- The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

---------------------------------------------------------Next change -------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.Y-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.Y-1: Inter gNB-DU LTM

1. The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2. The gNB-CU determines to initiate LTM configuration.

3. The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing one target candidate cell ID. The gNB-CU indicates the source gNB-DU ID, and requests PRACH resources from the Candidate gNB-DU. The gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration.

4. If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration (e.g., TCI state configuration and RACH configuration) and the RS configuration  for the accepted target candidate cell.

5. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the collected RS configuration, TCI state configuration and RACH configuration for the accepted target candidate cell(s) in other gNB-DUs.

6. The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which may include the RS configuration of the source cell (FFS), prepared candidate cells (FFS) and the generated CSI report configuration (FFS).

7. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing  the cell ID(s) of the prepared candidate cell(s) , associated RS configuration for each candidate cell in other candidate gNB-DU(s). The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

Note: The candidate cell may be the same cell as source cell.

8. The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated low layer configuration in which contains the generated CSI report configuration..

FFS on step 7 and 8 on whether should be parallel or single.

Note: Step 7 and step 8 may be skipped if subsequent LTM is not configured.

9. The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10. The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11. The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12. The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13. Early synchronization is performed as specified in TS 38.300 [2].

14. The candidate gNB-DU sends the TA, the associated CFRA resource information , the candidate cell ID and the source gNB-DU ID to the source gNB-DU via the gNB-CU.

FFS on the message name.

15. The UE sends the lower layer measurement result to the source gNB-DU.

16. The source gNB-DU decides to execute LTM to a candidate target cell.

17. The source gNB-DU sends LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

18. The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the TCI state ID.

19. The gNB-CU sends the target cell ID and the TCI state ID to the target gNB-DU.

FFS on the message to be used.

20. FFS:  how the target gNB-DU detects the UE access.

21. The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

22. The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

23. The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

---------------------------------------------------------next change -------------------------------------------------------------

#### 8.2.1.Z	LTM with gNB-CU-UP change

Figure 8.2.1.z-1 shows the procedure used for LTM with the change of gNB-CU-UP within a gNB.

Figure 8.2.1.z LTM with the change of gNB-CU-UP

0. The source gNB-DU forwards the Measurement Report to the gNB-CU-CP.

1. The gNB-CU-CP decides to initiate LTM configuration.

2. The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the target gNB-CU-UP.

3.	The target gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, DL TNL address information for NG-U, and the TNL address information for data forwarding to the target gNB-CU-UP.

4 . LTM configuration procedures are performed between gNB-CU and candidate gNB-DUs, and between gNB-CU and source gNB-DU as specified from step 3 to step 8 in section 8.2.1.Y.

5 - 6.The gNB-CU-CP sends the RRC Reconfiguration message to the UE.

7.  The UE sends the lower layer measurement result to the source gNB-DU, and the source gNB-DU decides to execute LTM to a candidate target cell.

8. The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU-CP with the selected target cell ID.

9-10. The gNB-CU-CP performs the Bearer Context Modification procedure to retrieve the PDCP UL/DL status and to exchange the TNL address information for data forwarding for the bearers.

11-12.	The gNB-CU-CP performs the Bearer Context Modification procedure to send the DL TNL address information for F1-U and the PDCP UP/DL status to the target gNB-CU-UP.

13.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

14.  The target gNB-DU detects the UE in the target cell.

15. The target gNB-DU sends an ACCESS SUCCESS message to the gNB-CU-CP.

16 - 18. Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

19-20.	Bearer Context Release procedure may be performed to release the UE context in the source gNB-DU.

--------------------------------------------------------- the end of changes -------------------------------------------------------------