3GPP TSG-RAN WG3 Meeting #121bis	R3-235863

Xiamen, China, 9th – 13th October 2023                                         Revision of R3-235415

Agenda Item:	15.2

Source:	CATT, CMCC, CBN, Nokia, Nokia Shanghai Bell, Huawei, Ericsson, ZTE, Samsung, Qualcomm, Lenovo

Title:	(TP for BL CR to 38.401) Support of Broadcast in RAN sharing scenario

Document for:	Discussions & Approval

1	Introduction

This document reflects the following agreement reached in this meeting:

Close the open issue on how to identify identical content for location dependent services by simply stating that the “MBS Service Area is taken into account”

2	Text Proposal for TS 38.401

-----------Start of the First Change-----------------

7.7	Support for NR MBS

The Support of NR MBS in non-split gNB case is specified in TS 38.300 [2].

7.7.1	Support of dynamic PTP and PTM switching

For UEs in RRC_CONNECTED, NG-RAN supports dynamic switch between PTP and PTM for MBS as specified in TS 38.300 [2].

In case of split gNB architecture, for a MRB with common PDCP involving both PTP (RLC leg) and PTM (RLC leg), upon receiving the MBS data from the gNB-CU via a shared F1-U tunnel, the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg) or both.

For UEs in RRC_INACTIVE only PTM is supported as specified in TS 38.300 [2].

7.7.x	Support of resource efficiency for RAN Sharing

7.7.x.1	General

Editor’s Note:	TBD

7.7.x.2	Support of resource efficiency for MOCN

Information enabling identifying broadcast MBS sessions for which RAN resources could be shared by the involved gNB-DUs and the gNB-CU-UP is provided by the gNB-CU-CP. Only one set of shared F1-U resources is established and kept as long as there is one PLMN keeping the Multicast service.

7.7.x.3	Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast

gNB-DUs sharing the same physical cell resources receive via F1-C information enabling identifying broadcast MBS sessions providing identical content. The identification is based on Associated Session ID, for location dependent MBS services, the MBS Service Area is also taken into account.

Applying resource efficiency for RAN Sharing with multiple cell-ID broadcast

-	resolve different QoS requirements received from the participating 5GCs in an implementation specific way.

-	F1-U resources are established towards either all involved gNB-CUs or only some of them which is decided by the entity controlling the involved gNB-DUs sharing the same physical cell resources.

-	the gNB-CU-CP takes into account the decision F1-U resources have been established or not to decide whether to establish NG-U resources.

Editor’s Note:	Whether there is a one-one relationship between one set of F1-U tunnels and one NG-U tunnel is FFS.

7.7.y	Support of Multicast reception for UEs in RRC_INACTIVE

F1AP supports enabling of multicast reception for UEs in RRC_INACTIVE.

The gNB-CU decides whether multicast reception in RRC_INACTIVE is applied.

Editor’s Note:	FFS whether additional assistance information is sent by gNB-DU to gNB-CU to help gNB-CU's decision making.

------------End of the Change-----------------