3GPP TSG-RAN WG3 #121-bis	R3-235903

Xiamen, China, 9th – 13th October 2023

Source:	CATT, ZTE, Lenovo, Nokia, Nokia Shanghai Bell, Ericsson, Samsung

Title:	(TP to BLCR for 38.420) Support of AI/ML for NG-RAN

Agenda Item:	12.2.1

Document for:	Discussion and decision

- Introduction

In previous meeting, it has been agreed to introduce Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to support function of AI/ML for NG-RAN.The contribution captures the function in TS38.420.

- TP for TS 38.420

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

AI: as defined in TS 38.300 [8].

Boundary IAB-node: as defined in TS 38.401 [2].

corresponding node: as defined in TS 38.425 [7].

F1-terminating IAB-donor-CU: as defined in TS 38.401 [2].

IAB-DU: as defined in TS 38.300 [8].

IAB-MT: as defined in TS 38.300 [8].

ML: as defined in TS 38.300 [8].

Non-F1-terminating IAB-donor-CU: as defined in TS 38.401 [2].

NG-RAN node: as defined in TS 38.300 [8].

secondary node: as defined in TS 37.340 [9].

5	Functions of the Xn interface

5.1	General

The following clauses describe the functions supported in Xn interface.

5.2	Functions of Xn-C

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

5.2.13	MBS management support function

This function is used to support the management of MBS Sessions, including the addition of MBS related information in interface management and mobility procedures, and the support of RAN Multicast paging.

5.2.X	AI/ML for NG-RAN function

This function is used to support AI/ML for NG-RAN, including initiation of data collection and reporting of collected data.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

6	Xn interface procedures

6.1	General

The Xn interface supports procedures over the control plane (Xn-C) and user plane (Xn-U).

6.2	Control plane protocol procedures

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

6.2.13	QMC support procedures

The following procedures are used to transfer QMC configuration and session information to the target NG-RAN node during a UE’s intra-system intra-RAT mobility:

-	Handover Preparation

-	Retrieve UE Context

6.2.X	AI/ML for NG-RAN procedures

The following procedures are used to initiate data collection and report collected data:

-	Data Collection Reporting Initiation

-	Data Collection Reporting

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>