3GPP TSG RAN WG3#121bis	R3-235925

Xiamen, China, October 9 – 13,2023

Title:	Reply LS on IAB-node De-authorization handling

Response to:	LS in R3-233730 (S2-2308158) on IAB-node De-authorization handling

Release:	Rel-18

Work Item:	FS_VMR

Source:	RAN3

To:	SA2

Cc:

Contact person:

Name:	Georg Hampel

Tel. Number:

E-mail Address:	ghampel@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 would like to thank SA2 for their LS on IAB-node De-authorization handling (R3-233730/S2-2308158).

SA2 provided the following questions:

However, when an MBSR is no longer authorized as indicated to the gNB by AMF by N2 message, it’s not clear what is the gNB behaviour. SA2 would like to check whether this is a correct understanding: - The gNB attempts to perform handover of all the UEs served by the MBSR - When the step 1 completes, the gNB releases the F1-C Q1: is the understanding above correct? Q2: Whether and how is the Mobile IAB-DU indicated to refrain from setting up a F1-C connection to the donor CU immediately after the F1-C release (is there an indication in the F1-C release instructing the DU to not try again)?

RAN3 has discussed the questions Q1 and Q2, and converged on the following answers:

On Q1: RAN3’s confirms SA2’s understanding.

On Q2: RAN3 does not support a RAN-based mechanism that prevents the mIAB-DU from re-attempting F1 Setup after F1-release due to the mobile IAB node’s authorization status change to “non-authorized”. Nevertheless, RAN3 agrees that the IAB-donor will not allocate backhaul resources to the mobile IAB node if it is “not-authorized”. In that respect, RAN3 assumes that not providing backhaul resources to the mobile IAB node, or the NAS-based signaling to the mobile IAB node provided in TS 23.501 section 5.35A.4, v.18.3.0 is sufficient to prevent the mobile IAB-DU attempting to reestablish F1 in case it is “non-authorized”.

RAN3 asks SA2 to take the above feedback into account.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting	 #122 – November 13 to 17, 2023       Chicago, IL, USA

TSG-RAN3 Meeting	 #123 – Feb 26 – March 1, 2024       Athens, Greece