3GPP TSG-RAN WG3 Meeting #121-bis	R3-235934

Xiamen, China, 9th – 13th  October, 2023

Agenda Item:	23.2.2

Source:	Huawei, CATT, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, Samsung, Xiaomi

Title:	(TP to BL CR for TS 38.470) Support of cross-cell SRS configuration

Document for:	Approval

# 1	Introduction

This document provides a TP on F1AP 38.470 TP for BL CR according RAN3#121 bis agreement

# TP for 38.470

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

### 5.2.11	Positioning function

This function allows to transfer location management messages between gNB-CU and gNB-DU. With this function, gNB-CU request TRP information from gNB-DU, and gNB-DU response to gNB-CU with the TRP information. With this function, gNB-CU request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements.

The function allows the gNB-CU to

-	transfer the positioning assistance data to gNB-DU. The gNB-DU is responsible for broadcasting the positioning assistance data according to the scheduling parameters available.

-	request the gNB-DU to configure SRS transmissions for UE.

-	request the gNB-DU to configure PRS transmissions.

-	request the gNB-DU to configure measurement gap or PRS processing window.

-	request the gNB-DU to activate or deactivate preconfigured measurement gap or PRS processing window.

-	request the gNB-DU to broadcast positioning system information.

-    request the gNB-DU to reserve or un-reserve the SRS resources for LPHAP.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>