3GPP TSG-RAN3 Meeting #121bis	R3-235974

# Xiamen, China, 09 – 13 October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0308 | rev | 2 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.401) Support for mobile IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Core |  |  |  |  |  | Date: |  |  | 2023-10-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add change for support mobile IAB |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are: - Add procedure for mobile IAB-MT performing Xn based handover - Add description for mobile IAB node authorization - Add description for donor CU based NCGI reconfiguration - Add procedure for mobile IAB-node integration - Add procedure for mobile IAB-MT performing NG-based handover - Add procedure for mobile IAB-DU migration - Add description for IAB-node OAM to support mobility |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobile IAB cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 8.9.11, 8.9.X1, 8.9.X2, 8.12.X, 8.YY (new), 8.YY.1 (new), 8.YY.2 (new) , 8.YY.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1：R3-235120, Update based on the latest spec version, and submit to the RAN3-121bis meeting. Rev 2： R3-235974, merge agreed TPs: R3-235776, and R3-235920. |  |  |  |  |  |  |  |  |

Start of Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[5]	3GPP TS 38.414: "NG-RAN; NG data transport".

[6]	3GPP TS 38.424: "NG-RAN; Xn data transport".

[7]	3GPP TS 38.474: "NG-RAN; F1 data transport".

[8]	ITU-T Recommendation G.823 (2000-03): "The control of jitter and wander within digital networks which are based on the 2048 kbit/s hierarchy".

[9]	ITU-T Recommendation G.824 (2000-03): "The control of jitter and wander within digital networks which are based on the 1544 kbit/s hierarchy".

[10]	ITU-T Recommendation G.825 (2001-08): "The control of jitter and wander within digital networks which are based on the synchronous digital hierarchy (SDH)".

[11]	ITU-T Recommendation G.8261/Y.1361 (2008-04): "Timing and Synchronization aspects in Packet networks".

[12]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[13]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[14]	3GPP TS 38.410: "NG-RAN; NG general aspect and principles".

[15]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles"

[16]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[17]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[18]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP Network Layer Security".

[19]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA), Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[20]	3GPP TS 32.422: "Trace control and configuration management".

[21]	3GPP TS 37.470: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and NG-RAN; W1 general aspects and principles; Stage-2".

[22]	3GPP TS 38.340: "NR; Backhaul Adaptation Protocol (BAP) specification".

[23]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[24]	3GPP TS 38.425: "NG-RAN; NR user plane Protocol".

[25]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[26]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[27]	3GPP TS 23.247: " Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[28]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture Description".

[29]	IETF RFC 4555 (2006-06): "RFC IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[30]	3GPP TS 38.321 “NR; Medium Access Control (MAC) protocol specification”.

[31]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[xx1]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[xx2]	3GPP TS 28.532: “Management and orchestration; Generic management services”.

Next Change

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Associated QoS Flow: as defined in TS 23.247 [27].

Associated QoS flow information: Information encompassing: QoS flow QoS parameters for associated QoS flows and mapping information between mapped (unicast) QoS flows and associated QoS flows. The respective information is included in a way that non-supporting RAN nodes would not establish respective RAN resources irrespective the multicast session state.

Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery.

Conditional Handover: as defined in TS 38.300 [2].

Conditional PSCell Addition: as defined in TS 37.340 [12].

Conditional PSCell Change: as defined in TS 37.340 [12].

DAPS Handover: as defined in TS 38.300 [2].

eNB-CP: as defined in TS 36.401 [28].

eNB-UP: as defined in TS 36.401 [28].

en-gNB: as defined in TS 37.340 [12].

Early Data Forwarding: as defined in TS 38.300 [2].

F1-terminating IAB-donor of boundary IAB-node: Refers to the IAB-donor that terminates F1 for the boundary IAB-node.

gNB: as defined in TS 38.300 [2].

gNB Central Unit (gNB-CU): a logical node hosting RRC, SDAP and PDCP protocols of the gNB or RRC and PDCP protocols of the en-gNB that controls the operation of one or more gNB-DUs. The gNB-CU terminates the F1 interface connected with the gNB-DU.

gNB Distributed Unit (gNB-DU): a logical node hosting RLC, MAC and PHY layers of the gNB or en-gNB, and its operation is partly controlled by gNB-CU. One gNB-DU supports one or multiple cells. One cell is supported by only one gNB-DU. The gNB-DU terminates the F1 interface connected with the gNB-CU. For DC operation, the MgNB-DU designates the gNB-DU of an en-gNB or a gNB acting as master node, and the SgNB-DU designates the gNB-DU of an en-gNB or a gNB acting as secondary node.

gNB-CU-Control Plane (gNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the gNB-CU for an en-gNB or a gNB. The gNB-CU-CP terminates the E1 interface connected with the gNB-CU-UP and the F1-C interface connected with the gNB-DU. For DC operation, the MgNB-CU-CP designates the gNB-CU-CP of the gNB-CU for an en-gNB or a gNB acting as master node, and the SgNB-CU-CP designates the gNB-CU-CP of the gNB-CU for an en-gNB or a gNB acting as secondary node.

gNB-CU-User Plane (gNB-CU-UP): a logical node hosting the user plane part of the PDCP protocol of the gNB-CU for an en-gNB, and the user plane part of the PDCP protocol and the SDAP protocol of the gNB-CU for a gNB. The gNB-CU-UP terminates the E1 interface connected with the gNB-CU-CP and the F1-U interface connected with the gNB-DU. For DC operation, the MgNB-CU-UP designates the gNB-CU-UP of the gNB-CU for an en-gNB or a gNB acting as master node, and the the SgNB-CU-UP designates the gNB-CU-UP of the gNB-CU for an en-gNB or a gNB acting as secondary node.

IAB-node: as defined in TS 38.300 [2].

IAB-donor: as defined in TS 38.300 [2].

IAB-donor-CU: the gNB-CU of an IAB-donor, terminating the F1 interface towards IAB-nodes and IAB-donor-DU.

IAB-donor-DU: the gNB-DU of an IAB-donor, hosting the IAB BAP sublayer (as defined in TS 38.340 [22]), providing wireless backhaul to IAB-nodes.

IAB-DU: as defined in TS 38.300 [2].

IAB-MT: as defined in TS 38.300 [2].

IAB Topology: as defined in TS 38.300 [2].

Mapped QoS flows: Unicast QoS flows requested to be established, i.e. included in the legacy QoS flow lists in a way, that non-support RAN nodes would attempt to establish unicast QoS flows and supporting RAN nodes can identify them as mapped QoS flows based on the associated QoS information.

Master node: as defined in TS 37.340 [12].

Master gNB: see TS 37.340 [12].

MBS session resource: This term is used for specification of NG, Xn, F1 and E1 interfaces. It denotes NG-RAN interface and radio resources provided to support an MBS Session.

ng-eNB: as defined in TS 38.300 [2].

ng-eNB Central Unit (ng-eNB-CU): as defined in TS 37.470 [21].

ng-eNB Distributed Unit (ng-eNB-DU): as defined in TS 37.470 [21].

ng-eNB-CU-Control Plane (ng-eNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the ng-eNB-CU for an ng-eNB. The ng-eNB-CU-CP terminates the E1 interface connected with the ng-eNB-CU-UP and the W1-C interface connected with the ng-eNB-DU.

ng-eNB-CU-User Plane (ng-eNB-CU-UP): a logical node hosting the user plane part of the PDCP protocol and the SDAP protocol of the ng-eNB-CU for an ng-eNB. The ng-eNB-CU-UP terminates the E1 interface connected with the ng-eNB-CU-CP and the W1-U interface connected with the ng-eNB-DU.

NG-RAN node: as defined in TS 38.300 [2].

Non-F1-terminating IAB-donor of boundary IAB-node: Refers to the IAB-donor that has an RRC connection with the boundary node but does not terminate F1 with this boundary node.

PDU Session Resource: This term is used for specification of NG, Xn, and E1 interfaces. It denotes NG-RAN interface and radio resources provided to support a PDU Session.

Public Network Integrated NPN: as defined in TS 23.501 [3].

RRC-terminating IAB-donor: Refers to the IAB-donor that terminates the RRC connection of the mobile IAB-node. The RRC-terminating IAB-donor may be an F1-terminating or a non-F1-terminating IAB-donor.

Secondary gNB: see TS 37.340 [12].

Stand-alone Non-Public Network: as defined in TS 23.501 [3].

U2N Relay UE: as defined in TS 38.300 [2].

U2N Remote UE: as defined in TS 38.300 [2].

Next Change

3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

BAP	Backhaul Adaptation Protocol

BH	Backhaul

CAG	Closed Access Group

CHO	Conditional Handover

CLI	Cross-Link Interference

CM	Connection Management

CMAS	Commercial Mobile Alert Service

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DU	Distributed Unit

EM	Element Manager

EN-DC	E-UTRA-NR Dual Connectivity

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

FTEID	Fully Qualified TEID

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

L2					Layer-2

MBS	Multicast Broadcast Service

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MN	Master Node

MgNB	Master Gnb

mIAB	Mobile IAB

mIAB-DU	Mobile IAB Distributed Unit

mIAB-MT	Mobile IAB Mobile Termination

MRB	MBS Radio Bearer

MRDC	Multi-Radio Dual Connectivity

NAS	Non-Access Stratum

NID	Network identifier

NPN	Non-Public Network

NSA	Non Standalone

OAM	Operation, Administration and Maintenance

PNI-NPN	Public Network Integrated Non-Public Network

PTP	Point to Point

PTM	Point to Multipoint

PWS	Public Warning System

QoE	Quality of Experience

QoS	Quality of Service

RET	Remote Electrical Tilting

RIM	Remote Interference Management

RIM-RS	Remote Interference Management Reference Signal

RNL	Radio Network Layer

RRC	Radio Resource Control

SA	Standalone

SAP	Service Access Point

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SFN	System Frame Number

SgNB	Secondary gNB

SM	Session Management

SMF	Session Management Function

SN	Secondary Node

SNPN	Stand-alone Non-Public Network

SRAP	Sidelink Relay Adaptation Protocol

TCE	Trace Collection Entity

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TEID	Tunnel Endpoint Identifier

TMA	Tower Mounted Amplifier

TNL	Transport Network Layer

U2N	UE-to-Network

UL 	Uplink

Next Change

8.9.11	IAB-node OAM

NOTE:	The general principles and procedures described in this clause does not apply to ng-eNB.

The IAB-node receives commands, configuration data and software downloads (e.g. for equipment software upgrades) from its OAM system. The IAB-node can also send alarms and traffic counter information to its OAM system. The transport connection between the IAB-node and its OAM, using IP, is provided by the IAB-MT’s PDU session via 5G network, or the IAB-MT’s PDN connection via LTE network when IAB-MT uses EN-DC.

NOTE:	The transport connection between the IAB-node and its OAM may also be provided using the Backhaul IP layer by implementation.

Alarms in the IAB generate bursts of high-priority traffic, to be transported in real time. Traffic counters generate bursts of traffic, but their transport need not be real-time. Configuration messages from OAM to the IAB will also generate small bursts of traffic, possibly with lower priority than alarms but still delay-sensitive: when a configuration is committed on the OAM, the time interval between the commitment and the effect on the equipment shall be small.  Alarm messages and commands should be transported on a high-priority bearer, while counters may be transported on a lower priority bearer.

OAM software download to the IAB may generate larger amounts of data, but both the required data rate and the priority of this kind of traffic are much lower than in the case of alarms, commands and counters.

For different types of OAM traffic, it is necessary to use different DRBs between the IAB-MT and the serving DU, and different BH RLC channels for intermediate hops, with different QoS parameters. Aggregation of F1-U traffic for OAM with other F1-U traffic on the same BH RLC channels is not precluded. The QoS parameters are provided to the IAB-donor during the IAB-MT’s PDU session establishment, or the IAB-MT’s PDN connection establishment when IAB-MT uses EN-DC.

NOTE:	When the transport connection between the IAB-node and its OAM is provided by the Backhaul IP layer, the OAM traffic may be aggregated with other traffic types on the same BH RLC channel. The QoS for OAM is ensured by implementation.

The continuity of OAM connectivity needs to be ensured as the mobile IAB-node moves across the mobile network.

Next Change

8.9.X1 Mobile IAB node authorization

During mobile IAB-node integration procedure, the RRC-terminating IAB-donor-CU receives the authorization status of the mobile IAB-node from the 5GC. If the authorization status is “not authorized”, the RRC-terminating IAB-donor-CU will not establish any backhaul resources (including BAP address, TNL address and default BAP configuration) for this mobile IAB-node. If the authorization status for the mobile IAB-node changes, the 5GC sends an updated authorization status to the RRC-terminating IAB-donor.

In case the mobile IAB-MT and its co-located mIAB-DU connect to different IAB-donor-CUs, i.e. the RRC-terminating IAB-donor is different from the F1-terminating IAB-donor, the RRC-terminating IAB-donor will inform the F1-terminating IAB-donor about the authorization status of the mobile IAB-node via XnAP signalling once the authorization status has been updated.

NOTE: In absence of Xn between RRC-terminating IAB-donor-CU and F1-terminating IAB-donor-CU, the passing of the authorization status is left up to implementation.

If the updated authorization status for the mobile IAB node is “not authorized”, the F1-terminating donor may hand over the UEs served by the mobile IAB-node, and should releases the F1 interface towards the mobile IAB-DU. After that, the F1-terminating IAB-donor requests from the RRC-terminating donor the release of all the offloaded traffics, and then the RRC-terminating donor releases all backhaul resources (including BAP address, TNL address and default BAP reconfiguration) for this mobile IAB node.

Next Change

8.9.X2	IAB-donor-CU-based NR Cell Identity (NCI) (re)configuration for mobile IAB cells

The NCIs of the cells served by a mobile IAB-DU configured by the OAM can be reconfigured by the F1 terminating IAB-donor-CU serving the mobile IAB-DU, in case of an NCI collision with cells of other gNB-DUs supported by the IAB-donor-CU. The reconfiguration of NCI pertains to the reconfiguration of the cellLocalId part of the NCI, where the new cellLocalId(s) are based on a list of NCIs that has been configured on this F1-terminating IAB-donor-CU.

The value change of cellLocalId(s) shall be indicated to the OAM system of the mobile IAB-DU following the NCI reconfiguration. The mobile IAB-DU can notify OAM about the reconfigured cellLocalId(s) using notifications specified in TS 28.532 [xx2].

NOTE:	This shall not affect the existing procedure of configuring NCGI of cells served by a stationary gNB-DU via the OAM.

Next Change

8.12.X  Mobile IAB node integration

During the integration, the mobile IAB-MT and mobile IAB-DU can connect to the same IAB-donor or to different IAB-donors. The procedure for the latter case is shown in Figure 8.12.X-1.

Figure 8.12.X-1: Decoupled mobile IAB node integration procedure

Phase 1: Equivalent procedure to Phase 1 of the IAB-node integration in SA mode in section 8.12.1, where the mobile IAB-node and the RRC-terminating IAB-donor correspond to IAB-node 2 and the IAB-donor, respectively. The mobile IAB-node selects the parent node based on a mobile-IAB-specific over-the-air indication (transmitted in SIB1). The mobile IAB-MT includes a mobile-IAB-node-specific indication in the RRCSetupComplete message to assist the RRC-terminating IAB-donor to select an AMF supporting mobile IAB.

Phase 2-1: Same as Phase 2-1 of procedure in section 8.12.1.

Phase 2-2: Same as Phase 2-2 of procedure in section 8.12.1.

Phase 3: Mobile IAB-DU part setup. In this phase, the mobile IAB-DU is configured via OAM. This configuration includes the information for the establishment of F1-C to the F1-terminating IAB-donor-CU.  The mobile IAB-DU initiates the TNL establishment, and F1 setup (as defined in clause 8.5) with the F1-terminating IAB-donor-CU using the default BAP routing ID and default BH RLC channel configured by the RRC terminating IAB-donor-CU in Phase 2-1 for upstream traffic. During the F1 setup, the mobile IAB-DU includes the gNB ID of the RRC-terminating IAB-donor-CU and its BAP address in the F1 SETUP REQUEST message. The mobile IAB-node obtains this gNB ID from the over-the-air broadcast by the RRC-terminating IAB-donor (SIB1).

After the F1 is set up, the mobile IAB-node can start serving UEs. The F1-terminating IAB-donor-CU can initiate the IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor-CU as defined in section 8.17.3.1.

Next Change

8.YY	Mobile IAB migration procedures

8.YY.1 Migration of mobile IAB-MT via Xn handover

The mIAB-MT of a mobile IAB-node can be migrated from a source RRC-terminating IAB-donor-CU to a target RRC-terminating IAB-donor-CU using the Xn handover procedure. During this migration, the mIAB-DU co-located with the mIAB-MT is connected to an F1-terminating IAB-donor-CU, which may be the same as the source RRC-terminating IAB-donor-CU or the target RRC-terminating IAB-donor-CU, or it can be different from both the source and the target RRC-terminating IAB-donor-CU.

Figure 8.YY.1.1-1 shows an example of mIAB-MT migration via Xn handover. In this example, the mIAB-MT is connected to the source RRC-terminating IAB-donor-CU via a source path of an IAB topology before the migration, and it is connected to the target RRC-terminating IAB-donor-CU via a target path of a different IAB topology after the migration.

Figure 8.YY.1.1-1: Procedure for Xn-based migration of mobile IAB-MT

1. Steps 1-14 of the topology adaptation procedure of Section 8.17.3.1 are performed to conduct Xn handover of the mIAB-MT from the source parent IAB-node connected to the source RRC-terminating IAB-donor-CU to the target parent IAB-node connected to the target RRC-terminating IAB-donor-CU. In these steps, the mIAB-node corresponds to the migrating IAB-node of Section 8.17.3.1, and the mIAB-MT’s source and target RRC-terminating IAB-donor-CUs correspond to the respective source and target IAB-donor-CUs of Section 8.17.3.1.

2. Same as step 15 of the topology adaptation procedure of Section 8.17.3.1, where the F1-C connection between the co-located mIAB-DU and its F1-terminating IAB-donor-CU is switched to the target path using the new TNL address information of the IAB-MT. In this step, the mIAB-node corresponds to the migrating IAB-node, and the F1-terminating IAB-donor-CU corresponds to the source IAB-donor-CU.

3. The mIAB-DU passes the gNB ID of the target RRC-terminating IAB-donor-CU and the mIAB-node’s BAP address allocated by the target RRC-terminating IAB-donor-CU to the F1-terminating IAB-donor-CU via F1AP.

4. Steps 16-20 of the topology adaptation procedure of Section 8.17.3.1, where the F1-terminating IAB-donor-CU initiates the IAB Transport Migration Management procedure towards the target RRC-terminating IAB-donor-CU to provide the context of the traffic offloaded. The target RRC-terminating IAB-donor-CU reconfigures the BAP sublayer and/or BH RLC channels on the target path accordingly, and provides the UL BH information for UL BH reconfigurations to be conducted by the F1-terminating IAB-donor-CU on the mIAB-node. Then, the F1-U connections of the mIAB-node are migrated to the target path.

Next Change

8.YY.2 Migration of mobile IAB-MT via NG handover

The mIAB-MT of a mobile IAB-node can be migrated from a source RRC-terminating IAB-donor-CU to a target RRC-terminating IAB-donor-CU using the NG handover procedure. During this migration, the mIAB-DU co-located with the mIAB-MT is connected to an F1-terminating IAB-donor-CU, which may be the same as the source RRC-terminating IAB-donor-CU or the target RRC-terminating IAB-donor-CU, or it can be different from both the source and the target RRC-terminating IAB-donor-CU.

Figure 8.YY.2.1-1 shows an example of mIAB-MT migration via NG handover. In this example, the mIAB-MT is connected to the source RRC-terminating IAB-donor-CU via a source path of an IAB topology before the migration, and it is connected to the target RRC-terminating IAB-donor-CU via a target path of a different IAB topology after the migration.

Figure 8.YY.2.1-1: Procedure for NG-based migration of mobile IAB-MT

1. Similar to Step 1-14 in Section 8.17.3.1, where the NG-based handover procedure is used instead as defined in Section 4.9.1.3.2 and 4.9.1.3.3 in TS 23.502[xx1].

2. Same as step 2 to step 4 of Section 8.YY.1.

NOTE:     How to exchange the IAB Transport Migration Management/Modification messages between the F1-terminating IAB-donor-CU and the target RRC-terminating IAB-donor-CU without Xn interface is up to implementation.

8.YY.3 Mobile IAB-DU migration procedure

The RAN may perform the mobile IAB-DU migration procedure. During this procedure, the mobile IAB-node concurrently supports two logical mobile IAB-DUs, which have F1AP associations with the source F1-terminating IAB-donor-CU and target F1-terminating IAB-donor-CU, respectively. The mobile IAB-MT’s IAB-donor-CU may be the same as either the source F1-termainting IAB-donor CU or the target F1-terminating IAB-donor-CU, or it may be different from both source and target F1-terminating IAB-donor-CUs.

UEs connected to the mobile IAB-node are handed over from the cell(s) of the source logical mobile IAB-DU associated with the source F1-terminating IAB-donor-CU to the cell(s) of the target logical mobile IAB-DU associated with the target F1-terminating IAB-donor-CU. After the UEs are handed over, the source logical mobile IAB-DU’s F1AP association with the source F1-terminating IAB-donor-CU may be released.

Figure 8.YY.3.1-1 shows an example of the mobile-IAB-DU migration procedure. In this example, the source and target F1-terminating IAB-donor-CUs are different from the RRC-terminating IAB-donor-CU.

Figure 8.YY.3.1-1: Mobile IAB-DU inter-CU migration procedure

- The source F1-terminating IAB-donor-CU sends an MIAB F1 SETUP TRIGGERING to the source logical mIAB-DU to initialize the F1 Setup procedure towards the target F1-terminating IAB-donor-CU. The MIAB F1 SETUP TRIGGERING message includes the gNB ID of the target F1-terminating IAB-donor-CU and the information needed to establish the TNL connection with the target F1-terminating IAB-donor-CU for F1-C.

NOTE:     The mIAB-DU migration can also be triggered by the OAM. In this case, OAM provides the mIAB-node with all information to initiate the F1 Setup procedure toward the target F1-terminating IAB-donor-CU, and step 1 is omitted.

- The target logical mIAB-DU initiates TNL establishment and F1 setup (as defined in clause 8.5) with the target F1-terminating IAB-donor-CU. During the F1 Setup procedure, the target logical mobile IAB-DU includes the gNB ID of the RRC-terminating IAB-donor-CU, and the BAP address of the mIAB-node in the F1 SETUP REQUEST message.

- The target F1-terminating IAB-donor-CU responds to the target logical mIAB-DU with an F1 SETUP RESPONSE message. After F1 setup with the target F1-terminating IAB-donor-CU, the target logical mobile IAB-DU can serve UEs via the target mobile IAB-DU’s activated cell(s).

- By sending the MIAB F1 SETUP OUTCOME NOTIFICATION, the source logical mIAB-DU informs the source F1-terminating IAB-donor-CU about the outcome of the F1 interface setup between the co-located target logical mIAB-DU and the target F1-terminating IAB-donor-CU. The source logical mIAB-DU may provide the source F1-terminating IAB-donor-CU a mapping between activated cells of the source logical mIAB-DU and those of the target logical mIAB-DU.

- The source F1-terminating IAB-donor-CU hands over the UE from a source cell served by the source logical mobile IAB-DU to a target cell served by the target logical mobile IAB-DU. The target F1-termianting IAB-donor-CU initiates IAB Transport Migration management procedure towards the RRC-terminating IAB-donor-CU during this step.

Editor’s NOTE: The sequence of procedures of UE HO and IAB TMM procedures is FFS.

NOTE:     How to exchange the IAB Transport Migration Management/Modification messages between the target F1-terminating IAB-donor-CU and the RRC-terminating IAB-donor-CU without Xn interface is up to implementation.

- After all the UEs are handed over, the source F1-terminating IAB-donor-CU may initiate the removal of the F1 interface towards the source logical mIAB-DU.

End of Change