# 3GPP TSG-RAN WG3 Meeting #121bis	R3-235998

# Xiamen, China, 9th – 13th October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0285 | rev | 7 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BLCR to 38.401) Introduction of NR SL relay enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Huawei, Samsung, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2023-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of NR SL relay enhancements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the inter-DU path addition procedure - Added the intra-DU path addition procedure - Added the U2N Remote UE direct-to-indirect path switch with gNB-CU change procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR SL relay enhancements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.19.4, 8.xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Resubmission to RAN3#119bis-e meeting Rev2: Capture agreed TPs R3-231956 and R3-232088. Rev3: Capture agreed TP R3-233447. Rev4: Resubmission to RAN3#121 meeting Rev5: Resubmission to RAN3#121bis meeting Rev6: Update the editable figures in Clause 8.xx Rev7: Capture agreed TP R3-235771. |  |  |  |  |  |  |  |  |

----------------Start of the First Change---------------

8.19.4.X	Inter-gNB-CU switch from direct to indirect path

The signalling flow for U2N Remote UE switch from direct to indirect path with gNB-CU change is shown in Figure 8.19.4.X-1.

Figure 8.19.4.X-1. U2N Remote UE Direct-to-indirect Path Switch with gNB-CU change procedure

1.	The Uu measurement configuration and measurement report signalling is performed between U2N Remote UE and source gNB-CU to evaluate both relay link measurement and Uu link measurement. The U2N Remote UE may report one or multiple candidate U2N Relay UE(s) and Uu measurement results after it measures/discovers the candidate U2N Relay UE(s).

2.	The source gNB-CU decides to switch the U2N Remote UE to one of the candidate U2N Relay UE(s).

3.	The source gNB-CU sends the HANDOVER REQUEST message to the target gNB-CU. The HANDOVER REQUEST message may include a list of candidate U2N Relay UE(s) of same cell of the target gNB.

4.	The target gNB-CU decides to accept the indirect path switching to the target U2N Relay UE among the candidate U2N Relay UE(s).

5.	The reconfiguration to target U2N Relay UE is performed among target U2N Relay UE, the target gNB-DU and target gNB-CU, if the target U2N Relay UE is in RRC_CONNECTED state. The target gNB-CU allocates the local ID for the U2N Remote UE. The target gNB-CU sends an RRCReconfiguration message to the target U2N Relay UE. If the target U2N Relay UE is in RRC_IDLE/INACTIVE state, this step is skipped and the configuration to the target U2N Relay UE is performed in Step 15.

6.	The target gNB-CU sends the UE CONTEXT SETUP REQUEST message for the U2N Remote UE to the target gNB-DU, which contains the path switch configuration at least.

7.	The target gNB-DU responds with the UE CONTEXT SETUP RESPONSE message to the target gNB-CU.

8.	The target gNB-CU responds the source gNB-CU with an HANDOVER REQUEST ACKNOWLEDGE message by including the RRCReconfiguration message. The contents in the RRCReconfiguration message may include at least path switch configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s).

9.	The source gNB-CU sends to the source gNB-DU the UE CONTEXT MODIFICATION REQUEST message to send the handover command to the U2N Remote UE, and to indicate to stop the data transmission for the U2N Remote UE.

10.	The source gNB-DU sends the RRCReconfiguration message to the U2N Remote UE. The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the source gNB.

11.	The source gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the source gNB-CU.

12.	The source gNB-CU sends an SN STATUS TRANSFER message to the target gNB-CU.

13.	Data Forwarding may be performed from the source gNB-CU to the target gNB-CU.

14.	The U2N Remote UE establishes PC5 connection with target U2N Relay UE.

15.	The U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the target gNB-DU via the target U2N Relay UE. In case the target U2N relay UE is in RRC_IDLE/ INACTIVE state when receiving the RRCReconfigurationComplete message, the reception of the RRCReconfigurationComplete message will first trigger RRC setup/resume procedure for the target U2N relay UE to enter RRC_CONNECTED state.

16.	The target gNB-DU sends the UL RRC MESSAGE TRANSFER message to target gNB-CU by including the RRCReconfigurationComplete message.

17.	Path Switch procedure is performed to switch the DL data path towards the target gNB-CU and to establish an NG-C interface instance towards the target gNB-CU.

18.	The target gNB-CU sends an UE CONTEXT RELEASE message to the source gNB-CU.

19.	The source gNB-CU initiates a F1 UE Context Release procedure to release the UE context of the U2N Remote UE in the source gNB-DU.

-----------------End of the First Change---------------

----------------Start of the Second Change---------------

8.xx	Overall procedures for multi-path support

8.xx.1	Inter-DU direct path addition on top of indirect path

The signalling flow for inter-DU direct path addition is shown in Figure 8.xx.1-1.

Figure 8.xx.1-1: Signalling procedure of inter-DU direct path addition on top of indirect path

1.	The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the direct path to remote UE under a different gNB-DU (i.e., gNB-DU1). Mode 1 resource configuration cannot be configured for remote UE in inter-gNB-DU multi-path relay in this release.

3.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU1, which contains at least the direct path configuration.

4.	The gNB-DU1 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

5.	The gNB-CU sends an RRCReconfiguration message to the relay UE to update the indirect path configuration if necessary.

6.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for remote UE by including the RRCReconfiguration message to the gNB-DU2. The contents in the RRCReconfiguration message may include at least direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).

7.	The gNB-DU2 sends the RRCReconfiguration message to the remote UE.

8.	The gNB-DU2 sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

9.	The remote UE performs random access procedure at the gNB-DU1.

10.	The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU2. In case duplicate SRB1 is configured, the remote UE sends the RRCReconfigurationComplete message to the gNB via both direct path and indirect path. Otherwise, the remote UE sends the RRCReconfigurationComplete message to the gNB via direct path.

11.	The gNB-DU2 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

8.xx.2	Inter-DU indirect path addition on top of direct path

The signalling flow for inter-DU indirect path addition is shown in Figure 8.xx.2-1.

Figure 8.xx.2-1 Signalling procedure of inter-DU indirect path addition on top of direct path

1. If the remote UE is connected with the relay UE using sidelink, the Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

In case that the remote UE is connected with the relay UE using non-3GPP link, the remote UE reports at least C-RNTI of the relay UE and the cell ID of the serving relay UE.

2.	The gNB-CU decides to add the indirect path via relay UE to remote UE under a different gNB-DU (i.e., gNB-DU2). Mode 1 resource configuration cannot be configured for remote UE in inter-gNB-DU multi-path relay in this release.

3.	The reconfiguration to relay UE is performed among relay UE, gNB-DU2 and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped.

4.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU2, which contains the indirect path configuration at least.

5.	The gNB-DU2 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU1. If the remote UE using the sidelink has no PC5 connection with the target relay UE, the contents in the RRCReconfiguration message may include at least indirect path addition configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s). If the remote UE is using non-3GPP link, the contents in the RRCReconfiguration message may include at least indirect path addition configuration, bearer mapping and the associated radio bearer(s).

7.	gNB-DU1 sends the RRCReconfiguration message to the remote UE.

8.	If the remote UE using the sidelink has no PC5 connection with the target relay UE, the remote UE establishes PC5 connection with relay UE. If the remote UE is using non-3GPP link, this step is skipped.

9.	The remote UE may complete the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU1. In case duplicate SRB1 is configured, the remote UE sends the RRCReconfigurationComplete message to the gNB via both direct path and indirect path. Otherwise, the remote UE sends the RRCReconfigurationComplete message to the gNB via direct path.

Editor’s Note: FFS: In case the relay UE is in RRC_IDLE/ INACTIVE state, how to trigger the relay UE enter RRC_CONNECTED state, RAN3 wait for RAN2’s further progress.

10.	The gNB-DU1 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

8.xx.3	Intra-DU direct path addition on top of indirect path

The signaling flow for intra-DU direct path addition is shown in Fig. 8.xx.3-1.

Figure 8.xx.3-1: Signalling procedure of intra-DU direct path addition on top of indirect path

1.	The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the direct path to remote UE under the same gNB-DU.

3.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for the remote UE to the gNB-DU, which contains at least the direct path configuration.

4.	The gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message.

5.	The gNB-CU sends an RRCReconfiguration message to the relay UE to update the indirect path configuration if necessary.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to the gNB-DU. The contents in the RRCReconfiguration message may include at least direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).

7.	The gNB-DU sends the RRCReconfiguration message to the remote UE.

8.	The remote UE performs random access procedure at the gNB-DU.

9.	The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU. In case duplicate SRB1 is configured, the remote UE sends the RRCReconfigurationComplete message to the gNB via both direct path and indirect path. Otherwise, the remote UE sends the RRCReconfigurationComplete message to the gNB via direct path.

10.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

8.xx.4	Intra-DU indirect path addition on top of direct path

The signaling flow for intra-DU indirect path addition is shown in Fig. 8.xx.4-1.

Figure 8.xx.4-1: Signalling procedure of intra-DU indirect path addition on top of direct path

1. If the remote UE is connected with the relay UE using sidelink, the Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

In case that the remote UE is connected with the relay UE using non-3GPP link, the remote UE reports at least C-RNTI of the relay UE and the cell ID of the serving relay UE.

2.	The gNB-CU decides to add the indirect path via relay UE to remote UE under the same gNB-DU.

3.	The reconfiguration to relay UE is performed among relay UE, gNB-DU and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped.

4.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for the remote UE to the gNB-DU, which contains the indirect path configuration at least.

5.	The gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU. If the remote UE using the sidelink has no PC5 connection with the target relay UE, the contents in the RRCReconfiguration message may include at least indirect path addition configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s). If the remote UE is using non-3GPP link, the contents in the RRCReconfiguration message may include at least indirect path addition configuration, bearer mapping and the associated radio bearer(s).

7.	gNB-DU sends the RRCReconfiguration message to the remote UE.

8.	If the remote UE using the sidelink has no PC5 connection with the target relay UE, the remote UE establishes PC5 connection with relay UE. If the remote UE is using non-3GPP link, this step is skipped.

9.	The remote UE may complete the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU. In case duplicate SRB1 is configured, the remote UE sends the RRCReconfigurationComplete message to the gNB via both direct path and indirect path. Otherwise, the remote UE sends the RRCReconfigurationComplete message to the gNB via direct path.

Editor’s Note: FFS: In case the relay UE is in RRC_IDLE/ INACTIVE state, how to trigger the relay UE enter RRC_CONNECTED state, RAN3 wait for RAN2’s further progress.

10.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

-----------------End of the Second Change---------------