3GPP TSG-RAN WG3 Meeting #121bis	R3-235236

Xiamen, China, 9th – 13th October 2023

Agenda item:	 11.3

Source:		Lenovo

Title:	On Remaining issues of QoE measurement in NR-DC

Document for:		Discussion and Decision

- Introduction

This paper discuss the remaining main issues for NR-DC:

-	Support mobility in NR-DC, including MN/SN initiated SN change without MN change, inter-MN handover with/without SN change;

-	SCG failure;

-	QoE pause/resume in NR-DC.

- Discussion

  - Support mobility in NR-DC

MN initiated SN change without MN change

- In RAN3#120 meeting, it was agreed that:

At SN change without MN change, the QoE configuration information is passed from the old/source SN to the new/target SN via the MN.

- Although there are coordination between MN and SN, the MN has no QoE Configuration Information for management-based QoE activation received directly by the SN from the OAM.

- There are two cases for SN change:

-	MN initiated SN change; and

-	SN initiated SN change.

- For MN initiated SN change, the MN initiates the SN change by requesting the target SN to allocate resources for the UE by means of the SN Addition procedure. Before SN Addition procedure, the MN needs to trigger the MN-initiated SN Modification procedure to the source SN to retrieve the QMC Configuration Information of SN. And then the MN needs to include the QMC Configuration Information of source SN in the SN Addition procedure.

For MN initiated SN change, MN needs to trigger the MN-initiated SN Modification procedure to the source SN to retrieve the QMC Configuration Information of the source SN.

- More specifically, the MN needs to send a QMC Configuration Query indication to the SN in the S-NODE MODIFICATION REQUEST message. The QMC configuration Query indication is used to request the source SN provide the QMC Configuration Information that have been configured to the UE and received from OAM.

- Upon receives the QMC configuration Query indication, the SN provides the QMC Configuration Information to the MN in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message. The QMC Configuration Information can reuse the existing QMC Configuration Information IE.

The MN sends a QMC Configuration Query indication to the source SN in the S-NODE MODIFICATION REQUEST message.

The source SN provides the QMC Configuration Information to the MN in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message if the M-based QoE measurements have been configured to the UE.

SN initiated SN change without MN change

- For SN initiated SN change, the source SN initiates the SN change procedure by sending SgNB Change Required message. Similarly, the source SN provides the QMC Configuration Information to the MN in the S-NODE CHANGE REQUIRED message. The QMC Configuration Information can reuse the existing QMC Configuration Information IE.

The source SN provides the QMC Configuration Information to the MN in the S-NODE CHANGE REQUIRED message if the M-based QoE measurements have been configured to the UE.

- Since at both MN initiated and SN initiated SN change without MN change, the QoE configuration information is passed from the old/source SN to the new/target SN via the MN, the MN needs to forward the QoE Configuration Information to the target gNB in the S-NODE ADDITION REQUEST message.

The MN forwards the QMC Configuration Information to the target SN in the the S-NODE ADDITION REQUEST message.

- It is assumed that both MN and SN can configure QoE measurement independently. For example, the MN configures QoE measurement by SRB1 while SN can configure QoE measurement by SRB3. During SN change procedure, in order to support delta configuration in target SN, the source SN provides source SCG related QoE configuration information to the MN. One of solution is the source SCG related QoE configuration information is provided by scg-CellGroupConfig IE in the CG-Config IE that is an IE of inter-node RRC message as specified in TS 38.331:

:

scg-CellGroupConfig - Contains the RRCReconfiguration message (containing only secondaryCellGroup and/or measConfig and/or otherConfig and/or conditionalReconfiguration and/or bap-Config and/or iab-IP-AddressConfigurationList and/or appLayerMeasConfig)

- The MN provides the source SCG related QoE configuration information to the target SN. The MN may also provide the MN related QoE configuration information. The MN includes the information in the SN Addition Request message.  Specifically, the MN includes the source SCG related QoE configuration information in the sourceConfigSCG IE in the CG-ConfigInfo IE that is an IE of inter-node RRC message:

sourceConfigSCG - Includes all of the current SCG configurations used by the target SN to build delta configuration to be sent to UE, e.g. during SN change. The field contains the RRCReconfiguration message, i.e. including secondaryCellGroup and measConfig, and appLayerMeasConfig. The field is signalled upon change of SN, unless MN uses full configuration option. Otherwise, the field is absent.

- The target SN uses the source SCG related QoE configuration information to generate the delta configuration of the QoE configuration.

In order to support delta configuration of a QoE measurement configuration in target SN during SN change procedure, source SN provides the source SCG related QoE configuration information to target SN, forwarded via MN.

- During SN change procedure, if we allow the target SN to change the report leg (e.g. from SRB4 to SRB5), it would need additional coordination between MN and SN. For example, the SN sends the QMC Modification Request IE to the MN and MN responds the QMC Modification Response to the SN. This coordination seems like difficult to be done during SN Addition procedure. It’s more reasonable to not allow the target SN to change the report leg during SN Addition procedure which will cause additional delay for SN addition. The switching of report leg can be done SN Modification related procedure followed by SN Addition procedure.

QMC modification (e.g., switching of reporting SRB) is not performed during SN Addition procedure for SN change.

Inter MN handover with/without SN change

- For inter MN handover with/without SN change case, the QMC Configuration Information should be forward form source SN -> Source MN -> Target MN -> Target SN. The transmission QMC Configuration Information between Source SN and Source MN, between Target MN and Target SN is the same with SN change without MN change. The additional thing is the source MN needs to forward the QMC Configuration Information of the source SN to the Target MN in the Handover Request message.

The source MN forwards the QMC Configuration Information of the source SN to the target MN in the Handover Request message.

  - SCG Failure

- The following cases are considered as when SCG failure occurs:

-	A QoE Measurement is configured by MN, but the QoE measurement reporting is configured as SCG that means that the UE sends the QoE measurement report to network by SCG leg by SRB5.

-	A QoE Measurement is configured by SN, and the QoE measurement reporting is configured as MCG or SCG that means that the UE sends the QoE measurement report to network by MCG leg or SCG leg either by a SRB4 or SRB5.

- When SCG failure is detected, it is possible that the QoE measurement report is available at UE RRC layer (e.g. application layer measurement report container has been received from upper layers when SCG failure happens) since QoE measurement reporting has lower priority. When the reporting leg is configured as ‘SCG’, the UE is not able to send the QoE measurement report via SCG leg due to SCG failure. In this case, the following solutions can be considered:

-	Option 1: The UE switches the reporting leg to be MCG automatically upon detecting SCG failure;

-	Option 2: The UE suspends the QoE measurement reporting via SCG leg and buffers the QoE measurement report in RRC until instruction from MN is received. the UE may also send ‘QoE measurement result available’ indication to MN in the SCGFailureInformation message.

-	Option 3: The UE discards the QoE measurement report that reporting leg is configured as ‘SCG leg’.

RAN3 discusses how to handle the available QoE measurement report in SCG leg when SCG failure happens.

  - QoE pause/resume in NR-DC

- In Rel-17, QoE Measurement Collection pause/resume procedure is used to pause/resume the reporting for all QoE reports or to pause/resume QoE reporting per QoE configuration in a UE in RAN overload situation. However, in NR-DC, since both MN/MCG leg and SN/SCG leg can be used for the QoE measurement reports, how to configure QoE measurement pause/resume function needs to be addressed.

- For example, the MN configures QMC pause/resume taking both MN and SN overload information into account. The UE then pauses or resumes the QoE measurement reporting on both DC legs. The MN can infer the SN overload status for QoE measurement reporting by implementation e.g. according to the flow control info.

- For example, the MN decides to pause to the QoE measurement reporting if both MN and SN are overloaded for QoE measurement reporting. The MN sets pauseReporting to true and send it to UE.

When setting ‘pause or resume’, both MN and SN overload should be taken into account.

3	Conclusion

This contribution further discusses how to support QoE measurement in NR-DC from RAN3 perspective. And we propose:

- For MN initiated SN change, MN needs to trigger the MN-initiated SN Modification procedure to the source SN to retrieve the QMC Configuration Information of the source SN.

- The MN sends a QMC Configuration Query indication to the source SN in the S-NODE MODIFICATION REQUEST message.

- The source SN provides the QMC Configuration Information to the MN in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message if the M-based QoE measurements have been configured to the UE.

- The source SN provides the QMC Configuration Information to the MN in the S-NODE CHANGE REQUIRED message if the M-based QoE measurements have been configured to the UE.

- The MN forwards the QMC Configuration Information to the target SN in the the S-NODE ADDITION REQUEST message.

- In order to support delta configuration of a QoE measurement configuration in target SN during SN change procedure, source SN provides the source SCG related QoE configuration information to target SN, forwarded via MN.

- QMC modification (e.g., switching of reporting SRB) is not performed during SN Addition procedure for SN change.

- The source MN forwards the QMC Configuration Information of the source SN to the target MN in the Handover Request message.

- RAN3 discusses how to handle the available QoE measurement report in SCG leg when SCG failure happens.

- When setting ‘pause or resume’, both MN and SN overload should be taken into account.

Reference

[1] RP-221803. WID update for Enhancement on NR QoE. China Unicom.