3GPP TSG-RAN WG3 Meeting #121-bis	R3-235356

Xiamen, CN, 09–13 Oct, 2023

Agenda Item:	10.2.1

Source:	CMCC

Title:	Discussion on SON enhancement for SPR

Document for:	Discussion and Decision

# 1 Introduction

In the last RAN3 meeting RAN3#121[1], the agreement on SPR was achieved:

SPR:

In case of MN initiated PSCell change, MN will have the final say on the T310/T312 SPR thresholds.

And the following issues were captured for MN initiated PSCell change and the objective of T304 SPR trigger, and need to be continued in next meeting:

FFS whether SN can propose its preferred T310/T312 SPR thresholds to MN

FFS whether the T310/T312 timer values can be provided as assistance information from SN to MN (e.g., to assist in the root cause analysis)

FFS whether the objective of T304 SPR trigger is to only optimize RACH access issues in target SN or whether it can be also used to optimize mobility configurations in the initiating node.

In case the objective of T304 SPR trigger is to optimize both RACH access issues in target SN and to optimize mobility configurations in the initiating node, RAN3 should discuss and clarify:

- Whether this might cause any issue e.g., result in conflicting optimizations in the target SN and the initiating node?

- Any spec impacts (e.g., whether we need to capture something in stage-2) or can we leave it up to gNB implementation on which node(s) the SPR is to be forwarded in case of T304 trigger being met?

In this contribution, we would further discuss on MN-initiated PScell change /CPC and the objective of T304 SPR trigger, and provide some proposals on these issues for SPR.

# 2 Discussion

2.1  MN-initiated classic PScell change /CPC

For MN-initiated classic PScell change /CPC, the following agreement was achieved in the last meeting:

In case of MN initiated PSCell change, MN will have the final say on the T310/T312 SPR thresholds.

This means that the MN node which initiates the procedure should decide the T310/T312 SPR triggers and perform root cause analysis for the related parameters and configuration optimization.

FFS whether SN can propose its preferred T310/T312 SPR thresholds to MN

FFS whether the T310/T312 timer values can be provided as assistance information from SN to MN (e.g., to assist in the root cause analysis)

From our point of view, the MN node could decide the T310/T312 triggers threshold by itself without source SN inputs and it is also helpful for source SN to optimize T310/T312 timer values if needed. And there is no Xn impact for coordination between the MN and the source SN.

Proposal 1: The MN node could decide the T310/T312 triggers by itself.

But we think that the T310/T312 timer values are needed for MN to perform root cause analysis. Based on the T310/T312 SPR thresholds and the T310/T312 timer values, MN can identify the problem node (e.g. MN or source SN). If MN is the problem node, it is responsible to optimize PSCell change configuration and associated mobility thresholds. And if source SN is the problem node, it is responsible to optimize T310/T312 timer values.

Observation 1: Based on the T310/T312 SPR thresholds and the T310/T312 timer values, MN can identify the problem node (e.g. MN or source SN), which need to optimize the related parameters and configuration.

Proposal 2: The T310/T312 timer values are needed for MN to perform root cause analysis.

If source SN provides the T310/T312 timer values to MN, there are some details need to be further discussed. For example,

- If the T310/T312 timer values are sent to MN before SPR generation, this means that MN has to keep the T310/T312 timer values for every UE which is configured SPR function for a long time. It will introduce extra burden and storage requirement to MN and possibly there is no SPR generation at all for some UEs.

- If the T310/T312 timer values are sent to MN after SPR generation, the potential signaling procedure impacts should be considered.

- Observation 2: If source SN provides the T310/T312 timer values to MN, there are some details need to be further discussed. For example,

- If the T310/T312 timer values are sent to MN before SPR generation, this means that MN has to keep the T310/T312 timer values for every UE which is configured SPR function for a long time. It will introduce extra burden and storage requirement to MN and possibly there is no SPR generation at all for some UEs.

- If the T310/T312 timer values are sent to MN after SPR generation, the potential signaling procedure impacts should be considered.

2.2  Objective of T304 SPR trigger

FFS whether the objective of T304 SPR trigger is to only optimize RACH access issues in target SN or whether it can be also used to optimize mobility configurations in the initiating node.

In case the objective of T304 SPR trigger is to optimize both RACH access issues in target SN and to optimize mobility configurations in the initiating node, RAN3 should discuss and clarify:

- Whether this might cause any issue e.g., result in conflicting optimizations in the target SN and the initiating node?

- Any spec impacts (e.g., whether we need to capture something in stage-2) or can we leave it up to gNB implementation on which node(s) the SPR is to be forwarded in case of T304 trigger being met?

From our point of view, the RACH issues are mainly caused by the RACH resource shortage in target SN. Mobility configurations optimization in the initiating node could not directly change the state of RACH resource shortage in target SN.

On the other hand, considering the issues shown above should to be sufficiently analyzed in case the objective of T304 SPR trigger is to optimize both RACH access issues in target SN and to optimize mobility configurations in the initiating node. It is proposed that only RACH access issues optimization in target SN is considered if the SPR trigger is T304.

Proposal 3: The objective of T304 SPR trigger is to only optimize RACH access issues in target SN in Rel-18.

# 3 Conclusion

In this paper, we discuss on the open issue of MN-initiated PScell change /CPC and the objective of T304 SPR trigger, we have the following observations and proposals:

Proposal 1: The MN node could decide the T310/T312 triggers by itself.

Observation 1: Based on the T310/T312 SPR thresholds and the T310/T312 timer values, MN can identify the problem node (e.g. MN or source SN), which need to optimize the related parameters and configuration.

Proposal 2: The T310/T312 timer values are needed for MN to perform root cause analysis.

- Observation 2: If source SN provides the T310/T312 timer values to MN, there are some details need to be further discussed. For example,

- If the T310/T312 timer values are sent to MN before SPR generation, this means that MN has to keep the T310/T312 timer values for every UE which is configured SPR function for a long time. It will introduce extra burden and storage requirement to MN and possibly there is no SPR generation at all for some UEs.

- If the T310/T312 timer values are sent to MN after SPR generation, the potential signaling procedure impacts should be considered.

Proposal 3: The objective of T304 SPR trigger is to only optimize RACH access issues in target SN in Rel-18.

# Reference

- RAN3_121_agenda_20230825_EOM1