3GPP TSG-RAN WG3 Meeting #121-bis	     R3-235614

Xiamen, China, 09 – 13 October, 2023

Title:	[Draft] Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index

Response to:	S5-235777

Release:	3GPP Release 18

Work Item:	NR_AIML_NGRAN-Core

Source:	Nokia [to be RAN3]

To:	3GPP SA5

Cc:

Contact Person:

#### Name:	Anna Pantelidou

E-mail Address:	anna.pantelidou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

3GPP RAN3 thanks 3GPP SA5 for their response LS on AI/ML for NG-RAN Energy Saving Energy Cost index.

3GPP RAN3 would like to provide the following answers to the questions for clarification sent by 3GPP SA5.

- Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other?

Answer 1: The unified mapping rule should be defined by the operator, who may define a rule involving more than one scaling factor, each of which corresponds to a given condition or set of conditions satisfied at the NG-RAN node.

- Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)?

Answer 2: Energy Cost calculation should be based on a unitless average. The time duration over which the average is calculated is up to implementation.

- Should the Energy Cost be calculated in near-real time or non-real time (see above)?

Answer 3: We do not anticipate SA5 specification to address this aspect. Also the terms “near-real time” and “non-real time” do not seem relevant for an average cost.

- What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

Answer 4: In case of split architecture, normalization of the Energy Cost and encoding needs to be performed in the gNB-DU. The gNB-CU receives individual Energy Costs reported by the different gNB-DUs it manages and sums up the individual Energy Costs to obtain the total NG-RAN node Energy Cost.

2. Actions:

To SA5 : RAN3 kindly asks SA5 to take the above answers into consideration when designing their solutions and inform RAN3 if needed.

3. Dates of Next TSG-RAN WG3 Meetings:

3GPP TSG RAN WG3 #122                            13 - 17 November, 2023	             Chicago, US

3GPP TSG RAN WG3 #123                               26 February - 01 March, 2024	Athens, Greece